3GPP TSG-RAN WG3 Meeting #129	R3-255451

Bengaluru, India, 25 – 29 August, 2025

Agenda item:	11.4

Source:	Nokia, Deutsche Telekom, Orange

Title:	(TP to TS 38.401) Stage 2 updates for split architecture support

Document for:	Text Proposal

# 1	Introduction

In the past meetings, we have made several agreements related to split architecture support involving UE Performance metrics sent over F1 and E1 interfaces.

In particular, in RAN3 #126 the following agreement was made:

Change WA to agreement: Transferring the predicted CCO issue via the CU configuration update procedure and transferring the Future Coverage State via the DU configuration update procedure over F1.

In RAN3 #127bis we made the following agreement:

Packet delay measured for UE Performance feedback is sent from DU to CU via ASSISTANCE INFORMATION DATA frame over F1-U in the case of CU-DU split architecture.

Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over E1 interface to transfer UL/DL UE throughput and UL/DL packet delay from CU-UP to CU-CP upon request.

Agree the TP to 37.480 and TP to 37.483 in this meeting for the new procedure over E1 interface.

Change the agreement: Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 interface to transfer measured EC from DU to CU.

To the following WA:

WA: Using the Legacy procedure, i.e., resource status initiation procedure & resource status report procedure to request and report measured EC over F1 interface.

In RAN3 #128 we made the following agreement:

Adopt Opt2: Option2: gNB-CU directly forwards the received further CCO state of neighbor cells to gNB-DU.

DL/UL UE Throughput Measurement will not be supported in split architecture in R19.

Turn WA to agreement: Using the Legacy procedure, i.e., resource status initiation procedure & resource status report procedure to request and report measured EC over F1 interface.

In this contribution, we provide a stage 2 TP for BLCR to TS 38.401.

# Annex A			  TP for BL CR to TS 38.401

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

# 7.11	Support of AI/ML for NG-RAN

### 7.11.1	General

The support of AI/ML for NG-RAN in non-split gNB case is specified in TS 38.300 [2].

In case of CU-DU split architecture, the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

For a split gNB, the Energy Cost (EC) of the gNB is the sum of the Energy Cost of its gNB-DUs.

### 7.11.2	OAM requirements

OAM configures the following on gNB-DU:

-	the minimum and maximum energy consumption values corresponding to the minimum and maximum EC index values respectively, based on an implementation-specific mapping rule, which is unified within a defined area.

-	the recommended time interval within which an NG-RAN node selects an implementation-specific time window for averaging of the measurements of the NG-RAN node's consumed energy.

### 7.11.3 Data Collection and Reporting

The following information can be configured to be reported by a gNB-DU:

- 	Energy Cost (EC)

-	UE performance feedback

The following information can be configured to be reported by a gNB-CU-UP:

-	UE performance feedback

The collection and reporting of Energy Cost is configured through the F1AP Resource Status Reporting Initiation procedure, while the actual reporting is performed through the Resource Status Reporting procedure over F1 interface. Packet delay measurements collection for UE Performance from gNB-DU is triggered through the F1AP UE Context Setup Request and UE Context Modification Request and is reported from gNB-DU to gNB-CU-UP through the ASSISTANCE INFORMATION DATA frame (NR user plane protocol). The collection and reporting of PDCP-level UE Performance is configured through the E1AP Data Collection Reporting Initiation procedure while the actual reporting is performed through the Data Collection Reporting procedure over E1 interface. The collection of UE Performance from gNB-CU-UP is triggered through the E1AP Bearer Context Setup Request and Bearer Context Modification Request.

F1AP signalling support related to AI/ML enabled Coverage and Capacity Optimization is described in clause 7.9.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>