3GPP TSG-RAN WG3 Meeting #129	R3-255454

Bengaluru, India, 25 – 29 August, 2025

Agenda item:	11.4

Source:	Nokia

Title:	Open points on UE Performance in Split Architecture

Document for:	Decision

# 1	Introduction

In the last RAN3 meeting we achieved good progress in the topic of split architecture support for Rel-18 use cases.

Specifically, for E1AP we made the following agreements:

Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over E1 interface to transfer UL/DL UE throughput and UL/DL packet delay from CU-UP to CU-CP upon request.

Packet delay measured for UE performance feedback sent from CU-UP to CU-CP is per DRB level.

Send the LS to SA5 to seek the clarification on UE throughput UL/DL at PDCP level and UL/DL Packet Loss Rate at PDCP layer.

WA: In case of both split architecture and non-split architecture, UE throughput UL/DL at PDCP level can be considered for the AI/ML for NG-RAN function in Rel-19.

In addition, for F1AP, the following was agreed related to the packet delay measurement, replacing an earlier agreement that packet delay measured for UE Performance feedback is sent via DDDS in case of split architecture:

Packet delay measured for UE Performance feedback is sent from DU to CU via ASSISTANCE INFORMATION DATA frame over F1-U in the case of CU-DU split architecture.

Two open points were captured, namely:

Defer the discussion on packet loss rate in the split architecture in the next meeting.

For Packet delay, whether/how to enhance UE Context Setup/Modification procedure to introduce the configuration of data collection, e.g., start, stop, UL, DL, and periodicity?

In RAN3 #127 we had also made an agreement related to measuring and reporting of Average Packet Delay for UE Performance Feedback in split architecture:

Packet delay measured for UE Performance feedback is sent from CU-UP (the delay in DU side is aggregated in the CU-UP) to the CU-CP in the case of CP-UP split architecture.

In other words, we agreed to re-use to the extent possible the QoS Monitoring framework introduced in Rel-16 for the purpose of reporting of Average Packet Delay from gNB-CU-UP to gNB-CU-CP.

In this paper we provide a short overview of the Rel-16 QoS Monitoring Framework. We also provide our views on the needed modifications to this framework to report UE Performance Delay from gNB-CU-UP to gNB-CU-CP. In [1], we provide our views that AI/ML Slicing should be supported in split architecture in Rel-19. In this paper, we also provide our views with respect to this support.

# 2	Discussion

## 2.1	Support of the Rel-19 AI/ML Slicing use case in split architecture

### 2.1.1	Impact overview

Potential impact for split architecture support for AI/ML Slicing (see also our paper in [1]) relates to configuration and reporting of UE performance measurements. These are as follows:

DL/UL UE throughput: RAN3 already made the following agreement:

- DL/UL UE Throughput Measurement will not be supported in split architecture in R19.

Observation 1: There is no F1 or E1 impact relative to DL/UL UE throughput for AI/ML Slicing in split architecture.

DL/UL UE Packet Loss: This is agreed to be supported at PDCP level in Rel-19 as per the following agreement:

- Introduce new IE to include DL and UL Packet Loss Rate at PDCP Level in R19 refer to SA5 spec

We see two possible options:

Option 1:

In current E1AP BLCR the DL/UL UE Packet Loss is generically supported in the DATA COLLECTION REQUEST message (configuration) and reported per DRB in the DATA COLLECTION UPDATE message. Depending on RAN3’s decision, this reporting granularity could serve the purpose of the slicing use case provided that the gNB-CU-CP averages or aggregates information received separately per DRB into a per slice granularity. By the current E1AP CR, it can be noted that such averaging or aggregation in the gNB-CU-CP is already needed in order to obtain a per-UE granularity, if the measured per-UE packet loss is to be transferred over Xn for the purpose of UE Performance reporting in Rel-18. Aggregation or averaging of this information received separately per DRB into a per-slice granularity would follow similar algorithm. It can also be noticed that such algorithm anyway is needed for averaging or aggregation of UE packet delay measurements.

Option 2:

Option 2 would be to enhance the E1AP BLCR to support configuration and reporting of DL/UL UE Packet Loss per slice. SA5 indicated in their reply LS [2] for throughput measurements:

“In summary, although the measurement inputs are taken per DRB, they are aggregated to provide throughput metrics at the UE level”

RAN3 didn’t ask similar question for packet loss, but in SA5 specification the granularities (per DRB / S-NSSAI / UE) are described in the same way for throughput and for packet loss. By option 2, RAN3 would therefore assume that the SA5-defined UE packet loss measurement could directly provide measurement result per S-NSSAI granularity.

However, for consistency, option 2 would in our view also require changing the E1AP BLCR for Rel-18 UE Performance reporting so that the gNB-CU-CP can receive packet loss measurements reported per UE granularity. In this case E1AP would use similar measurement and configuration framework as XnAP.

Observation 2: Configuration and reporting of DL/UL UE Packet Loss per slice can be supported according to the two options above.

Even though we believe that both options are feasible, Option 1 will avoid impact on the E1AP BLCR, and also provide more detailed information to the gNB-CU-CP.

Proposal 1: DL/UL Packet Loss is reported from gNB-CU-UP to gNB-CU-CP separately per DRB as per E1AP BLCR, and averaged or aggregated into per S-NSSAI granularity in the gNB-CU-CP.

DL/UL UE Packet Delay: This measurement is agreed to be reported per DRB over E1 according to the following agreement, already covered by the  E1AP BLCR:

Packet delay measured for UE performance feedback sent from CU-UP to CU-CP is per DRB level.

Also, for the AI/ML slicing use case, DRB level reporting of DL/UL UE Packet Delay is needed over E1 due to the handling in the gNB-CU-CP which integrates the UL PDCP delay reported by the UE per DRB. There is therefore no additional E1AP or F1AP impact for support of AI/ML slicing for this measurement.

Observation 3: There is no additional E1AP or F1AP impact to support AI/ML Slicing for the DL/UL UE Packet Delay measurement.

From the observations above, because many aspects of split architecture support of UE performance reporting have now been clarified for the purpose of the Rel-18 use cases, it can be seen that option 2 creates stage 3 (E1AP) impact for support of AI/ML Slicing in split architecture.  Option 1 therefore seems to be most suitable.

Proposal 2: RAN3 agrees that AI/ML Slicing use case is supported in split architecture within the present work item.

## 2.2	DL packet loss – measurement description

In reply LS from SA5 [3] on clarification on UE performance measurement at PDCP level, SA5 acknowledged the importance of monitoring packet loss at the PDCP level in both UL and DL directions. In addition, SA5 indicated that DL PDCP SDU Drop Rate measured in gNB-CU-UP is defined in TS 28.552 clause 5.1.3.2.1 and that UL PDCP SDU Loss Rate measured at gNB-CU-UP is defined in TS 28.552 clause 5.1.3.1.1. SA5 in their LS encouraged RAN3 to review these existing definitions and provide feedback on whether they satisfy the intended use cases, or if additional definitions are required.

In addition, in SA5 specification TS 28.552 packet drop rate and packet loss rate are defined as follows:

From the above definitions, it is easy to see that the packet drop rate corresponds only to packets that were never sent to the target and therefore were dropped. Those packets are considered in the calculation of the packet loss rate, which additionally takes into account packets that were dropped. Therefore, packet loss rate seems to better suit the impacts in the UE Performance after a Handover event since it is more inclusive.

Observation 4: Packet Loss Rate is more suitable to reflect the impact of a Handover event to the UE Performance as opposed to Packet Drop Rate.

Having said this, since currently there does not exist Packet Loss Rate DL measurement measured at PDCP in SA5 specifications (but only Packet Drop Rate exists instead) and given the positive input from SA5 on packet loss measured at PDCP level we think that this measurement needs to be introduced. Since this measurement is for UE Performance feedback, the new measurement should be reported per UE.

Proposal 3: Ask SA5 to introduce PDCP Level Packet Loss Rate DL measurement for UE Performance feedback, as a per UE measurement.

The corresponding draft reply LS to SA5 is provided in [4] and consolidates questions and requests towards SA5 on several aspects related to UE Performance.

## 2.3	UL packet loss – measurement description

In [3] Reply LS on clarification on UE performance measurement at PDCP level, it was acknowledged that there is a measurement UL PDCP SDU Loss Rate defined in TS 28.552 clause 5.1.3.1.1.  It is a measure of the UL packet loss including any packet losses in the air interface, in the gNB-CU and on the F1-U interface. In fact, UL PDCP SDU Loss Rate measured at gNB-CU-UP is also defined in TS 28.558 clause 6.3.1.3.1, meaning that the per-UE measurement is already defined by SA5.

Observation 5: UL PDCP SDU Loss Rate measured at gNB-CU-UP is defined in TS 28.558 clause 6.3.1.3.1 and therefore the measurement is defined per UE.

Currently the semantics of Average Packet Loss UL IE in UE Performance IE clause 9.3.1.aa point to a reference to TS 28.552. This reference needs to be updated with the correct reference to TS 28.558.

Proposal 4: Update the semantics of Average Packet Loss UL IE in UE Performance IE clause 9.3.1.aa as follows: “Corresponds to UL PDCP SDU Loss Rate as specified in TS 28.558 [XX] clause 6.3.1.3.1”.

## 2.4	UL/DL packet loss – measurement accuracy

Another aspect relates to the accuracy of the packet loss rate in terms of the number of bits needed to represent it. The proposed range of the accuracy of Average Packet Loss DL IE and Average Packet Loss UL IE is 10-7 while there is an editor’s note on the semantics description of the IE. The measurement result field of this measurement corresponds to the loss rate multiplied by the inverse of the accuracy (accuracy being 10-6 as indicated e.g., in TS 28.558 clause 6.3.1.3.1 field d). Even though the accuracy in SA5’s specification is set to 10-6, the accuracy in RAN3 specification (e.g. XnAP) is 10-3:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Packet Loss Rate | M |  | INTEGER (0..1000, ...) | Ratio of lost packets per number of packets sent, expressed in tenth of percent. |

Therefore, as an example, a measured loss rate of 10-2 will be signalled with the value 10 (10-2  * 103) on a legacy E1 interface with accuracy 10-3 .

The statistical accuracy of an individual packet loss rate measurement result is dependent on how many packets have been lost during the observation period and will therefore also depend on the packet loss rate itself and on the throughput (number of packets per second) of the UE. If during the observation period, no packets are lost or a single packet is lost one can only conclude that the observed packet loss rate is better than the inverse of the number of packets transmitted. A packet loss rate measurement should therefore be based on observation periods that are long enough so that it is possible to observe at the very least 2 lost packets, but fixing this number to e.g. 10 will of course greatly improve the statistical accuracy. However, when adjusting the duration of the observation period, we also need to take into account that UE performance in a target cell is provided for a maximum duration of 5 seconds.

The source node is aware of the UE throughput in the target node (reported as part of UE performance), of the average packet size at the time of the handover and configured data collection duration at the target node. Therefore, it may consider this information to assess the statistical accuracy of the reported packet loss rate measurement and accordingly evaluate the AI/ML based action.

Observation 6: Statistical accuracy of the reported packet loss rate depends on the bit rate of the UE, on the average packet size, and on the measurement duration, and this information is available in the source node.

Based on the above, we provide in the following table an overview of maximum achievable measurement accuracy using on a period of 1 second (which allows to obtain 5 measurements during the Collection Time Duration), and by fixing the number of required lost packets to 3 per observation period. The packet size is set to 512 bytes.

| Achievable packet loss rate measurement accuracy | Measurement volume [MB] | Required throughput [Mbps] |
| --- | --- | --- |
| 1E-02 | 0.154 | 1.23 |
| 1E-03 | 1.536 | 12.3 |
| 1E-04 | 15.36 | 123 |
| 1E-05 | 153.6 | 1 229 |
| 1E-06 | 1 536 | 12 288 |
| 1E-07 | 15 360 | 122 800 |
| 1E-08 | 153 600 | 1 228 800 |

Under these conditions we notice that measurements with an accuracy of 10-3, as currently supported on network interfaces, are achievable for a UE transmitting at 12.3 Mbps in the UL and/or in the DL. Correspondingly, a measurement accuracy of 10-5 requires a UE throughput of more than 1 Gbps, while a measurement accuracyy of 10-6 requires a UE throughput of 12 Gbps. Even though these throughput values seem reasonable, a higher measurement accuracy (e.g., of 10-8) would in the above conditions require more than 1 Tbps, which seems like a less realistic bit rate, at least in the 5G era.

Similarly, we can derive from the table that based on a UE throughput (UL or DL) of 10 Mbps, a duration of 34 hours is needed to transmit the 154 GB required to measure a packet loss of 10-8, which is of course reduced to 3.4 hours for UE throughput of 100 Mbps, but still far beyond a reasonable observation duration for UE performance measured by the network.

Based on this analysis, we believe that the current accuracy of 10-3 used on network interfaces would remain a suitable value for UE performance observation in many practical scenarios.

However, to be more future proof we can consider other scenarios also, such as NR-DC, where a peak throughput of 6 Gbps can be observed (total peak throughput for 2 cells), shared across the UEs in the cells. In the corner case of a single UE being served, a measurement with an accuracy of 10-8 requires an observation period of 3 minutes 25 seconds based on the observation of 3 lost packets with packet size of 512 bytes (and a similar measurement with 10-10 accuracy takes nearly 6 hours) while the accuracy of 10-6 can at 6 Gbps be obtained in 2 seconds. This would be aligned with the accuracy of the loss rate defined in SA5 specifications. Therefore, based on the above analysis we think that the accuracy of the packet loss measurement can be increased to 10-6 in Rel-19.

Proposal 5: Update the accuracy of Average Packet Loss UL IE and Average Packet Loss DL IE to 10-6 for reporting over the E1 interface.

## 2.5	Packet delay – configuration aspects

In order to trigger the reporting of packet delay from gNB-DU to gNB-CU-UP, we have agreed to reuse the QoS Monitoring framework. Still a question remains on whether a reporting periodicity needs to be introduced over F1 interface or not, which translates to the following two options:

- Option 1: Leverage the existing F1AP framework without Reporting Periodicity.

- Option 2: Introduce Reporting Periodicity over F1AP

In QoS framework, option 1 is implementation-dependent. The delay components calculated at gNB-DU are reported through DDDS and Assistance Information to the gNB-CU-UP over F1-U. For F1-U delay measurement (done in the gNB-CU-UP), the gNB-DU reports its internal feedback delay in report polling triggered DDDS. Other delay components measured in the gNB-DU are accumulated in the gNB-DU (UL, DL) and provided in Assistance Information. The gNB-DU starts the measurement and reports it to the gNB-CU-UP as indicated in TS 38.425: “The Transfer of Assistance Information procedure may be invoked if the corresponding node is configured to perform the QoS monitoring and to send the QoS monitoring results to the node hosting the NR PDCP entity for the concerned data radio bearer.” As a result, the reporting type (event-triggered or periodic reporting) and reporting frequency from the gNB-DU are based on OAM configuration or implementation. The gNB-CU-UP calculates the total delay result by combining gNB-DU Delay Result together with its own delay result and reports it to the PSA UPF in an UL data packet or in a dummy UL packet.

This may lack flexibility in terms of signalling-based configuration and could e.g. lead implementations to report delays over F1-U more frequently than required. Through OAM configuration, gNB-CU-UP can be made aware of the configuration value at gNB-DU and hence it will be possible for gNB-CU-UP to identify if it can support the requested periodicity or whether it needs to apply special handling when it receives a request for UE Performance from gNB-CU-CP (e.g. to skip some measurements). RAN3 should therefore discuss whether introduction of signalled reporting periodicity in control of gNB-DU reporting is needed. However, our preference would be to introduce this parameter.

Proposal 6: Introduce reporting periodicity over F1 in UE CONTEXT SETUP REQUEST message through a new Performance Delay Monitoring Reporting Periodicity IE that has the same periodicity as the Reporting Periodicity for Data Collection IE in XnAP, namely encoded as an ENUMERATED(500ms, 1000ms, 2000ms, 5000ms,10000ms, …).

Another aspect that remains open is whether we need to trigger Performance Delay Monitoring in the DRB to be Setup in the UE CONTEXT MODIFICATION REQUEST message for a given UE. In the last meeting, some companies claimed that during the time when UE Performance Feedback is requested from a target node, there is no valid scenario when new DRBs would be needed to be setup. In our view, there could be such scenarios e.g., when core network decides to setup new DRBs for a UE. However, if we enable the reporting of UE Performance over new DRBs it raises a different concern, that of comparability of UE Performance between the source and the target. If new DRBs are added at the target node for the UE, this means that the UE Performance at the target will be necessarily higher than the UE Performance at the source since more resources are given to the UE. Therefore, allowing monitoring of UE Performance when new DRBs are setup would create issues of incomparability of the UE Performance at the target and at the source node and for this reason we think it should not be supported.

However, the F1AP ASN.1 uses a common IE (DRB-Information) for all use cases of DRB setup and DRB modification, which is included both in the UE CONTEXT SETUP REQUEST message and in the UE CONTEXT MODIFICATION REQUEST message. Therefore, because the PerformanceDelayMonitoring IE needs to be supported for DRBs to be setup in UE CONTEXT SETUP REQUEST, it will therefore also be supported for DRBs to be setup in the UE CONTEXT MODIFICATION REQUEST message. We therefore propose to add the statement “This IE is ignored by the gNB-DU” for the occurrence of the Performance Delay Monitoring IE within the DRB to Be Setup Item IE in the tabular description of the UE CONTEXT MODIFICATION REQUEST message.

Proposal 7: Address the FFS by adding semantics text to ignore the Performance Delay Monitoring IE in the UE CONTEXT MODIFICATION REQUEST message in the DRB to be Setup List.

# 3	Conclusion

In this paper we provide the following observations and proposals:

Observation 1: There is no F1 or E1 impact relative to DL/UL UE throughput for AI/ML Slicing in split architecture.

Observation 2: Configuration and reporting of DL/UL UE Packet Loss per slice can be supported according to the two options above.

Proposal 1: DL/UL Packet Loss is reported from gNB-CU-UP to gNB-CU-CP separately per DRB as per E1AP BLCR, and averaged or aggregated into per S-NSSAI granularity in the gNB-CU-CP.

Observation 3: There is no additional E1AP or F1AP impact to support AI/ML Slicing for the DL/UL UE Packet Delay measurement.

Proposal 2: RAN3 agrees that AI/ML Slicing use case is supported in split architecture within the present work item.

Observation 4: Packet Loss Rate is more suitable to reflect the impact of a Handover event to the UE Performance as opposed to Packet Drop Rate.

Proposal 3: Ask SA5 to introduce PDCP Level Packet Loss Rate DL measurement for UE Performance feedback, as a per UE measurement.

Observation 5: UL PDCP SDU Loss Rate measured at gNB-CU-UP is defined in TS 28.558 clause 6.3.1.3.1 and therefore the measurement is defined per UE.

Proposal 4: Update the semantics of Average Packet Loss UL IE in UE Performance IE clause 9.3.1.aa as follows: “Corresponds to UL PDCP SDU Loss Rate as specified in TS 28.558 [XX] clause 6.3.1.3.1”.

Observation 6: Statistical accuracy of the reported packet loss rate depends on the bit rate of the UE, on the average packet size, and on the measurement duration, and this information is available in the source node.

Proposal 5: Update the accuracy of Average Packet Loss UL IE and Average Packet Loss DL IE to 10-6 for reporting over the E1 interface.

Proposal 6: Introduce reporting periodicity over F1 in UE CONTEXT SETUP REQUEST message through a new Performance Delay Monitoring Reporting Periodicity IE that has the same periodicity as the Reporting Periodicity for Data Collection IE in XnAP, namely encoded as an ENUMERATED(500ms, 1000ms, 2000ms, 5000ms,10000ms, …).

Proposal 7: Address the FFS by adding semantics text to ignore the Performance Delay Monitoring IE in the UE CONTEXT MODIFICATION REQUEST message in the DRB to be Setup List.

In [4] we provide a draft reply LS to SA5, consolidating questions and requests towards SA5 on several aspects related to UE Performance discussed in this paper.

# References

[1]	R3-255449, (TP to TS 38.423) AI/ML Slicing Remaining Aspects, Nokia

[2]	R3-253023, Reply LS on per-UE UE performance metrics, SA5

[3]	R3-253824 - S5-252839, LS in R3-252492 from RAN3 on clarification on UE performance measurement at PDCP level, SA5

[4]		R3-255450, [Draft] Reply LS on clarification on UE performance measurement at PDCP level, Nokia