3GPP TSG-RAN WG3 Meeting #129	R3-255460

# Bengaluru, India, 25th – 29rd Aug, 2025

Title: 	On packet loss metric in UE performance

Source: 	Ericsson, Deutsche Telekom, Jio Platforms

Agenda item:	9.2

Document Type:	Discussion and agreement

# 1. Introduction

In this contribution, we discuss aspects related to the Packet Loss metric, which is part of UE performance measurements. Although RAN3 in Release-19 is discussing split architecture as a Release-18 leftover, i.e., specify functionality to bring parity between split and non-split architecture, we show in this paper that there exist shortcomings in deriving meaningful feedback from the Packet Loss metric as per Release-18 specification. In the discussion below we bring forward the different shortcomings and propose on a way forward for Release-18.

In RAN3#127-bis, the issue of the Rel.18 packet loss metric was discussed and resulted in the following point to be continued at part of Rel18 corrections:

Whether to correct packet loss metric in UE performance in R18?

- Opt1: Remove the IE

- Opt2: Introduce new IE for packet loss metric in UE performance to replace the old IE

- Opt3: Ignore the IE in R18

- Opt4: Do nothing

While the following agreement was also captured as part of the Rel19 discussions:

  - A Rel18 correction to the Average Packet Loss DL IE may be made when agreements on whether and how to specify Average Packet Loss in Rel19 are taken.

# 2. Discussion

Obtaining feedback to actions derived via AI/ML is an important step in bridging the gap between an AI/ML model and the real-world. In the context of AI/ML based use-cases such as mobility optimization, RAN3 specified a mechanism to collect UE performance after an AI/ML derived mobility action. The UE performance metrics include Average UE throughput DL, Average UE throughput UL, Average Packet Delay UL, Average packet delay DL, and Average Packet loss DL, which is collected by a target gNB after handover and is signalled back to the source gNB upon fulfilment of specified exit conditions.

The source gNB, after receiving the metrics in the UE performance, may take into account the performance at the target cell and use it to tune AI/ML derived mobility actions, e.g. prioritizing actions that maximize UE performance, understanding resulting performance in relation to earlier decisions. Such process may lead to model re-training/updating. UE performance may also be used to collect input data for model training where feedback for handed over UEs is requested without the action being derived using AI/ML models.

## Issues with the Packet Loss metric

## Issues with the existing definitions

Packet Loss for UE performance is defined as follows in TS 38.423.

#### 9.2.3.11 Packet Loss Rate This IE indicates the Packet Loss Rate for a QoS flow. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Packet Loss Rate; M; INTEGER (0..1000, ...); Ratio of lost packets per number of packets sent, expressed in tenth of percent..

Based on the above, the specified range for the packet loss rate is an integer between 0 to 1000 with steps of 0.1%.

It needs to be highlighted that the IE above was first introduced as part of the GBR QoS Flow Information IE (TS 38.423, clause 9.2.3.6). This IE indicates the QoS Parameters for a GBR QoS Flow for downlink and uplink, namely the QoS targets. Packet Loss Rate in this IE provides an indication of the maximum rate for lost packets of the QoS Flow that can be tolerated in the uplink and downlink direction. It is at this point important to understand that this parameter was not introduced to measure Packet Loss Rate, but to indicate a maximum upper limit for such metric.

Observation 1: The Packet Loss Rate IE has been introduced to indicate a maximum upper limit for the Loss Rate in UL/DL and not to indicate the measured values of such metric. The Packet Loss Rate IE in Rel18 has been reused to measure UL/DL Packet Loss Rate for UE Performance exposure purposes.

The essence of the above observation is that reusing the Packet Loss Rate IE to indicated measurements of an UL/DL Packet Loss Rate is semantically incorrect. Any potential change to the Packet Loss Rate IE will affect the meaning and intended use of both UE performance and GBR QoS Flow Information IEs.

Next, we note the definition of the Packet Loss Rate IE, which states the following:

This IE indicates the Packet Loss Rate for a QoS flow.

Such a description made sense when the Packet Loss Rate IE was introduced to indicate a maximum upper limit per QoS Flow for the UL/DL Packet Loss Rate. However, this definition does not make sense when the IE is used to represent a measured UE Performance parameter. UE performance, as the name suggests, is a per-UE granularity metric. The mismatch between the two granularities is obvious, i.e., per QoS flow vs per UE. Therefore, it is obvious that the definition of the Packet Loss Rate IE is inappropriate when using this IE for UE Performance Feedback measurements.

Observation 2: There exists a mismatch in the definition of the Average Packet Loss DL IE to its underlying IE’s definition (Packet Loss Rate IE) in relation to its intended use-case as a per-UE performance metric.

Finally, the Packet Loss Rate IE is defined as an integer with a range from 0 to 1000, and each step indicates a 0.1% change in the ratio of lost packets to the number of packets sent, namely the minimum reportable loss rate is of 10-3. In NR, there exist several different 5QIs, which translate to a certain QoS offered to the data stream (available in TS 23.501). The range of packet loss rates guaranteed by the different 5QIs ranges from 10-2 to 10-8, i.e., a ratio of loss of 0.01 to 0.00000001. The Packet Loss Rate IE in its existing form cannot express packet loss that is less than 10-3. Therefore, the Average Packet Loss DL IE, in its current standardised form, would not even be able to represent the measured packet loss rate in a real network (as the value range of the IE is too coarse). Even if the Average Packet Loss DL IE is used in its current form, any collected data at or below loss rates of 10-3 will at best show value 0 (if the measurement is approximated to no packet losses) or 1 (if the measurement is approximated to the minimum reportable packet loss ratio).

In past RAN3 meetings some companies were of the opinion that the current Average Packet Loss DL IE could be correctly used. We explain here with some examples why this is not possible because it would lead to erroneous Packet Loss Rate measurements.

For this example, we take a typical scenario where a UE is on a video streaming service for a DL bit rate of 1Mbps. Assuming an average packet size of 512 bytes, the packets per second are calculated as follows:

- Packets per second in DL = (1,000,000 bits/s) / (512 bytes/packet * 8 bits/byte) = 1950 packets/s

Let us assume that the UE performance collection period is of 1 second, namely the Reporting Periodicity for Data Collection IE is set to 1000ms. The packets transmitted to the UE for each reporting period are therefore 1950

- Reporting Periodicity for Data Collection IE is set to 1000ms

Let´s now assume a usual case where the Packet Loss Rate for this service is of 10-4. In this case the packets lost during the reporting period are 0.195.

- Assuming a usual case packet loss rate of 10-4, namely 0.01%, the packets lost during a reporting period are 0.195.

However, the Rel18 Average Packet Loss DL IE allows to report, at the very minimum a packet loss rate of 0.1%, i.e. a loss rate of 10-3. In this case the measurement reported in the Average Packet Loss DL IE would be set to “1”, which implies that the UE was subject to a loss of 1.95 packets during the reporting period.

- The Rel18 Average Packet Loss DL IE is able to report as a minimum a value of “1”, corresponding to a loss of 1.95 packets during a reporting period, which is way above the actual losses of 0.195 packets.

The conclusion from this example is that the Rel18 Average Packet Loss DL IE will report erroneous packet loss rate measurements that will create interoperability issues. In fact, the receiving node will not be able to understand if the UE performance was truly affected by a 0.1% loss rate or if the loss rate was below that. If the receiver assumes that the loss rate is truly 0.1%, which is a very high loss rate, this will have a big impact on e.g. AI/ML model retraining/retuning and future AI/ML based mobility decisions. Namely mobility towards a target cell where the packet loss rate is of 0.1% would be strongly discouraged as such drop rate is very poor.

Observation 3: The Rel18 Average Packet Loss DL IE enables reporting of Packet Loss Rates from 0.1% to 100%. This does not allow to capture real packet loss rate measurements that are way below 0.1%, ending up in interoperability issues where the receiver does not know if the reported loss rate is truly measured or is due to rounding to the minimum reportable value

During the last RAN3 meeting some companies mentioned that, in some cases, the Average Packet Loss DL IE could provide meaningful measurements. However, the standard shall not define an IE that sometimes represents a correct measurement and sometimes it does not. Again, this creates an interoperability issue where it is not known whether the measurement shall be taken into account as UE performance feedback or not.

Therefore, the Average Packet Loss DL IE introduced in TS 38.423 is of no use and ignoring this IE would bring no impact to the overall functionality.

Proposal 1: RAN3 to acknowledge the following issues with the Average Packet Loss DL IE in UE performance:

- The Packet Loss Rate IE has been introduced as part of the GBR QoS Flow Information IE to indicate a maximum upper limit for the Loss Rate in UL/DL and not to indicate the measured values of such metric

- Average Packet Loss DL IE (which reuses the Packet Loss Rate IE definition) is defined at a per-UE granularity, while the Packet Loss Rate IE is defined at a per-QoS-Flow level

- The Average Packet Loss DL IE, in its current form is too coarse to represent real-world expected packet loss in 5G.

- Modifications to the definition of the Packet Loss Rate IE has an impact on both the GBR QoS Flow Information and on UE Performance information.

## Solutions

Discussions during RAN3#127-bis captured the following alternative solutions:

  - Opt1: Remove the IE

  - Opt2: Introduce new IE for packet loss metric in UE performance to replace the old IE

  - Opt3: Ignore the IE in R18

  - Opt4: Do nothing

From the list of available options above, and from the list of drawbacks with Average Packet Loss DL IE shown as part of Proposal 1, it is obvious that doing nothing (Opt 4) would lead to a standard that signals wrong or incorrect information. 
In context of Release 18, any introduced change/correction should cause the least amount of disruption to the standard. Therefore, introducing a new IE for packet loss (Opt 2) should not be pursued for Release 18. This is a much better choice for the Release 19 work RAN3 is carrying out. In fact, our paper to AI 11.4 (R3-255476) proposes the introduction of a new metric for Packet Loss Rate in UE performance which addresses the measurement granularity and other issues.

The remaining options are to remove the IE (Opt 1) or to ignore the IE in Rel.18 (Opt 3). We believe that ignoring the IE is a better approach to keep the specifications backwards compatible.

The UE performance collection mechanism relies on the Report Characteristics bitstring in the Data Collection Request message. Removing the IE would lead to a need to change all of the subsequent bits in the bitstring. This will not be backwards compatible. Instead, the option to ignore/null the Average Packet Loss DL IE and the associated Report Characteristics bits results in a much cleaner solution. A proposal of this is show in the companion CR.

Based on the above discussion, we propose:

Proposal 2: RAN3 to agree that Rel18 Average Packet Loss DL IE is not a suitable candidate metric for AI/ML UE performance feedback in split architecture use cases.

Proposal 3: RAN3 to agree to a correction to ignore the Average Packet Loss DL IE in Release 18 and to introduce a new IE to measure packet loss rate for Rel19.

As an alternative to ignoring the Release 18 Average Packet Loss DL IE, the gNB may keep the existing definition of the IE and in addition send another indication together with the UE Performance metrics indicating if the signaled Average Packet Loss DL IE is indeed the measured value, or if the signaled value is an approximation of the measured value that had to be performed due to the limited reporting granularity of the existing IE. A neighbor gNB receiving this metric and the supplementary indication may chose to handle the received value appropriately.

Proposal 4: As an alternative to Proposal 2 and Proposal 3, RAN3 to consider signaling an additional indication together with the Average Packet Loss DL measurements indicating if the signaled value is the measured value or if it is an approximation/rounded value.

# 3. Conclusions

This paper makes the following observations and proposals:

Observation 1: The Packet Loss Rate IE has been introduced to indicate a maximum upper limit for the Loss Rate in UL/DL and not to indicate the measured values of such metric. The Packet Loss Rate IE in Rel18 has been reused to measure UL/DL Packet Loss Rate for UE Performance exposure purposes.

Observation 2: There exists a mismatch in the definition of the Average Packet Loss DL IE to its underlying IE’s definition (Packet Loss Rate IE) in relation to its intended use-case as a per-UE performance metric.

Observation 3: The Rel18 Average Packet Loss DL IE enables reporting of Packet Loss Rates from 0.1% to 100%. This does not allow to capture real packet loss rate measurements that are way below 0.1%,ending up in interoperability issues where the receiver does not know if the reported loss rate is truly measured ot is due to rounding to the minimum reportable value

Proposal 1: RAN3 to acknowledge the following issues with the Average Packet Loss DL IE in UE performance:

- The Packet Loss Rate IE has been introduced as part of the GBR QoS Flow Information IE to indicate a maximum upper limit for the Loss Rate in UL/DL and not to indicate the measured values of such metric

- Average Packet Loss DL IE (which reuses the Packet Loss Rate IE definition) is defined at a per-UE granularity, while the Packet Loss Rate IE is defined at a per-QoS-Flow level

- The Average Packet Loss DL IE, in its current form is too coarse to represent real-world expected packet loss in 5G.

- Modifications to the definition of the Packet Loss Rate IE has an impact on both the GBR QoS Flow Information and on UE Performance information.

Proposal 2: RAN3 to agree that Rel18 Average Packet Loss DL IE is not a suitable candidate metric for AI/ML UE performance feedback in split architecture use cases.

Proposal 3: RAN3 to agree to a correction to ignore the Average Packet Loss DL IE in Release 18 and to introduce a new IE to measure packet loss rate for Rel19.

Proposal 4: As an alternative to Proposal 2 and Proposal 3, RAN3 to consider signaling an additional indication together with the Average Packet Loss DL measurements indicating if the signaled value is the measured value or if it is an approximation/rounded value.

A CR reflecting the main proposals in this paper is presented in R3-255461.