3GPP TSG-RAN WG3 Meeting #129	R3-255464

Bengaluru, India, 25-29 August 2025

Title: 	AI/ML enabled Network Slicing

Source: 	Ericsson, Jio Platforms

Agenda item:	11.2

Document Type:	Discussion and agreement

# Introduction

At the RAN3#127-bis meeting the following agreement and FFS were captured regarding the per slice UE performance reporting:

Slice UE performance request:

For Slice UE performance request, take Opt2: Extend Existing Report Characteristics Bitmap.

Failed Report Characteristics:

Extending the existing Node Measurement Failed Report Characteristics IE to include slice-level UE performance metrics for indicating the failed result for slice-level UE performance metrics.

Per Slice UE performance report:

Add Slice UE performance IE in the DATA COLLECTION UPDATE message.

Per S-NSSAI per UE performance metrics is reported as part of the Slice UE Performance IE in the Data Collection Update message. An FFS on how the metrics are calculated will be added in the TP. If no SA5 definition on how to calculate the metrics is available, RAN3 to decide the metrics are derived by an average or weighted average measurement.

Introduce the measured CCCV within the predicted slice available capacity.

At the RAN3#128 meeting, it was agreed to add per slice UE performance feedback metrics in the Data Collection Reporting procedure. Among these metrics, UL and DL average per slice UE throughput is collected at RLC layer for exchange over the Xn interface, while for packet loss rate, the following was agreed for split architectures:

Introduce new IE to include DL and UL Packet Loss Rate at PDCP Level in R19 refer to SA5 spec

We therefore expect that

- Per S-NSSAI RLC level throughput measurements will be signalled over the Xn

- Per S-NSSAI PDCP level UL/DL Packet Loss Rate metrics will be signalled over the Xn.

Through this contribution we propose to list the S-NSSAIs for which the UE performance is reported. We further propose to define the Packet Loss Rate IE with its recommended range.

# 2. Discussion

## Requesting list of slices for UE Performance

For the source gNB to request from the target gNB the reporting of per slice UE performance after a successful UE handover the following agreement was made at RAN3#127-bis:

For Slice UE performance request, take Opt2: Extend Existing Report Characteristics Bitmap.

In addition, the source gNB must be able to indicate to the target gNB which are the network slices for which the per slice UE Performance reporting is requested. In absence of this indication, the target gNB will have no options but to report the per slice UE Performance for all the network slices for which the UE has an active PDU session in the target cell despite that:

- the source gNB may not need the per slice UE Performance to be reported for this entire set of slices. Indeed, for some slice such as the default slice or slices for eMBB, there might not be the need for UE performance reporting, as UE performance in these cases is best effort

- some of the slices the UE is using in the target gNB may not be available or supported in the source gNB. In this case the source gNB would receive per slice performance information that is not useful for AI/ML purposes because they cannot be compared with any slice performance at the source.

Observation 1: Lack of an indication from source gNB to Target gNB of the slices for which UE performance is needed implies signalling from target to source of unnecessary and useless per slice performance information

To avoid unrequested transferring of per slice UE Performance reports from the target gNB to the source gNB we propose adding the list of slices for which the per slice UE performance reporting is requested by the source gNB into the UE Performance Collection Configuration IE as shown below:

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Collection Time Duration for UE Performance | M |  | INTEGER(1..5000, ...) | Time duration starting at successful handover within which the UE performance measurements are collected. Unit: millisecond |
| Slice To Report List for UE Performance |  | 0..1 |  | S-NSSAI list for which UE performance measurements are requested. |
| >Slice To Report Item for UE Performance |  | 1 .. < maxnoofBPLMNs > |  |  |
| >>PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN |
| >>S-NSSAI List |  |  | 1 |  |
| >>>S-NSSAI Item |  | 1 .. < maxnoofSliceItems> |  |  |

As it can be seen, signalling from source to target gNB which are the slice of interest fo the collection of per slice UE performance metrics is simple and effective.

Proposal 1: RAN3 to agree adding a list of slices to the UE Performance Collection Configuration IE of the DATA COLLECTION REQUEST message, to enable the source gNB to indicate to the target gNB which are the slices for which per slice UE Performance reporting is requested.

## Packet Loss Rate

In RAN3#128, it has been agreed to add per slice UE performance feedback metrics in the Data Collection Reporting procedure. Note that it has been agreed to collect UL and DL Packet Loss Rate at PDCP layer, but the exact definition of these measurements is still FFS , including the value range of these IEs.

The Packet Loss Rates are FFS because RAN3 is pending to receive a reply from SA5 as a consequence of the LS sent by RAN3 in R3-253816. In this LS RAN3 asks SA5 whether Packet Loss Rate metrics can be defined with a per UE granularity. It is plausible to assume that SA5 will include the packet loss rate metrics in TS328.558, in order to have the metrics with a per UE granularity scope.

Nevertheless, it is also plausible to assume that the definition for the UL and DL Packet Loss Rate metrics currently in TS28.552, will be kept if these metrics are ported to TS28.558. The current UL PDCP SDU Loss Rate defined in TS 28.552 clause 5.1.3.1.1 and the DL PDCP SDU Drop Rate in gNB-CU-UP defined in TS 28.552 clause 5.1.3.2.1, are currently encoded as integers with a value range 0 to 1000000. For this reason, and to align with the metric definition in SA5 specifications, it is sufficient to define the UL/DL Packet Loss Rate metrics as INTEGER (0..10000000, ...). An example of how to capture this in the specification is provided below.

#### 9.2.3.pp	Slice Based UE Performance

This IE represents UE performance metrics per S-NSSAI.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Average Slice UE Throughput DL | O |  | 9.2.3.4 | Corresponds to Average UE Throughput DL per S-NSSAI as specified in TS 28.558 [XX] clause 6.3.1.4.1. |
| Average Slice UE Throughput UL | O |  | 9.2.3.4 | Corresponds to Average UE Throughput UL per S-NSSAI as specified in TS 28.558 [XX] clause 6.3.1.4.2. |
| Average Slice Packet Delay | O |  | 9.2.3.187 | Corresponds to the Average Packet Delay per S-NSSAI as specified in TS 28.558 [XX], clause 6.3.1.1. |
| Average Slice Packet Loss DL | O |  | INTEGER (0..10000000, ...) | Corresponds to DL PDCP SDU Drop Rate per S-NSSAI as specified in TS 28.552 [XX] clause 5.1.3.2.1. |
| Average Slice Packet Loss UL | O |  | INTEGER (0..10000000, ...) | Corresponds to • UL PDCP SDU Loss Rate per S-NSSAI as specified in TS 28.552 [XX] clause 5.1.3.1.1. |

Proposal 2: RAN3 to agree defining the Average Slice Packet Loss DL and Average Slice Packet Loss UL as INTEGER (0..10000000, ...).

It is understood that SA5 is discussing the details of how to enhance their specifications to better describe how per S-NSSAI metrics are calculated. RAN3 has asked SA5 to define UL/DL packet loss rates with a per UE granularity. It is assumed that if SA5 define these metrics with a per UE granularity, the metrics will also have a per UE/per S-NSSAI granularity. 
We therefore propose the following:

- If a reply from SA5 reaches RAN3 within RAN3-129, RAN3 to update the semantic descriptions of the Average Slice Packet Loss UL/DL with a reference to the SA5 specification where such measurement is defined

- If a reply from SA5 does not reach RAN3 within RAN3-129, RAN3 to leave the semantic description of the Average Slice Packet Loss UL/DL generic (e.g. to leave it empty) and to update it with a reference to the SA5 specification where such measurement is defined once a reply from SA5 is received.

It should be avoided that RAN3 defines a new way to derive per S-NSSAI measurements because this may contradict any enhancements SA5 applies to their specifications when it comes to the description of per S-NSSAI measurements. Hence it is important that RAN3 reference to the SA5 specification where measurements are defined. However, it is also important for RAN3 to complete their specifications at RAN3-129, as this is the last meeting for Rel19.

Observation 2: RAN3 should avoid defining how per S-NSSAI metrics are calculated as this may contradict any current or future description on per S-NSSAI metrics in SA5 specifications

Proposal 3: In the semantics description of the Average Slice Packet Loss UL/DL IEs, RAN3 to reference to the SA5 specification where such measurement is defined. This reference can be added at RAN3-129, if available, or at later meetings.

# Conclusions

In this contribution we discuss the details of per slice UE Performance reporting. Throughout the discussion we derived the following observations  and proposals:

Observation 1: Lack of an indication from source gNB to Target gNB of the slices for which UE performance is needed implies signalling from target to source of unnecessary and useless per slice performance information

Proposal 1: RAN3 to agree adding a list of slices to the UE Performance Collection Configuration IE of the DATA COLLECTION REQUEST message, to enable the source gNB to indicate to the target gNB which are the slices for which per slice UE Performance reporting is requested.

Proposal 2: RAN3 to agree defining the Average Slice Packet Loss DL and Average Slice Packet Loss UL as INTEGER (0..10000000, ...)Observation 2: RAN3 should avoid defining how per S-NSSAI metrics are calculated as this may contradict any current or future description on per S-NSSAI metrics in SA5 specifications

Proposal 3: In the semantics description of the Average Slice Packet Loss UL/DL IEs, RAN3 to reference to the SA5 specification where such measurement is defined. This reference can be added at RAN3-129, if available, or at later meetings.

The proposals above are reflected in the TP on AI/ML enabled Network Slicing for (BL CR to TS 38.423) provided in R3-255465.

# References

[1]	3GPP TS 38.413 V18.4.0 (2024-12)