3GPP TSG-RAN WG3 Meeting #129	R3-255466

Bengaluru - India, 25th – 29th August 2025

Title: 	Prediction time for CCO issue

Source: 	Ericsson, Jio Platforms, Orange, InterDigital, BT, FiberCop

Agenda item:	11.3

Document Type:	Discussion and agreement

# 1. Introduction

In relation to feedback for AI/ML enabled CCO, in this contribution we provide further discussion on the following open point captured at RAN3#127:

Timing information for predicted CCO issue over Xn is not needed?

# 2. Discussion

At previous meetings, the option of signaling timing information for predicted CCO issue over Xn has been discussed, and no consensus could be reached so far.

We think there are a number of reasons why it is important for a gNB2 neighbouring a gNB1 where a CCO issue is predicted to receive this information (in addition to the already signalled IEs indicating the predicted CCO issue (Predicted Coverage Modification Cause), the future CCO state (Future Cell Coverage State, Future SSB Coverage State), and the time of the future CCO state (Time for Future Coverage State).

In the following, we discuss why signaling this piece of information in Xn is important in various scenarios.

Firstly, we consider the case where one CCO issue is predicted at gNB1, and the gNB1 does not cancel nor update it.   In principle, a prediction can be correct or erroneous, and for assessing this, a neighbouring gNB2 can monitor radio measurements (e.g., UE radio measurements) revealing whether there is a coverage degradation or a cross cell interference that can be associated with the CCO issue predicted by gNB1. However, without knowing the Time for predicted CCO Issue of gNB1, the gNB2 cannot know when the radio measurements shall be monitored for this purpose. Therefore, it is not possible for gNB2 to determine if the predicted CCO issue will materialise in practice or not

Observation 1: If gNB2 does not know the timing information of a CCO issue predicted at gNB1, it does not know when it is appropriate to monitor metrics (e.g., UE radio measurements) revealing coverage or cross cell interference degradations due to the predicted CCO issue.

If gNB1 does not cancel or update the predicted CCO issue, the gNB1 will apply the future CCO state at a time which is known by gNB2 (Time for Future Coverage State). However, the gNB2 does not know whether Time for Future Coverage State is before or after Time for predicted CCO Issue and the difference between them. For an implementation which tries to avoid the predicted CCO issue, the Time for predicted CCO Issue represents the maximum time limit within which the future CCO state is applied. Within this limit, different implementations of gNB1 may decide to apply the future CCO state with different margins (e.g., 15 seconds, or 10 seconds, or 5 seconds before the Time for predicted CCO Issue). Without knowing the Time for predicted CCO Issue, a gNB2 wanting to avoid the predicted CCO issue, is practically mandated to apply the matching future CCO state at the same time when the future CCO state is applied. This implies unnecessary complexity because it forces gNB2 to apply a Future CCO State change at a time when such change may not be possible, e.g. due to implementation constraints. Such strict time requirements concerning when to apply the Future CCO State should not be mandated by the standard.

Without knowing the Time for predicted CCO Issue of gNB1, there can also be potential interoperability problem and uncontrolled fluctuations in the coverage boundaries. For example, it is not precluded that a gNB1 implementation decides to apply Future CCO State changes after the time of the predicted CCO issue, e.g. to verify whether the predicted CCO issue materialises and what are its real impacts. In this case the Time for Future Coverage State is after the Time for Predicted CCO Issue, but this is not known at gNB2. Hence, the neighbour gNB has no means to decide whether to set its own future CCO state timing before or after the predicted CCO issue.

Observation 2: If gNB2 does not know the timing information of the CCO issue predicted at gNB1, gNB2 is mandated to apply the matching future CCO state at the same time when the future CCO state changes.

As a second scenario, we consider the case of one CCO issue predicted at gNB1, and gNB1 cancels or updates it.

The flow chart below shows, at a high level, the steps needed to either “cancel” or “update” a previously predicted CCO issue (for better readability, some simplifications are made, for example some messages are omitted).

According to current agreements, the gNB2 receives the time information of the future CCO state decided by DU11. However, knowing only this timing information may not be sufficient at gNB2 to properly support cancelling/updating the previously predicted CCO issue. For example, the DU21 in gNB2 may decide to apply a matching future CCO state before the predicted CCO issue is cancelled. In this case, a new change of CCO state would be needed, reverting the coverage to the initial CCO state. Instead, it would have been better for DU21 to wait a bit more, e.g. closer to the time of the predicted CCO issue, and in case the predicted CCO issue was cancelled, do not apply any change in the CCO state change. If DU21 knows the timing of predicted CCO issue at gNB1, it can try to apply the matching future CCO state of its cells and beams without the need to revert to the original CCO state.

Observation 3: At a neighbouring gNB, if the predicted CCO issue is cancelled after the matching future CCO state is applied, the DU needs to revert the coverage to the initial CCO state.

By sending the Time for predicted CCO Issue over Xn we minimize the risk at the neighbour gNB2 to revert its coverage to the initial CCO state after having already adopted a matching future CCO state. If the gNB2 is a split gNB, this information should be passed to the underlying DUs (in the example, from CU2 to DU21).

Observation 4: If a neighbouring gNB knows the Time for predicted CCO Issue, the neighbouring gNB can take into account the possibility that the predicted CCO issue is cancelled or updated, before adopting the matching future CCO state.

As a third scenario, we consider the case of multiple CCO issues predicted. For example, a CCO issue is predicted at gNB1 and another at gNB3, and the gNB2 is neighbouring both gNB1 and gNB3. If the gNB2 does not receive the timing information of the two predicted CCO issues, it cannot know which one it can expect to occur first and cannot prioritize the solution of one against the other.

Observation 5: In case of multiple predicted CCO issues, a neighbouring gNB cannot know which one will occur first and cannot prioritize the solution of one against another.

In summary, we see several problems and limitations if the Time for Predicted CCO Issue of CU1 is not sent via Xn from the node where the CCO issue is predicted, to a neighbouring gNB. The neighbouring gNB can use it to assess whether the predicted CCO issue is correct or not, to avoid uncontrolled fluctuations in the coverage boundaries, and having to revert the change in case the predicted CCO issue is cancelled or updated. Further, if the neighbouring gNB is a split gNB, this information should be forwarded from CU to DU, to support the DU to take appropriate decision regarding whether and when it is most appropriate to apply the matching future CCO state. We propose then to send the Time for Predicted CCO Issue via Xn, using the same encoding as in F1AP.

Proposal 1: The gNB where the CCO issue is predicted sends the Time for Predicted CCO Issue to neighbour gNBs in the NG-RAN CONFIGURATION UPDATE XnAP message.

Proposal 2: At a split gNB receiving the Time for Predicted CCO Issue for a CCO issue predicted at a neighbour gNB, the CU forwards this information to its DUs in the CU-DU CONFIGURATION UPDATE F1AP message.

Proposal 3: Agree the TP to XnAP provided in R3-255467.

# 3. Conclusions

Based on the discussion in this paper, the observations and proposals listed below are derived.

Observation 1: If gNB2 does not know the timing information of a CCO issue predicted at gNB1, it does not know when it is appropriate to monitor metrics (e.g., UE radio measurements) revealing coverage or cross cell interference degradations due to the predicted CCO issue.

Observation 2: If gNB2 does not know the timing information of the CCO issue predicted at gNB1, gNB2 is mandated to apply the matching future CCO state at the same time when the future CCO state changes.

Observation 3: At a neighbouring gNB, if the predicted CCO issue is cancelled after the matching future CCO state is applied, the DU needs to revert the coverage to the initial CCO state.

Observation 4: If a neighbouring gNB knows the Time for predicted CCO Issue, the neighbouring gNB can take into account the possibility that the predicted CCO issue is cancelled or updated, before adopting the matching future CCO state.

Observation 5: In case of multiple predicted CCO issues, a neighbouring gNB cannot know which one will occur first and cannot prioritize the solution of one against another.

Proposal 1: The gNB where the CCO issue is predicted sends the Time for Predicted CCO Issue to neighbour gNBs in the NG-RAN CONFIGURATION UPDATE XnAP message.

Proposal 2: At a split gNB receiving the Time for Predicted CCO Issue for a CCO issue predicted at a neighbour gNB, the CU forwards this information to its DUs in the CU-DU CONFIGURATION UPDATE F1AP message.

Proposal 3: Agree the TP to XnAP provided in R3-255467.