3GPP TSG-RAN WG3 Meeting #129	R3-255468

Bengaluru - India, 25th – 29th August 2025

Title: 	Cancel or update a predicted CCO issue

Source: 	Ericsson, Jio Platforms, Orange, InterDigital, Deutsche Telekom, FiberCop

Agenda item:	11.3

Document Type:	Discussion and agreement

# 1. Introduction

At the last RAN3 meeting, the following agreements and open points were captured:

Adopt Opt2. (gNB-CU directly forwards the received further CCO state of neighbor cells to gNB-DU.)

Specify mechanisms (F1, Xn) to update/cancel a prediction, no new IE is needed to identify the previous prediction (affected cells and beams will identify the previously signalled issue).

Addition of new code-point in predicted coverage modification cause (Xn) / predicted CCO issue (F1), with details FFS.

Current mechanism enables the update of predicted CCO issue and/or future CCO state. A new detected CCO issue/a new predicted CCO issue for the same affected cells and beams after a predicted CCO issue will cancel the prediction.

Check the scenario whether has the possibility that there will be isolated multiple CCO issues detected for different cells or beams? Specific Cancel towards each CCO issue is needed?

Timing information for predicted CCO issue over Xn is not needed?

In this contribution we discuss the mechanisms to cancel or update a predicted CCO issue.

# 2. Discussion

## Cancel or update a predicted CCO issue

For handling a predicted CCO issue, a CU sends to DU the following information:

- A type of predicted CCO issue (coverage, cell-edge-capacity)

- The cells and beams predicted to be affected by the predicted CCO issue type

- The time when the type of predicted CCO issue is predicted to occur

There can be different factors leading to a predicted CCO issue to become outdated or incorrect. Just to name a few:

- UE movements or change in UE traffic may lead to a previously predicted interference at cell edge not to manifest,

- actions taken by the node predicting the CCO issue, or actions taken by the neighbouring node (e.g., load balancing actions) may change the load at cell edge,

- new CCO issues may be detected or predicted by the same node predicting the CCO issue or by a neighbouring node.

Based on current agreements and baselines, once a CCO issue of a certain type is predicted at a gNB1 (CU1):

- If the gNB1 is a split gNB with two (or more) DUs, e.g., DU11 and DU12, and the predicted affected cells and beams are in DU11, the CU1 informs the DU11 about the predicted CCO issue and the predicted affected cells and beams (to generate a future CCO state), and the same information is also sent from CU1 to DU12, to generate a matching future CCO state for cells and beams of DU12.

- The gNB1 signals the type of predicted CCO issue to a neighbour gNB2 (predicted coverage modification cause via Xn)

- If the gNB2 is a split gNB, the CU2 in gNB2 sends the predicted affected cells and beams of gNB1 to the DU (e.g., DU21) in gNB2, so that a matching future CCO state can be generated for cells and beams of DU21.

When/if a certain predicted CCO issue needs to be cancelled, a number of entities should be informed. In the scenarios just described:

- CU1 needs to inform DU11, because the future CCO state the DU11 has generated is not needed anymore

- CU1 needs to inform DU12, because the matching state the DU12 has generated is not needed anymore

- gNB1 needs to inform gNB2, because a matching state at gNB2 to address the predicted CCO issue is not needed anymore

- CU2 (in gNB2) needs to inform DU21, because the future matching state the DU21 has generated is not needed anymore

The flow chart below shows at a high level, the steps needed to either “cancel” or “update” a previously predicted CCO issue (for better readability, some simplifications are made, for example some messages are omitted).

At the last RAN3 meeting, the following agreement and open points were captured in relation to cancelling/updating a predicted CCO issue:

Specify mechanisms (F1, Xn) to update/cancel a prediction, no new IE is needed to identify the previous prediction (affected cells and beams will identify the previously signalled issue).

Addition of new code-point in predicted coverage modification cause (Xn) / predicted CCO issue (F1), with details FFS.

Current mechanism enables the update of predicted CCO issue and/or future CCO state. A new detected CCO issue/a new predicted CCO issue for the same affected cells and beams after a predicted CCO issue will cancel the prediction.

Firstly, one of the principles used in our specifications is to describe the behaviour of a node when receiving a certain information and we do not specify the receiving node behaviour when not receiving a certain information. If a certain information is absent in a certain message, this should not trigger any expected behaviour at the receiver. This is because, the receiver cannot deduce the reason why the sender did not sent the information. The information may be missing because it could not be produced, for example, which is totally plausible if the information is optional. Namely, the principle that the receiver´s behaviour should not be triggered by absence of information shall be maintained.

Observation 1: A receiving node cannot deduce the reason for absence of a certain information in a message. Hence, the receiver´s behaviour cannot be specified by absence of information.

According to this principle, and considering again the proposed example, it is clear that, if the CU1 has informed the DU11 about a predicted CCO issue and later wants to inform the DU11 that the same predicted CCO issue is cancelled, it needs to send this information explicitly.

Proposal 1: The information that a previously predicted CCO issue is cancelled is provided explicitly.

According to the agreement mentioned above, no new IE is needed to identify a previous prediction, and the affected cells and beams can be used. Hence, to clearly indicate that a specific predicted CCO issue is cancelled, a node should signal the same predicted affected cells and beams (identifying a previously predicted CCO Issue) and an explicit “cancel” command, in the form of a new code value for the Predicted CCO Issue.

Similarly, if a CU wants to inform a DU (or a gNB wants to inform a neighbour gNB) that a previously predicted CCO issue needs to be updated (for example, the CU wants to inform the DU that a previously predicted CCO issue will occur a bit later), the same approach can be used, that is, the CU signals to the DU (or the gNB signals to the neighbour gNB) the same predicted affected cells and beams and an updated Predicted CCO Issue.

Observation 2: A previously predicted CCO issue is uniquely identified in F1AP by the same predicted affected cells and beams.

Based on the presented analysis, we propose to use the following mechanism to cancel or update a predicted CCO issue:

- In a split gNB, if the CU wants to inform an underlying DU that a certain predicted CCO issue is cancelled, the CU sends to the DU the predicted CCO issue type set at a new code value “cancel” and the predicted affected cells and beams of the previously signalled predicted CCO issue.

- To cancel a predicted coverage modification cause (over Xn), a gNB sends to a neighbour gNB the predicted coverage modification cause set at a new code value “cancel” and the predicted affected cells and beams of the previously signalled predicted CCO issue.

- Upon cancelling a predicted CCO issue, new predicted CCO issues can be signalled, therefore producing an update of a previously signalled CCo issue. Namely, to update a previously predicted CCO issue, the source gNB would cancel the old prediction and issue a new prediction.

Proposal 2: In a split gNB, if the CU wants to inform an underlying DU that a certain predicted CCO issue is cancelled, the CU sends to the DU the predicted CCO issue type set at a new code value “cancel” and the predicted affected cells and beams of the previously signalled predicted CCO issue. After such cancelation, the gNB-CU can signal an update of the CCO Issue, if needed.

Proposal 3: -	To cancel a predicted coverage modification cause (over Xn), a gNB sends to a neighbour gNB the predicted coverage modification cause set at a new code value “cancel” and the predicted affected cells and beams of the previously signalled predicted CCO issue. After such cancelation, the gNB can signal an update of the CCO Issue, if needed.

Proposal 6: Agree the TP to F1AP provided in R3-255470.

Proposal 7: Agree the TP to XnAP provided in R3-255469.

# 3. Conclusions

Observation 1: A receiving node cannot deduce the reason for absence of a certain information in a message. Hence, the receiver´s behaviour cannot be specified by absence of information.

Proposal 1: The information that a previously predicted CCO issue is cancelled is provided explicitly.

Observation 2: A previously predicted CCO issue is uniquely identified in F1AP by the same predicted affected cells and beams.

Proposal 2: In a split gNB, if the CU wants to inform an underlying DU that a certain predicted CCO issue is cancelled, the CU sends to the DU the predicted CCO issue type set at a new code value “cancel” and the predicted affected cells and beams of the previously signalled predicted CCO issue. After such cancelation, the gNB-CU can signal an update of the CCO Issue, if needed.

Proposal 3: To cancel a predicted coverage modification cause (over Xn), a gNB sends to a neighbour gNB the predicted coverage modification cause set at a new code value “cancel” and the predicted affected cells and beams of the previously signalled predicted CCO issue. After such cancelation, the gNB can signal an update of the CCO Issue, if needed.

Proposal 6: Agree the TP to F1AP provided in R3-255470.

Proposal 7: Agree the TP to XnAP provided in R3-255469.