3GPP TSG-RAN WG3 Meeting #129	R3-255474

Bengaluru, India, August 25th–29th, 2025

Title:	(TP to TS 38.423) AI/ML support for NR-DC

Source:	Ericsson, Jio Platforms

Agenda item:	11.4

Document Type:	Discussion and agreement

# 1. Introduction

In this paper, we present a TP to TS 38.423 for AI/ML assisted mobility optimization for NR-DC, based on the proposals in R3-255473.

# 2. TP to XnAP - AI/ML assisted mobility opt. for NR-DC

-------------------------------------- START OF CHANGES --------------------------------------

#### 9.2.3.179	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average UE Throughput DL | O |  | Bit Rate 9.2.3.4 |  |
| Average UE Throughput UL | O |  | Bit Rate 9.2.3.4 |  |
| Average Packet Delay | O |  | 9.2.3.187 |  |
| Average Packet Loss DL | O |  | Packet Loss Rate 9.2.3.11 |  |
| PSCell ID | C-ifSN |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates the PSCell where the UE performance was collected. |

| Condition | Explanation |
| --- | --- |
| ifSN | This IE shall be present if the UE performance was collected by the S-NG-RAN node in NR-DC. |

-------------------------------------- NEXT CHANGE --------------------------------------

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

-------------------------------------- skip unchanged --------------------------------------

id-MobileIABCell,

id-XR-Bcast-Information,

id-MaximumDataBurstVolume,

id-CPAC-Preparation-Type,

id-MN-only-MDT-collection,

id-BarringExemptionforEmerCallInfo,

id-Transmission-Bandwidth-asymmetric,

id-NRPPaPositioningInformation,

id-SliceToReportForDataCollection-List,

id-PredictedSliceAvailableCapacity,

id-SliceMeasurementInitiationResult,

id-SliceUEPerformance,

id-PSCellID,

maxEARFCN,

maxnoofAllowedAreas,

maxnoofAMFRegions,

maxnoofAoIs,

maxnoofBPLMNs,

maxnoofCAGs,

maxnoofCAGsperPLMN,

maxnoofCellsinAoI,

maxnoofCellsinNG-RANnode,

-------------------------------------- NEXT CHANGE --------------------------------------

#### -- U

-------------------------------------- skip unchanged --------------------------------------

UEPerformance ::= SEQUENCE {

dL-UE-AverageThroughput						BitRate					OPTIONAL,

uL-UE-AverageThroughput						BitRate					OPTIONAL,

uE-AveragePacketDelay						AveragePacketDelay		OPTIONAL,

uE-AveragePacketLossDL						PacketLossRate			OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { UEPerformance-ExtIEs} } OPTIONAL,

...

}

UEPerformance-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-PSCellID		CRITICALITY ignore	EXTENSION GlobalNG-RANCell-ID 	PRESENCE conditional},

-- This IE shall be present if the UE performance was collected by the S-NG-RAN node in NR-DC.

...

}

-------------------------------------- NEXT CHANGE --------------------------------------

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

-------------------------------------- skip unchanged --------------------------------------

id-UserPlaneFailureIndication																		ProtocolIE-ID ::= 469

id-MN-only-MDT-collection																			ProtocolIE-ID ::= 470

id-BarringExemptionforEmerCallInfo																	ProtocolIE-ID ::= 471

id-Transmission-Bandwidth-asymmetric																ProtocolIE-ID ::= 472

id-SRSPositioningConfigOrActivationRequest															ProtocolIE-ID ::= 473

id-NRPPaPositioningInformation																		ProtocolIE-ID ::= 474

id-Future-Coverage-Modification-List																ProtocolIE-ID ::= xx1

id-SliceToReportForDataCollection-List																ProtocolIE-ID ::= xx2

id-PredictedSliceAvailableCapacity																	ProtocolIE-ID ::= xx3

id-SliceMeasurementInitiationResult																	ProtocolIE-ID ::= xx4

id-SliceUEPerformance																				ProtocolIE-ID ::= xx5

id-PSCellID																							ProtocolIE-ID ::= xx6

END

-- ASN1STOP

-------------------------------------- END OF CHANGES --------------------------------------