3GPP TSG-RAN WG3 Meeting #129	R3-255475

# Bengaluru, India, 25th – 29rd Aug, 2025

Title: 	On LS discussions with SA5 on Packet loss measurements

Source: 	Ericsson, Deutsche Telekom, Jio Platforms

Agenda item:	11.4

Document Type:	Discussion and agreement

# 1. Introduction

In this contribution, we tackle the ongoing discussions between RAN3 and SA5 on the topic of Packet Loss metric for UE Performance. The presented discussion is applicable to split and non-split architecture deployments for Release 18. RAN3 in Release-19 is discussing split architectures as a Release-18 leftover, i.e., specify functionality to bring parity between split and non-split architecture. However, earlier discussions, supported by further arguments in the received LS from SA5, show that there exist shortcomings in deriving meaningful feedback from the Packet Loss metric based on the Release-18 specification.

During R3#127-bis RAN3 sent an LS to SA5 (R3-252492) on the feasibility of measuring Packet Loss Rate in UL and DL at PDCP level. This was motivated based on discussions in RAN3 which concluded that PDCP-level measurements better reflect the UE performance at the service level. During RAN3#128, SA5’s response to the above LS was received (R3-253824), wherein SA5 recognised the importance of monitoring packet loss at the PDCP level in both UL and DL directions and provided references to the already available PDCP-level packet loss measurements in UL and DL in TS 28.552, which define NR cell level measurements.

An excerpt from this LS is reported below:

Packet Loss Rate at PDCP Level (UL and DL) SA5 recognises the importance of monitoring packet loss at the PDCP level in both UL and DL directions. The following measurements are already defined: - UL PDCP SDU Loss Rate – TS 28.552 clause 5.1.3.1.1 - DL PDCP SDU Drop Rate in gNB-CU-UP – TS 28.552 clause 5.1.3.2.1 These metrics allow packet loss monitoring in UL and packet drop monitoring in DL for PDCP-layer traffic. SA5 encourages RAN3 to review these existing definitions and provide feedback on whether they satisfy the intended use cases, or if additional definitions are required.

RAN3 sent a further reply to the received LS in R3-253824 enquiring if the PDCP-level packet loss measurements suggested by SA5 are available with a per UE granularity, where per DRB measurements are aggregated to generate per UE measurements, and if not to specify these measurements with per UE granularity.

It should also be noted that the R3#129 meeting is the last meeting for Release 19. In order to conclude the discussions in time, it is important that RAN3 takes the necessary work agreements and assumptions in order to complete the work in time.

# 2. Discussion

In the LS response sent by SA5 in R3-243824, the following are suggested as the PDCP-level packet loss measurements:

•	UL PDCP SDU Loss Rate – TS 28.552 clause 5.1.3.1.1

•	DL PDCP SDU Drop Rate in gNB-CU-UP – TS 28.552 clause 5.1.3.2.1

Shown below is the definition of the UL PDCP SDU Loss Rate as defined in TS 28.552.

This measurement provides the fraction of PDCP SDU packets which are not successfully received at gNB-CU-UP. It is a measure of the UL packet loss including any packet losses in the air interface, in the gNB-CU and on the F1-U interface. Only user-plane traffic (DTCH) and only PDCP SDUs that have entered PDCP (and given a PDCP sequence number) are considered.

This measurement is obtained as: 1000000* Number of missing UL PDCP sequence numbers, representing packets that are not delivered to higher layers, of a data radio bearer, divided by Total number of UL PDCP sequence numbers (also including missing sequence numbers) of a bearer, starting from the sequence number of the first packet delivered by UE PDCP to gNB-CU-UP until the sequence number of the last packet.

[…]

d)	Each measurement is an integer value representing the loss rate multiplied by 1E6. The number of measurements is equal to one. If the optional QoS and S-NSSAI level measurements are performed, the measurements are equal to the number of mapped 5QIs and the number of supported S-NSSAIs.

Shown below is the definition of the DL PDCP SDU Drop Rate in gNB-CU-UP as defined in TS 28.552.

This measurement provides the fraction of PDCP SDU packets which are dropped on the downlink, due to high traffic load, traffic management etc in the gNB-CU-UP. Only user-plane traffic (DTCH) is considered. A dropped packet is one whose context is removed from the gNB-CU-UP without any part of it having been transmitted on the F1-U or Xn-U or X2-U interface.

This measurement is obtained as: 1000000*Number of dropped DL PDCP SDU packets whose contexts are removed from the gNB-CU-UP without any part of it having been transmitted on the F1-U or Xn-U or X2-U interface, of a data radio bearer, divided by Number of DL PDCP SDU packets for data radio bearers that have entered PDCP-SAP after being decoded from GTP-U packets.

[…]

d)	Each measurement is an integer value representing the drop rate multiplied by 1E6. The number of measurements is equal to one. If the optional QoS and S-NSSAI level measurement are performed, the measurements are equal to the number of mapped 5QIs and the number of supported S-NSSAIs.

As a first observation it can be noted that each measurement referenced b SA5 is a (positive) integer, which is achieved by multiplying a decimal number (loss rate) by 106. It is therefore obvious to deduce that the value range of the measurements referenced by SA5 is INTEGER (0..1000000).

Observation 1: The UL PDCP SDU Loss Rate and the DL PDCP SDU Drop Rate in gNB-CU-UP, as defined in TS 28.552, are measurements represented by an integer with value range from 0 to 1000000

The measurements defined in TS 28.552 are defined for the NR cell with potential filters specified to obtain the cell level measurements at a per 5QI, PLMN, PCI, S-NSSAI, etc. The discussion in RAN3 on packet loss measurements concerns a per UE granularity. In this context, RAN3 has already sent an LS to SA5 (R3-252492) enquiring on how the above defined measurements could be obtained at a per DRB per UE level granularity. If the response from SA5 does not arrive in time, then RAN3 shall make the following working assumption to progress the work.

Proposal 1: RAN3 to make a working assumption that the equivalent per DRB per UE measurements of the UL PDCP SDU Loss Rate and DL PDCP SDU Drop Rate in gNB-CU-UP will be defined by SA5.

The Release 18 definition of the Average Packet Loss DL IE is tied to Packet Loss Rate IE (TS 38.423 clause 9.2.3.11), which is defined as an integer with a range (0..1000, …), where the represented value is a ratio of lost packets per number of packets sent, expressed in tenth of percent.

Comparing the definition of the PDCP-level packet loss metrics (from TS 28.552) and the Packet Loss Rate definition (in TS 38.423), there is an obvious disparity in the level of granularity of the two measurements. The packet loss metrics in TS 28.552 are defined as the drop rate (dropped packets / total number of packets) which is then scaled up by a factor of 106. The factor 106 is used to round the measurement to an integer value. Therefore, the smallest possible packet loss rate is 10-6 and the largest is 1, where each step has a granularity of 10-6. The Packet Loss Rate defined in TS 38.423 however, spans an integer range of 0 to 1000, where each step in the range has a granularity of a tenth of a percentage, i.e., 10-3, with this being the smallest representable value as well. If the existing definition of packet loss rate in TS 38.423 is used to represent the per-DRB per-UE PDCP-level packet losses, then

- The smallest possible representation of packet loss rate will be in the order of 10-3

- There is a loss of resolution/granularity of the measurement, as the measurement results are bucketed into a smaller range of values

- Assuming that RAN3 will reference the measurements defined by SA5, which we assume will follow the definition in TS28.552, there is a mismatch between the reference measurement and the value range of the IE used to represent the UL/DL loss rate

The above limitations of the packet loss rate metric in TS 38.423 is evident when considering that the SA5 definition in TS 28.552 allows for a measurement granularity as low as 10-6, i.e., the lowest possible measurement granularity of the Packet Loss Rate metric does not enable monitoring the entire range of packet loss rates as specified by SA5.

Observation 2: There exists a significant limitation in using the Packet Loss Rate metric as defined in TS 38.423 for the purpose of UE Performance measurements.

Furthermore, our paper to AI 9.2 in R3-255460 further discusses other drawbacks and discrepancies in using the existing definition of Packet Loss Rate in TS 38.423 for the purpose of UE Performance measurements. In fact, the issue with the existing definition has already been discussed in RAN3 in other contexts as well. Discussions during RAN3#127-bis captured the following alternative solutions:

  - Opt1: Remove the IE

  - Opt2: Introduce new IE for packet loss metric in UE performance to replace the old IE

  - Opt3: Ignore the IE in R18

  - Opt4: Do nothing

RAN3 did not filter the above list further as RAN3 was awaiting further clarification from SA5 in this regard. However, we believe that RAN3 has received sufficient information from SA5 in this regard and a decision can be made. From the list of available options above, it is obvious that doing nothing (Opt 4) would lead to a standard that signals wrong or incorrect information. This option should not be pursued.

In context of Release 18, any introduced change/correction should cause the least amount of disruption to the standard because Rel18 is a frozen release. Therefore, introducing a new IE for packet loss (Opt 2) should not be pursued for Release 18. This is a much better choice for the Release 19 work RAN3 is carrying out. In fact, our paper to AI 11.4 (R3-255476) proposes the introduction of a new metric for Packet Loss Rate in UE performance which address the measurement granularity and other issues.

The remaining options are to remove the IE (Opt 1) or to ignore the IE in Rel.18 (Opt 3). We believe that ignoring the IE is a better approach to keep the specifications backwards compatible.

The UE performance collection mechanism (defined in TS 38.423) relies on the Report Characteristics bitstring in the Data Collection Request message. Removing the IE would lead to a need to change all of the subsequent bits in the bitstring. This will not be backwards compatible. Instead, the option to ignore/null the Average Packet Loss DL IE and the associated Report Characteristics bits results in a much cleaner solution. A proposal of this is show in the companion CR.

Based on the above discussion, we propose:

Proposal 2: RAN3 to agree that Rel18 Average Packet Loss DL IE is not a suitable candidate metric for AI/ML UE performance feedback in split architecture use cases.

Proposal 3: RAN3 to agree to a correction to ignore the Average Packet Loss DL IE in Release 18 and to introduce a new IE to measure packet loss rate for Rel19.

In the event of RAN3 deciding to keep the Average Packet Loss DL IE in use, which we strongly discourage, an alternative to ignoring the IE could be that a gNB sends, in addition to the Average Packet Loss DL IE, another indication indicating if the signaled Average Packet Loss DL IE is indeed the measured value, or if the signaled value is an approximation of the measured value that had to be performed due to the limited reporting granularity of the existing IE. A neighbor gNB receiving this metric and the supplementary indication may choose to handle the received value appropriately.

Proposal 4: As an alternative to Proposal 1 and Proposal 2, RAN3 to consider signaling an additional indication together with the Average Packet Loss DL measurements indicating if the signaled value is the measured value or if it is an approximation/rounded value.

# 3. Conclusions

This paper makes the following observations and proposals:

Observation 1: The UL PDCP SDU Loss Rate and the DL PDCP SDU Drop Rate in gNB-CU-UP as defined in TS 28.552 are measurements represented by an integer with value range from 0 to 1000000

Proposal 1: RAN3 to make a working assumption that the equivalent per DRB per UE measurements of the UL PDCP SDU Loss Rate and DL PDCP SDU Drop Rate in gNB-CU-UP will be defined by SA5.

Observation 2: There exists a significant limitation in using the Packet Loss Rate metric as defined in TS 38.423 for the purpose of UE Performance measurements.

Proposal 2: RAN3 to agree that Rel18 Average Packet Loss DL IE is not a suitable candidate metric for AI/ML UE performance feedback in split architecture use cases.

Proposal 3: RAN3 to agree to a correction to ignore the Average Packet Loss DL IE in Release 18 and to introduce a new IE to measure packet loss rate for Rel19.

Proposal 4: As an alternative to Proposal 1 and Proposal 2, RAN3 to consider signaling an additional indication together with the Average Packet Loss DL measurements indicating if the signaled value is the measured value or if it is an approximation/rounded value.