3GPP TSG-RAN WG3 Meeting #129	R3-255476

# Bengaluru, India, 25th – 29rd Aug, 2025

Title: 	Introduction of new packet loss metric in UE performance

Source: 	Ericsson, Deutsche Telekom, Jio Platforms

Agenda item:	11.4

Document Type:	Discussion and agreement

# 1. Introduction

In this contribution, we discuss aspects related to the Packet Loss metric, which is part of UE performance measurements. The presented discussion is applicable to split and non-split architecture deployments for Release 19. RAN3 in Release-19 is discussing split architectures as a Release-18 leftover, i.e., specify functionality to bring parity between split and non-split architecture. However, earlier discussions have shown that there exist shortcomings in deriving meaningful feedback from the Packet Loss metric based on the Release-18 specification. In the discussion below, we quickly recap the identified shortcomings and propose a way forward for Release-19 based on the already captured agreements.

In the context of Release 19 discussions on packet loss metric, the following has been captured in meeting notes:

  - From R3#127-bis: A Rel18 correction to the Average Packet Loss DL IE may be made when agreements on whether and how to specify Average Packet Loss in Rel19 are taken.

  - During R3#127-bis RAN3 sent an LS to SA5 (R3-252492) about the feasibility of measuring Packet Loss Rate in UL and DL at PDCP level.

  - During RAN3#128, RAN3 received an LS response from SA5 (R3-253824) concerning the above question, where SA5 answered that existing measurements in TS 28.552 (UL PDCP SDU Loss Rate and DL PDCP SDU Drop Rate in gNB-CU-UP) are indeed at the PDCP level.

- During RAN3#128, a LS was sent to SA5 (R3-253816) asking for clarification on whether the packet loss measurements defined in TS28.552 are aggregated per DRB per UE, and if not to specify these measurements with per UE granularity.

  - From R3#128: Wait for the response from SA5 on Rel-19 Packet Loss Rate measurement before updating the Rel-18 Packet Loss Rate.

  - From R3#128: Introduce new IE to include DL and UL Packet Loss Rate at PDCP Level in R19 refer to SA5 spec

# 2. Discussion

## Issues with the Packet Loss metric

Our companion paper to AI 9.2 in R3-255460 discusses in detail the issues with the current (Release 18) definition of the packet loss metric over the Xn interface. However, below, we summarise the central issue with the definition of the Average Packet Loss DL IE in Release 18 and why a different IE shall be adopted for this purpose in Release 19.

•	The Average Packet Loss DL IE references to the Packet Loss Rate IE, which has been introduced as part of the GBR QoS Flow Information IE to indicate a maximum upper limit for the Loss Rate in UL/DL and not to indicate the measured values of such metric.

•	Average Packet Loss DL IE (which reuses the Packet Loss Rate IE definition) is defined at a per-UE granularity, while the Packet Loss Rate IE is defined at a per-QoS-Flow level

•	The Average Packet Loss DL IE, in its current form is too coarse to represent real-world expected packet loss in 5G. As a consequence, the Average Packet Loss DL IE is not capable of representing the measured packet loss rate for a UE, as the minimum loss rate it can represent is 0.1%

•	Modifications to the definition of the Packet Loss Rate IE has an impact on both the GBR QoS Flow Information and on UE Performance information. Therefore, modifying the Packet Loss Rate IE to resolve the issues with the Average Packet Loss DL IE generates impacts on well-established functions such as QoS parameters exchange

Proposal 1: RAN3 to acknowledge and to design a solution which avoids the following issues with the Average Packet Loss DL IE in UE performance:

- The Packet Loss Rate IE has been introduced as part of the GBR QoS Flow Information IE to indicate a maximum upper limit for the Loss Rate in UL/DL and not to indicate the measured values of such metric

- Average Packet Loss DL IE (which reuses the Packet Loss Rate IE definition) is defined at a per-UE granularity, while the Packet Loss Rate IE is defined at a per-QoS-Flow level

- The Average Packet Loss DL IE, in its current form is too coarse to represent real-world expected packet loss in 5G.

- Modifications to the definition of the Packet Loss Rate IE has an impact on both the GBR QoS Flow Information and on UE Performance information.

## Further discussion

As shown in Sec. 2.1, both the granularity and the value range of the packet loss rate metric has to be extended (in relation to the Rel.18 definition) to be able to adequately represent a measured packet loss value to a neighbour node. Based on the definition of the QoS parameters for NR, the range of packet loss rates guaranteed by the different 5QIs ranges from 10-2 to 10-8, i.e., 1% to 0.000001%. In reality, one may observe measured packet loss rate values that range between 10-1 to 10-10.

Proposal 2: The new definition of Packet loss rate for UE performance shall accommodate for a range of packet loss rate values from 10-10 to 10-1.

The second aspect to consider is the LS response sent by SA5 in R3-243824. In the LS, the following metrics are suggested as the PDCP-level measurements:

•	UL PDCP SDU Loss Rate – TS 28.552 clause 5.1.3.1.1

•	DL PDCP SDU Drop Rate in gNB-CU-UP – TS 28.552 clause 5.1.3.2.1

The UL PDCP SDU Loss Rate is defined in TS 28.552 as follows.

This measurement provides the fraction of PDCP SDU packets which are not successfully received at gNB-CU-UP. It is a measure of the UL packet loss including any packet losses in the air interface, in the gNB-CU and on the F1-U interface. Only user-plane traffic (DTCH) and only PDCP SDUs that have entered PDCP (and given a PDCP sequence number) are considered.

This measurement is obtained as: 1000000* Number of missing UL PDCP sequence numbers, representing packets that are not delivered to higher layers, of a data radio bearer, divided by Total number of UL PDCP sequence numbers (also including missing sequence numbers) of a bearer, starting from the sequence number of the first packet delivered by UE PDCP to gNB-CU-UP until the sequence number of the last packet.

[…]

d)	Each measurement is an integer value representing the loss rate multiplied by 1E6. The number of measurements is equal to one. If the optional QoS and S-NSSAI level measurements are performed, the measurements are equal to the number of mapped 5QIs and the number of supported S-NSSAIs.

The DL PDCP SDU Drop Rate in gNB-CU-UP is defined in TS 28.552 as follows.

This measurement provides the fraction of PDCP SDU packets which are dropped on the downlink, due to high traffic load, traffic management etc in the gNB-CU-UP. Only user-plane traffic (DTCH) is considered. A dropped packet is one whose context is removed from the gNB-CU-UP without any part of it having been transmitted on the F1-U or Xn-U or X2-U interface.

This measurement is obtained as: 1000000*Number of dropped DL PDCP SDU packets whose contexts are removed from the gNB-CU-UP without any part of it having been transmitted on the F1-U or Xn-U or X2-U interface, of a data radio bearer, divided by Number of DL PDCP SDU packets for data radio bearers that have entered PDCP-SAP after being decoded from GTP-U packets.

[…]

d)	Each measurement is an integer value representing the drop rate multiplied by 1E6. The number of measurements is equal to one. If the optional QoS and S-NSSAI level measurement are performed, the measurements are equal to the number of mapped 5QIs and the number of supported S-NSSAIs.

The measurements defined in TS 28.552 are defined for the NR cell with potential filters specified to obtain the cell level measurements at a per 5QI, PLMN, PCI, S-NSSAI, etc. The discussion in RAN3 on packet loss measurements concerns a per UE granularity. In this context, RAN3 has already sent an LS to SA5 (R3-252492) on obtaining the defined measurements at a per DRB per UE level granularity. Given that the current meeting is the last meeting for Release 19 work, if the response from SA5 does not arrive in time, then RAN3 shall make the following working assumption to progress the work.

Proposal 3: RAN3 to make a working assumption that the equivalent per DRB per UE measurements of the UL PDCP SDU Loss Rate and DL PDCP SDU Drop Rate in gNB-CU-UP will be defined by SA5.

The Release 18 definition of the Average Packet Loss DL IE is tied to Packet Loss Rate IE (TS 38.423 clause 9.2.3.11), which is defined as an integer with a range (0..1000, …), where the represented value is a ratio of lost packets per number of packets sent, expressed in tenth of percent.

Comparing the definition of the PDCP-level packet loss metrics (from TS 28.552) (with the WA that these metrics are filtered to per DRB per UE granularity) and the Packet Loss Rate definition (in TS 38.423), there is an obvious disparity in the level of granularity of the two measurements. The packet loss metrics in TS 28.552 are defined as the drop rate (dropped packets / total number of packets) which is then scaled up by a factor of 106. The factor 106 is used to round the measurement to an integer value. Therefore, the smallest possible packet loss rate is 10-6 and the largest is 1, where each step has a granularity of 10-6. The Packet Loss Rate defined in TS 38.423 however, spans an integer range of 0 to 1000, where each step in the range has a granularity of a tenth of a percentage, i.e., 10-3, with this being the smallest representable value as well. If the existing definition of packet loss rate in TS 38.423 is used to represent the per-DRB per-UE PDCP-level packet losses, then

- The smallest possible representation of packet loss rate will be in the order of 10-3

- There is a loss of resolution/granularity of the measurement, as the measurement results are bucketed into a smaller range of integer values

The above loss of granularity when converting the raw measurement to a quantized value can be a significant limitation. This is due to the range of possible representations being too small compared to the original measurements. As with any measurement, a higher measurement granularity means that the sender can communicate finer details of the measurement to the receiver.

Proposal 4: The new definition of packet loss rate for UE performance shall accommodate a granularity that is detailed enough to use for applications with either very low or very high packet loss rates.

Based on the above proposed requirements, we conclude that, despite full granularity for packet loss rate measurements may be achieved with a value range that can represent packet loss rates of up to 10-10, a good compromise is to adopt the granularity already in use in the packet loss rate metrics defined by SA5, namely representing loss rates of up to 10-6. This enables reusing the parameters already defined by SA5. Below we show how such a metric, i.e., Average UE Packet Loss Rate in Rel.19, may be defined. This IE will indicate the measured Packet Loss Rate for a UE in both UL and DL.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Average UE Packet Loss Rate | M |  | 9.2.3.xx |  |

### 9.2.3.xx Average UE Packet Loss Rate

This IE indicates the Packet Loss Rate for a UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UL Average Packet Loss Rate | M |  | INTEGER (0..1000000, ...) | Corresponds to UL PDCP SDU Loss Rate as specified in TS 28.552 clause 5.1.3.1.1, expressed in units of 10-6. [FFS: Pending confirmation from SA5 if the measurement can be filtered per DRB per UE] |
| DL Average Packet Loss Rate | M |  | INTEGER (0..1000000, ...) | Corresponds to DL PDCP SDU Drop Rate in gNB-CU-UP as specified in TS 28.552 clause 5.1.3.2.1, expressed in units of 10-6. [FFS: Pending confirmation from SA5 if the measurement can be filtered per DRB per UE] |

Proposal 5: RAN3 to agree to the new definition for the Packet loss rate for UE performance metric.

As agreed in RAN3-128, a new should be introduced for DL and UL Packet Loss Rate at PDCP Level in R19, however, the normative specification of this IE is pending clarification from SA5 on the LS sent during the previous meeting (R3-253816). As this meeting is the last before Release 19 discussions are concluded, it is proposed to specify the IEs with a suitable note in case the response from SA5 does not arrive in time.

Proposal 6: RAN3 to introduce packet loss rate measurements at the PDCP level and signal the collected measurements over the newly introduced Data Collection procedures.

A TP reflecting the main proposals in this paper is presented in R3-255477.

# 3. Conclusions

This paper makes the following observations and proposals:

Proposal 1: RAN3 to acknowledge and to design a solution which avoids the following issues with the Average Packet Loss DL IE in UE performance:

- The Packet Loss Rate IE has been introduced as part of the GBR QoS Flow Information IE to indicate a maximum upper limit for the Loss Rate in UL/DL and not to indicate the measured values of such metric

- Average Packet Loss DL IE (which reuses the Packet Loss Rate IE definition) is defined at a per-UE granularity, while the Packet Loss Rate IE is defined at a per-QoS-Flow level

- The Average Packet Loss DL IE, in its current form is too coarse to represent real-world expected packet loss in 5G.

- Modifications to the definition of the Packet Loss Rate IE has an impact on both the GBR QoS Flow Information and on UE Performance information.

Proposal 2: The new definition of Packet loss rate for UE performance shall accommodate for a range of packet loss rate values from 10-10 to 10-1.

Proposal 3: RAN3 to make a working assumption that the equivalent per DRB per UE measurements of the UL PDCP SDU Loss Rate and DL PDCP SDU Drop Rate in gNB-CU-UP will be defined by SA5.

Proposal 4: The new definition of packet loss rate for UE performance shall accommodate a granularity that is detailed enough to use for applications with very low packet loss rates.

Proposal 5: RAN3 to agree to the new definition for the Packet loss rate for UE performance metric.

Proposal 6: RAN3 to introduce packet loss rate measurements at the PDCP level and signal the collected measurements over the newly introduced Data Collection procedures.

A TP reflecting the main proposals in this paper is presented in R3-255477.