3GPP TSG-RAN WG3 Meeting #129	R3-255482

Bengaluru, India, 25th – 29th August 2025

Agenda Item:	16.2

Source: 				LG Electronics Inc.

Title:			Discussion on remaining open issues in A-IoT

Document for:		Discussion and Decision

1.	Introduction

RAN2 sent a LS in [1] to provide the RAN2 agreement on the support of parallel service requests for overlapping reader scenario. Also, there are some remaining open issues for the inventory/command assistance information from the AIOTF. The detailed encodings of some IEs (i.e., Filter information, A-IoT correlation ID, AIOTF ID) need to be defined. In this contribution, we will discuss the remaining open issues in RAN3, and then provide our views on it.

2.	Discussion

2.1 Handling of parallel service requests for overlapping reader scenario

According to RAN2 LS [1], RAN2 had the discussion on the support of parallel service requests for overlapping reader scenario, and made the following agreements:

Agreement in RAN2#130: - Rel-19 devices are not expected to receive parallel service request for overlapping reader scenario based on network implementation. Capture this in RAN2 stage 2 specification. - The Rel-19 device always responds to the new service indicated by the received paging message applicable for that device. Capture this in RAN2 stage 3 specification.

According to RAN2 agreements, the A-IoT Device is expected to only perform one Inventory or Command procedure at a time. If the A-IoT Device receives parallel service request from multiple different readers having the overlapping areas, it should terminate the ongoing procedure and then respond to the new service request indicated by the latest A-IoT Paging message. Hence, it was agreed in RAN2 that the coordination among multiple readers is needed based on network implementation in order to avoid the transmission of parallel service requests at the same time. For example, since the gNB has the knowledge on the A-IoT Areas and location of the readers within the gNB, it may be able to properly control the resource allocation among overlapping readers based on implementation. As a result, we propose to capture the following NOTE into Clauses 16.xx.x3 and 16.xx.x4 of the RAN3 BL CR to TS 38.300 [2] to capture the RAN2 agreements for overlapping reader scenario.

- NOTE: It is up to the gNB implementation to make sure that the A-IoT device is not expected to receive parallel service requests from different readers with overlapping areas.

Regarding the case where different readers are served by different gNBs, we think that since the resource coordination between gNBs is not supported in this release, this can be handled by the AIOTF based on implementation. However, because the AIOTF behaviour is within SA2 remit, the scenario related to the overlapping readers served by different gNBs may be resolved by SA2 based on RAN2 LS [1].

Proposal 1: It is proposed to add the following NOTE into Clauses 16.xx.x3 and 16.xx.x4 of the BL CR to TS 38.300 [2] to reflect the RAN2 agreements for overlapping reader scenario:

- NOTE: It is up to the gNB implementation to make sure that the A-IoT device is not expected to receive parallel service requests from different readers with overlapping areas.

Based on the above proposal, RAN3 should provide the feedback to the RAN2 on how to reflect RAN2 agreements to the RAN3 specification. The above proposal is captured into a draft reply LS in Appendix#1. Therefore, the following proposal is suggested to RAN3:

Proposal 2: It is proposed to send a reply LS in Appendix#1 to RAN2 to provide the RAN3 feedback on overlapping reader scenario.

2.2 FFS on inventory assistance information

In last RAN3 meeting, it was agreed to define the Expected D2R Message Size IE in Inventory Request Transfer IE based on the RAN2 LS [3]. However, it is still FFS on the encoding and presence of this IE in order to wait for other WG’s progress. In the Inventory procedure, the A-IoT Device sends its Device ID (i.e., the A-IoT Device Permanent Identifier) to the gNB by the Inventory Response D2R message, and then the gNB forwards the received Device ID to the AIOTF by using the INVENTORY REPORT message. We think that because the A-IoT Device Permanent Identifier is composed as follows, the maximum length of 256 bits is enough to represent the contents of the Inventory Response D2R message. Therefore, the Expected D2R Message Size IE needs to be defined as INTEGER (1..256, …) unit: bit.

Figure 1: Structure of AIoT Device Permanent Identifier in agreed CT4 CR [4]

In addition, according to the agreed SA2 CR [5] in SA2 Reply LS [6], the AIOTF always provides the size of the Inventory Response D2R message from the A-IoT Device to the gNB. Hence, the presence of this IE should be mandatory.

Proposal 3: The Expected D2R Message Size IE is always provided in the Inventory Request Transfer IE to the gNB, and is defined as INTEGER (1..256, …) with the unit: bit.

Another open issue is whether to define the Time Interval IE in the inventory assistance information provided to the gNB. According to the agreed SA2 CR [5], the AIOTF sets this IE with a value that is equal or shorter than the time received from the AF. If the Time Interval information is not provided from the AF, it is also possible that the AIOTF determines the time interval value for the aggregation based on local configuration, and then provides this value to the gNB. Upon the reception of the Time Interval IE in the assistance information, the gNB can aggregate multiple Inventory Response D2R messages related to a specific Correlation ID before sending the INVENTORY REPORT message to the AIOTF. That is, there is no need to send the INVENTORY REPORT message to the AIOTF whenever receiving the Inventory Response D2R message from the A-IoT Device. According to the agreed SA2 CR [5], since the AIOTF optionally sends this IE to the gNB, the presence of this IE should be optional. Also, we think that the encoding of the Time Interval IE can be defined as INTEGER (1..3600, …) unit: seconds.

Proposal 4: The Time Interval IE is optionally provided in the Inventory Request Transfer IE to the gNB, and is defined as INTEGER (1..3600, …) unit: seconds.

Based on the above proposals, RAN3 should provide the feedback to the RAN2 and SA2 on the encoding of the Expected D2R Message Size IE. In addition, it is needed to provide to the SA2 the RAN3 feedback on the Time Interval IE in the inventory assistance information. The above proposals are captured into a draft reply LS in Appendix#2. Therefore, the following proposal is suggested to RAN3:

Proposal 5: It is proposed to send a reply LS in Appendix#2 to RAN2 and SA2 to provide the RAN3 feedback on inventory assistance information.

2.3 FFS on command assistance information

The remaining FFS on the command assistance information provided to the gNB is whether to introduce the Command Type (write, read, disable). According to TS 23.369, the AIOTF includes the Command Type into A-IoT NAS PDU, and then sends it to the A-IoT Device via gNB (i.e., RAN reader). For now, the gNB (i.e., RAN reader) is not aware of which command type is triggered. However, if the command type is also provided to the gNB, the gNB can properly perform the resource scheduling considering the command type. For example, in “Read” case, the gNB can allocate enough A-IoT radio resources to transmit the D2R message containing the application data to read. In “Disable” case, it is possible for the gNB to allocate a small AIoT radio resources for the D2R message because the AIoT Device just responds to the gNB to indicate whether the disable command was successful.

Note that although the Command Type IE is not provided, the gNB may implicitly deduce the command type set by the AF based on the Estimate of Expected D2R Message Size IE in the command assistance information. However, when the Estimate of Expected D2R Message Size IE is not provided in the command assistance information, the gNB cannot perform the resource allocation considering the command type. Therefore, the AIOTF needs to explicitly forward to the gNB the command type received from the AF. As a result, the Command Type IE needs to be defined in the Command Request Transfer IE as ENUMERATED (write, read, disable, ...).

Proposal 6: The Command Type IE is optionally included in the Command Request Transfer IE as ENUMERATED (write, read, disable, ...).

2.4 FFS on interface management

For the interface management, it is still FFS on whether to introduce the A-IoT Support Indicator in the NG SETUP RESPONSE message. Similar to the NG SETUP REQUEST message, the NG SETUP RESPONSE message also contains the mandatory IEs (e.g., Slice Support List IE) not applicable for the A-IoT service. Therefore, the gNB needs to ignore these mandatory IEs not applicable for the A-IoT service. Because the A-IoT Support IE is already included into the NG SETUP REQUEST message, this IE can be also applicable to the NG SETUP RESPONSE message to indicate that the IEs not applicable for the A-IoT service should be ignored.

Proposal 7: The A-IoT Support IE is also included in the NG SETUP RESPONSE message.

2.5 FFS on encoding of IE

Regarding the encoding of the IE, some remaining open issues should be resolved. First FFS is the encoding of the Filter Information. According to the Clause of 5.12.2 of TS 29.571 and the agreed CT4 CR [4], this IE is defined as STRING with format “byte” and has the maximum length of 256 bits. However, since the Filter Information can be constructed by one or multiple filtering elements as described in TS 23.369, our understanding is that this IE has variable length. Based on the observations, the Filter Information (i.e., Group Device Identity IE in the A-IoT Device Identification Requested IE) needs to be defined as OCTET STRING. Also, the reference to Clause of 5.12.2 of TS 29.571 can be captured into the semantics description of this IE.

Table 5.12.2-1: Simple Data Types from TS 29.571

| Type Name | Type Definition | Description |
| --- | --- | --- |
| AiotAreaId | string | Represents the AIoT area identifier encoded as a string. |
| AiotFilteringInformation | Bytes | AIoT Filtering Information is used to identify or filter multiple AIoT Devices. See clause aa.3 of 3GPP TS 23.003 [7]. String with format "byte" as defined in OpenAPI Specification [3], i.e. base64-encoded characters. |
| AiotDevPermId | string | String that uniquely representing AIoT Device Permanent Identifier, as specified in clause YY.2 of 3GPP TS 23.003 [7]. Pattern: "^[A-Fa-f0-9]+$" |

Proposal 8: The Group Device Identity IE in the A-IoT Device Identification Requested IE is defined as OCTET STRING with a reference to TS 29.571 in the semantics description of this IE.

Regarding the encodings of the A-IoT Correlation Identifier IE and the AIOTF Identifier IE, RAN3 had decided to wait for other WG’s inputs in last meeting. As shown in the agreed CT4 CR [7], the aiotfInfoList IE is newly defined in the NFProfile IE. When the AIOTF is to be discovered via NRF according to its serving area information (i.e., the values in the aiotfInfoList IE), the NRF returns the NF instance profiles (i.e., the NFProfile IE) related to the discovered AIOTF to the NEF in order to enable the NEF to determine the AIOTF instances(s). Generally, the NfInstanceId IE is mandatorily included in the NF instance profiles to uniquely identify the NF Instance. Therefore, we think that the AIOTF Identifier IE can be defined as OCTET STRING with a reference to the NfInstanceId IE.

Proposal 9: The AIOTF Identifier IE is defined as OCTET STRING with a reference to the NfInstanceId IE.

For now, our understanding is that the A-IoT Correlation Identifier IE is not yet introduced in TS 29.518 and TS 29.571. Based on the definition, this IE is allocated to uniquely identify an A-IoT session context within the AIOTF. Therefore, we think that the A-IoT Correlation Identifier IE can be defined as INTEGER (0..65535,…).

Proposal 10: The A-IoT Correlation Identifier IE is defined as INTEGER (0..65535,…).

According to the TS 23.369, when the AIOTF requests to the AMF to send the A-IoT data towards the A-IoT device(s), the A-IoT Correlation Identifier IE and the AIOTF Identifier IE are included in the Namf_AIoT_MessageDelivery Request message. Upon the reception of this message, the AMF sends to the gNB the A-IoT related NGAP message containing the both IEs. As mentioned above, however, both the A-IoT Correlation Identifier IE and the AIOTF Identifier IE are not yet captured in TS 29.518 or TS 29.571. Therefore, it is needed to send the Reply LS to CT4 to provide the RAN3 agreements on the encodings of both the A-IoT Correlation Identifier IE and the AIOTF Identifier IE. The above proposal is captured into a draft reply LS in Appendix#3. Therefore, the following proposal is suggested to RAN3:

Proposal 11: It is proposed to send a reply LS in Appendix#3 to CT4 to provide the RAN3 agreements on the encodings of the A-IoT Correlation Identifier IE and the AIOTF Identifier IE.

In order to capture the above observations and proposals, the following proposal is also suggested to RAN3:

Proposal 12: It is proposed to agree the corresponding TP for BL CR to TS 38.300 [2] in Appendix #1 of [8].

Proposal 13: It is proposed to agree the corresponding TP for BL CR to TS 38.413 [9] in Appendix #2 of [8].

3.	Conclusion

In this contribution, we focused on the remaining open issues in RAN3, and then provided our views on it. The following proposals are kindly suggested to RAN3:

Proposal 1: It is proposed to add the following NOTE into Clauses 16.xx.x3 and 16.xx.x4 of the BL CR to TS 38.300 [2] to reflect the RAN2 agreements for overlapping reader scenario:

- NOTE: It is up to the gNB implementation to make sure that the A-IoT device is not expected to receive parallel service requests from different readers with overlapping areas.

Proposal 2: It is proposed to send a reply LS in Appendix#1 to RAN2 to provide the RAN3 feedback on overlapping reader scenario.

Proposal 3: The Expected D2R Message Size IE is always provided in the Inventory Request Transfer IE to the gNB, and is defined as INTEGER (1..256, …) with the unit: bit.

Proposal 4: The Time Interval IE is optionally provided in the Inventory Request Transfer IE to the gNB, and is defined as INTEGER (1..3600, …) unit: seconds.

Proposal 5: It is proposed to send a reply LS in Appendix#2 to RAN2 and SA2 to provide the RAN3 feedback on inventory assistance information.

Proposal 6: The Command Type IE is optionally included in the Command Request Transfer IE as ENUMERATED (write, read, disable, ...).

Proposal 7: The A-IoT Support IE is also included in the NG SETUP RESPONSE message.

Proposal 8: The Group Device Identity IE in the A-IoT Device Identification Requested IE is defined as OCTET STRING with a reference to TS 29.571 in the semantics description of this IE.

Proposal 9: The AIOTF Identifier IE is defined as OCTET STRING with a reference to the NfInstanceId IE.

Proposal 10: The A-IoT Correlation Identifier IE is defined as INTEGER (0..65535,…).

Proposal 11: It is proposed to send a reply LS in Appendix#3 to CT4 to provide the RAN3 agreements on the encodings of the A-IoT Correlation Identifier IE and the AIOTF Identifier IE.

Proposal 12: It is proposed to agree the corresponding TP for BL CR to TS 38.300 [2] in Appendix #1 of [8].

Proposal 13: It is proposed to agree the corresponding TP for BL CR to TS 38.413 [9] in Appendix #2 of [8].

4.	Reference

- R3-255015/R2-2504935, “LS on parallel service request”, RAN2(Xiaomi), May, 2025.

- R3-255072, “(BL CR to 38.300) Introduction of Ambient IoT”, CMCC, Huawei, Nokia, August, 2025.

- R3-253031/R2-2503199, “LS on D2R message size for inventory response”, RAN2(Interdigital), April, 2025.

- C4-252464, “Definition of AIoT Device Permanent Identifier”, Huawei, CATT, Ericsson, China Mobile, Nokia, Lenovo, May, 2025.

- S2-2505848, “Clause 5.4 Assistance Information Update”, Huawei, HiSilicon, InterDigital Inc., May, 2025.

- R3-255021/S2-2505849, “Reply LS on D2R message size for inventory response”, SA2(Interdigital), May, 2025.

- C4-252327, “Support registration and discovery of AIOTF”, Huawei, China Mobile, May, 2025.

- R3-255496, “(TP to TS 38.300 and 38.413) TP for A-IoT support”, LG Electronics, August, 2025.

- R3-255076, “(BL CR to 38.413) Introduction of Ambient IoT”, Huawei, Nokia, August, 2025.

- R3-255006/C4-252466, “Reply LS on paging ID length”, CT4 (CICT Mobile), May, 2025.

5.	Appendix#1: Reply LS for R2-2504935 [1]

This appendix provides the draft reply LS to RAN2 based on the proposals of this contribution.

----------------Start of the First Change---------------

3GPP TSG-RAN WG3 #129 R3-25xxxx Bengaluru, India, 25th – 29th August 2025 # Title: [Draft] Reply LS on parallel service request # Response to: R3-255015/R2-2504935 # Release: Rel-19 # Work Item: Ambient_IoT_solutions Source: RAN3 To: RAN2 Cc: SA2 Contact Person: #### Name: Seokjung Kim #### E-mail Address: seokjung.kim@lge.com Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org # Attachments: R3-255496 # 1 Overall description RAN3 thanks RAN2 for the LS on parallel service request. RAN3 would like to provide the following feedback to RAN2 regarding the overlapping reader scenario raised by RAN2. Since the gNB has the knowledge on the A-IoT Areas and location of the readers within the gNB, it can properly control the resource allocation among overlapping readers based on implementation. Therefore, RAN3 has decided to add the following NOTE into TS 38.300 BL CR to reflect the RAN2 agreements for overlapping reader scenario (see attached agreed TP for TS 38.300 BL CR). - NOTE: It is up to the gNB implementation to make sure that the A-IoT device is not expected to receive parallel service requests from different readers with overlapping areas. 2. Actions: To RAN2: ACTION: RAN3 kindly asks RAN2 to take the above into account. 3. Date of Next RAN3 Meetings: 3GPP TSG RAN3#129bis 13 – 17 October 2025 Prague, CZ 3GPP TSG RAN3#130 17 – 21 November 2025 Dallas, US

-----------------End of the First Change---------------

6.	Appendix#2: Reply LS for S2-2505849 [6]

This appendix provides the draft reply LS to SA2 based on the proposals of this contribution.

----------------Start of the First Change---------------

3GPP TSG-RAN WG3 #129 R3-25xxxx Bengaluru, India, 25th – 29th August 2025 # Title: [Draft] Reply LS on D2R message size for inventory response # Response to: R3-255021/S2-2505849 # Release: Rel-19 # Work Item: AmbientIoT-ARC Source: RAN3 To: SA2, RAN2 Cc: - Contact Person: #### Name: Seokjung Kim #### E-mail Address: seokjung.kim@lge.com Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org # Attachments: R3-255496 # 1 Overall description RAN3 thanks SA2 and RAN2 for the Reply LS on D2R message size for inventory response. RAN3 would like to further provide the following feedback to SA2 and RAN2. With respect to “Expected D2R Message Size” in the inventory procedure, RAN3 agreed to define the Expected D2R Message Size IE in the Inventory Request Transfer IE as INTEGER (1..256, …) with the unit: bit. In addition, since the NG-RAN is expected to always receive the Expected D2R Message Size IE in the Inventory Request Transfer IE, the presence of the Expected D2R Message Size IE is mandatory. Regarding the Time Interval IE, RAN3 had discussed the usage of this IE, and then concluded that the Time Interval IE is optionally provided in the Inventory Request Transfer IE to the gNB, and is defined as INTEGER (1..3600, …) unit: seconds. RAN3 has agreed the TP for TS 38.413 BL CR to reflect the above agreements (see attached agreed TP for TS 38.413 BL CR). 2. Actions: To SA2 and RAN2: ACTION: RAN3 kindly asks SA2 and RAN2 to take the above into account. 3. Date of Next RAN3 Meetings: 3GPP TSG RAN3#129bis 13 – 17 October 2025 Prague, CZ 3GPP TSG RAN3#130 17 – 21 November 2025 Dallas, US

-----------------End of the First Change---------------

7.	Appendix#3: Reply LS for C4-2502466 [10]

This appendix provides the draft reply LS to CT4 based on the proposals of this contribution.

----------------Start of the First Change---------------

3GPP TSG-RAN WG3 #129 R3-25xxxx Bengaluru, India, 25th – 29th August 2025 # Title: [Draft] Reply LS on paging ID length # Response to: R3-255006/C4-252466 # Release: Rel-19 # Work Item: Ambient_IoT_solutions Source: RAN3 To: CT4 Cc: SA2, RAN2, SA3 Contact Person: #### Name: Seokjung Kim #### E-mail Address: seokjung.kim@lge.com Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org # Attachments: R3-255496 # 1 Overall description RAN3 thanks CT4 for the Reply LS on paging ID length. RAN3 would like to further provide the following feedback to CT4. Regarding the encoding of the AIOTF Identifier IE, RAN3 has found that the NfInstanceId IE can be used as the AIOTF Identifier IE. Therefore, RAN3 agreed to define the AIOTF Identifier IE as OCTET STRING with a reference to the NfInstanceId IE. In addition, RAN3 agreed to define the A-IoT Correlation Identifier IE as INTEGER (0..65535,…). According to SA2 specification, when the AIOTF requests to the AMF to send the A-IoT data towards the A-IoT device(s), the A-IoT Correlation Identifier IE and the AIOTF Identifier IE are included in the Namf_AIoT_MessageDelivery Request message. Upon the reception of this message, the AMF sends to the gNB the A-IoT related NGAP message (e.g., INVENTORY REQUEST message) containing the both IEs. However, RAN3 has found that both the A-IoT Correlation Identifier IE and the AIOTF Identifier IE are not yet captured in TS 29.518 or TS 29.571. RAN3 has agreed the TP for TS 38.413 BL CR to reflect the above agreements (see attached agreed TP for TS 38.413 BL CR). 2. Actions: To CT4: ACTION: RAN3 kindly asks CT4 to take the above into account. 3. Date of Next RAN3 Meetings: 3GPP TSG RAN3#129bis 13 – 17 October 2025 Prague, CZ 3GPP TSG RAN3#130 17 – 21 November 2025 Dallas, US

-----------------End of the First Change---------------