3GPP TSG-RAN WG3 Meeting #129	R3-255488

Bengaluru, India, 25th ~29th Aug, 2025

Title:	LTM ping pong

Agenda Item:	10.2

Source:	Huawei, Qualcomm, CMCC, Jio Platforms

Document for:	Discussion

# 1	Introduction

The topic of ping pong detection for LTM was discussed in RAN3#128. This paper presents a possible solution for this issue.

# 2	Background

Solving ping pong was one of the main reasons the UE history information was introduced early in LTE. With the new architecture without a central coordinator (RNC) there was a need to carry information from the source to target cell regarding the UE’s past behaviour. This information could also be used to understand the mobility properties of the UE, i.e., how the UE moved, in order to better understand how to handle it in the future. All this was previously possible in the RNC.

Ping pong was also part of the discussions from SON from the beginning. There has however been no motivation to describe the general detection and mitigation for intra-system mobility in stage 2 specs, since it was assumed that the node receiving the UE history would be the node to detect and mitigate the problem.

Only one scenario was described, namely inter-system ping pong. This because it was felt that performing an inter system mobility action was quite costly and should also span cases where the UE does not necessarily return to the source cell but also to another cell in the same system. Therefore, a mechanism for detecting this when the UE returned to the system and informing the source node for the inter-system ping pong was introduced. This is done in the Handover Report.

The latest addition to UE history information was when SCG mobility was added. This enables the MN to include decisions on SCG mobility in the information sent to the target MN. In this discussion (which is similar to the discussion on LTM) it was considered valuable to send the full UE history information to the SN. Two benefits were seen for SN triggered mobility decisions: the ability to avoid ping-pong and the possibility to understand the mobility pattern of the UE. The MN was considered in charge for collecting information to be passed over to the next MN, with some assistance from the SN. Since the MN was not always aware of the intra-SN mobility, SN was assigned the task to collect this information and pass it to MN which correlated the information in to the full history which is sent to the next target MN.

In the RAN3#128 discussions, two different aspects where discussed.

- Should the information only be sent to the DU when a potential failure was detected

- How much information should be sent to the DU

Regarding the first aspect the arguments where mainly in two directions:

- Whether it would be valuable for the DU to receive this information also if no ping pong occurred. There were different view whether the information would be useful for the source DU when e.g. understanding the velocity of the UE to determine e.g. when to perform LTM cell switch or the validity of the TA acquisition.

- Whether it be valuable for the DU to only receive this information when an analysis is expected, e.g. to indicate that a potential ping-pong event occurred. Without this understanding, the gNB-DU may be forced to always perform this task

Regarding the second aspect, the discussion was mainly about what information was useful for the DU. It was agreed that we could at least remove the PSCell information. The remining issues seems to be whether we ned to only send information related to LTM cell switch events or the history also including legacy mobility events.

One additional aspect worth mentioning was that RAN3 agreed on the L3 measurement based LTM cell switch as a correction to Rel-18. The conclusion was that for cases where the UE is capable of performing LTM cell switch, but not capable to report L1 measurands, the solutions would be that either:

- CU decides when to perform cell switch, or

- CU provides the L3 measurements to the DU and the DU decides when to perform LTM cell switch.

This agreement may also impact MRO for LTM.

# 3	Discussion

Regarding the first issue, one potential compromise would be to allow the DU to send this information to the DU also if no issue is detected, but also have the possibility to indicate the occurrence of a potential issue. The benefit of this is that we would be able to re-use functionality already in the gNB-CU to determine that a ping pong has occurred and could relieve the implementation of DUs for these implementations that prefers to only analyse the information when a potential issue has occurred.

The important principle of letting the DU deciding whether and how to adjust this should however still apply. The benefit of letting the DU making the final decisions that it is only the DU who would know how much he would be able to delay the mobility. The Du would (using MRO) have an understanding on the allowed range for mobility.

Hence, this indication from the gNB-CU is only an indication and does not mandate any behaviour in the gNB-DU. Since there is no requirement on behaviour in the gNB-DU, there is also no need to define this behaviour in the CU (when he includes this indicator). It should also be allowed for the CU to send this notification to multiple DUs, so that the DU with the available adjustment margin can perform the optimization.

Regarding the second aspect, the information to be included is somewhat difference from legacy UE history. In the legacy, the information provided to the target during handover preparation is the full UE history that will be forwarded as the UE is moving through the system. In this case however, the information is not forwarded further from the CU. Hence, there is strictly no need include information that the CU believes is not relevant for the DU. This will however not be visible in the specifications (since there is no statement as store and forward). But it means that we are not mandated to include all information.

For example, it seems agreeable to not include PSCell information. One additional reasonable limitation would be to only include NG-RAN information.

In the discussion in R3#128, the options where to only include the last sequence of LTM cell switch events or including the full history with a new indicator to identify if an event was LTM cell switch or a legacy mobility event.

The agreement on L3 measurement based LTM cell switch somewhat complicates this discussion since we may now have the following options:

- L3 based legacy mobility event (controlled by the CU)

- L3 based LTM cell switch controlled by the CU

- L3 based LTM cell switch controlled by the DU

- L1 based LTM cell switch (controlled by the DU)

We do not think there is a need to differentiate which node performed the mobility decision (b/c) since the nodes sending/receiving (CU and DU) the UE history information would be aware of the roles. If the decision is in the CU there is no need to send this information to the DU. This leaves an indicator that has three code points:

- L1 based LTM cell switch

- L3 based LTM cell switch

- L3 based legacy (default case)

# 5	Potential impact

Based on the discussion in the previous section, the proposed changes are:

- Add the a new IE containing UE history information in CU-DU CELL SWITCH NOTIFICATION

- Clarify in semantics, that this IE does not include PSCell information

- Include one new optional enumerated IE indicating whether there is a potential mobility related issue. At the moment this extendable IE only contains the value “ping-pong”

- Include a new optional indicator in the UE history information (NGAP) indicating whether the mobility event was a L1 based LTM cell switch or L3 based LTM cell switch. The absence of the indicator means that the event was a legacy mobility event.

This is illustrated below.

Impact on F1AP

#### 9.2.2.16	CU-DU CELL SWITCH NOTIFICATION

This message is sent by the gNB-CU to inform the gNB-DU about the initiation of the cell switch command to the UE.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |
| Last Visited Cells | O |  | 9.3.1.x |  | YES | ignore |

#### 9.3.1.x	Last visited Cells

This IE contains information about cells that a UE has been served by in RRC_CONNECTED state prior to the target cell.

NOTE:	The definition of this IE is aligned with the definition of the UE History Information IE in TS 38.413 [5].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Last Visited Cell List |  | 1 |  | Most recent information is added to the top of this list |
| >Last Visited Cell Item |  | 1..<maxnoofCellsinUEHistoryInfo> |  | Most recent information is added to the top of this list |
| >>Last Visited NG-RAN Cell Information | M |  | OCTET STRING | Defined in TS 38.413 [5]. This IE does not contain the Last Visited PSCell ListIE |
| Mobility issue | O |  | ENUMERATED(ping-pong, ...) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinUEHistoryInfo | Maximum number of last visited cell information records that can be reported in the IE. Value is 16. |

Impact on NGAP

#### 9.3.1.97	Last Visited NG-RAN Cell Information

This IE contains information about a cell. In case of NR cell, this IE contains information about a set of NR cells with the same NR ARFCN for reference point A, and the Global Cell ID IE identifies one of the NR cells in the set. The information is to be used for RRM purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Global Cell ID | M |  | NG-RAN CGI 9.3.1.73 |  | - |  |
| Cell Type | M |  | 9.3.1.98 |  | - |  |
| Time UE Stayed in Cell | M |  | INTEGER (0..4095) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in seconds. If the duration is more than 4095s, this IE is set to 4095. | - |  |
| Time UE Stayed in Cell Enhanced Granularity | O |  | INTEGER (0..40950) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in 1/10 seconds. If the duration is more than 4095s, this IE is set to 40950. | - |  |
| HO Cause Value | O |  | Cause 9.3.1.2 | The cause for the handover. | - |  |
| Last Visited PSCell List |  | 0..<maxnoofPSCellsPerPrimaryCellinUEHistoryInfo> |  | List of cells configured as PSCells. Most recent PSCell related information is added to the top of the list. | YES | ignore |
| >Last Visited PSCell Information | M |  | 9.3.1.235 | The PSCell related information. | - |  |
| LTM cell switch | O |  | ENUMERATED (l1-based,l3-based,...) |  | YES | Ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPSCellsPerPrimaryCellinUEHistoryInfo | Maximum number of last visited PSCell information records that can be reported in the IE. Value is 8. |

# 3	Conclusion

This document propose a compromise solutions for UE history information for LTM. It is proposed to agree on the following:

Add a new IE containing UE history information in CU-DU CELL SWITCH NOTIFICATION. Clarify in semantics, that this IE does not include PSCell information and include one new optional enumerated IE indicating whether there is a potential mobility related issue. At the moment this extendable IE only contains the value “ping-pong”

Include a new optional indicator in the UE history information (NGAP) indicating whether the mobility event was a L1 based LTM cell switch or L3 based LTM cell switch. The absence of the indicator means that the event was a legacy mobility event.