3GPP TSG-RAN WG3 Meeting #129	R3-255491

Bengaluru, India, 25th ~29th Aug, 2025

Title:	LTM Near failure TA

Agenda Item:	10.2

Source:	Huawei, Ericsson, Jio Platforms

Document for:	other

# 1	Introduction

In last meeting, we agreed to include a network based mechanism where the target DU sends information to the source DU in case of:

- Beam failure recovery (near-failure)

- Failure due to outdated TA

- Failure due to beam selection

In this paper we propose to add a mechanism for almost outdated TA.

## 2. Discussion

One of the challenging tasks in LTM is to determine when to update the TA. The task itself involves the UE, the CU and the target DU, where the UE is instructed to perform a RACH towards a target cell. This is done prior to LTM cell switch decision and is therefore a task that would be started well before the source DU decides to perform the cell switch.

It is difficult for the source DU to know how long time a TA is valid. It is possible to learn from failure events (based on the agreed mechanism). But this still requires a failure.

Optimising the validity time for the UE seems to be very beneficial if it is possible to reduce the periodicity of TA acquisitions, while still maintaining a low error rate. It is also envisaged that a smart DU may choose different validity for each UE, e.g. depending on UE history or detected time difference while this DU is serving the UE. If the optimisation is performed more on an individual level, the knowledge that could be drawn from error events may not be enough and the node may benefit from additional information.

The proposed solution is therefore that the target DU evaluates how much into the cyclic prefix the initial transmission from the UE is received and how much the TA should be adjusted to achieve a perfect timing in the target cell. This would give an indication to how close to failure the TA used by the UE is. The nice thing with the proposed solution is that it matched the agreed framework. Instead of including the TA that was used during the RACH, is possible to indicate how close to failure the actually used TA was.

A near-failure solution can actually be more useful than the failure case solution, because the failure case solution only works when a cell switch fails and when the LTM recovery cell is the same as the target cell. However, the analysis of whether there is a near-failure in a cell switch works for all cell switches. Also, if near-failures are avoided, failures will be less likely.

In order to control the amount of signalling, it is proposed that OAM should define threshold (e.g. TA difference between detected TA and optimal TA) for when this information should be included. Only when the value is bigger than a certain threshold, the report should be sent.

The potential impact on the specifications from this solution is illustrated using the last version of the BLCR:

### 9.2.10	Self Optimisation Support Messages

#### 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Beam Failure Recovery Information | O |  |  |  | YES | ignore |
| >Recovery TCI State | M |  | OCTET STRING | Includes the TCI-StateId IE used at Beam Failure Recovery, as defined in TS 38.331 [8]. | - |  |
| >gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | - |  |
| LTM Cell Switch Failure due to Wrong Beam Information | O |  |  |  | YES | ignore |
| >Target Beam Information | M |  | FFS | FFS if it corresponds to the target beam index or TCI state ID. | - |  |
| >gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | - |  |
| TA Information (name is FFS) | O |  |  |  | YES | ignore |
| >TA Value | M |  | INTEGER (0..4095) | Indicates the TA value used at successful Random Access, as defined in TS 38.213 [31]. | - |  |
| >gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | - |  |
| Near failure TA |  | 0..1 |  |  | YES | ignore |
| >TA Value_difference | M |  | INTEGER (-4095..4095) | Indicates the delta of the TA value detected from the initial successful transmission from the UE | - |  |
| >gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | - |  |

Based on the above, we propose:

- Include a solution reporting of near failure due to outdated TA

# 3	Conclusion

This document proposes:

Proposal 1:	Include a solution reporting of near failure due to outdated TA