3GPP TSG-RAN WG3 Meeting #129	R3-255504

Bengaluru, India, 25 – 29 August 2025

Agenda Item:	20.2

Source:	ZTE Corporation

Title:	(TP to BL CR to 38.455) Support of AI/ML assisted positioning (case 3a)

Document for:	Discussions & Approval

1	Introduction

During the last meeting, following progress are reached:

Define the Positioning Data Collection Needed IE as a new bitmap. The requested information in the bitmap is timing measurement (UL-TDOA, gNB Rx-Tx Time Difference) together with optional LoS/NLoS indicator.

FFS if UE location can be requested.

The Positioning Data Collection Needed IE is introduced in the MEASUREMENT REPORT message.

Introduce POSITIONING DATA COLLECTION REPORT message as class 2 procedure including Positioning Data Information IE and RAN/LMF Measurement IDs

Introduce an optional Positioning Data Unavailable IE (FFS on encoding) when the Positioning Data Information IE is absent in the POSITIONING DATA COLLECTION REPORT message. The codepoints indicating the reason.

RAN3 confirms that Performance monitoring is done in the RAN in case 3a.

RAN3 confirms the WA: For case 3a data collection, Part A is known internally by the gNB and is not signalled over any network interface.

Introduce the new flag “Inferred measurement” in the TRP Measurement Result to align with RAN1 conclusion

FFS: if LMF should specifically request for AI/ML inferred measurement.

In this contribution, we focus on discussing on the left issues for AI/ML assisted positioning (case 3a) based on the latest RAN1’s agreement.

2	Discussion

2.1 Encoding of Positioning data unavailable

It was agreed to introduce an optional Positioning Data Unavailable IE when LMF can not provide the requested positioning data (part B). And it is FFS on encoding of this IE. Companies suggests to indicate the reason why LMF can not provide the part B information, e.g., no data available, user consent not granted, positioning failure, etc. The same mechanism is also applied in the AI/ML for NG-RAN topic, when the target NG-RAN node can not provide the requested information, it can provide the cause value to the source NG-RAN node, i.e., requested object is not supported, requested object is not temporarily available. Therefore, when LMF cannot provide the part B information to NG-RAN node, “no data available” can be considered as one of the reasons. Additionally, part B information may not be provided because of user consent. Thus, the “user consent not granted” can also be considered as an acceptable reason.

However, with respect to “positioning failure”, we don't think it is necessary. Considering that when positioning failure happens, the legacy positioning failure message will be triggered. Then, NG-RAN node and LMF can know this failure situation. It is meaning less that LMF notify NG-RAN node again that positioning process is failed.

Proposal 1	Using the codepoint “no data available” and “user consent not granted” to encode the Positioning Data Unavailable IE.

2.2 AI/ML request from LMF

Introduce the new flag “Inferred measurement” in the TRP Measurement Result to align with RAN1 conclusion

FFS: if LMF should specifically request for AI/ML inferred measurement.

As we agreed that RAN3 introduces the new flag “Inferred measurement” in the TRP Measurement Result. And there is FFS if LMF should specifically request for AI/ML inferred measurement. Logically, if AI/ML positioning is considered as a positioning method, the LMF can inform the NG-RAN node that it requests an inferred measurement. But if the LMF indicates in the request message that AI/ML is requested for inferred measurement, why does the NG-RAN node still need to use a flag to distinguish between legacy measurement and inferred measurement? It seems redundant.

Proposal 2	RAN3 is kindly asked to discuss why does the NG-RAN node still need to use a flag to distinguish between legacy measurement and inferred measurement when the LMF indicates in the request message that AI/ML is requested for inferred measurement.

2.3 UE location

FFS if UE location can be requested.

In the legacy positioning procedure, it is not allowed for NG-RAN node to measure/obtain the UE location. Therefore, if we consider that data collection is a kind of optimistic solution, i.e., data collection during on-going positioning. It is better to follow the legacy principle that NG-RAN node can not obtain the actual UE location to avoid the issue of user privacy.

Proposal 3	Not to consider UE location as one of the requested data.

Agree the corresponding TP to 38.455

3	Conclusion

Following is our observations and proposals:

Proposal 1	Using the codepoint “no data available” and “user consent not granted” to encode the Positioning Data Unavailable IE.

Proposal 2	RAN3 is kindly asked to discuss why does the NG-RAN node still need to use a flag to distinguish between legacy measurement and inferred measurement when the LMF indicates in the request message that AI/ML is requested for inferred measurement.

Proposal 3	Not to consider UE location as one of the requested data.

4	Text Proposal

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 9.1.X	Messages for Positioning Data Collection Information Transfer Procedures

#### 9.1.X.1	POSITIONING DATA COLLECTION REPORT

This message is sent to report positioning information for data collection.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| Positioning Data Information | O |  | 9.2.Z2 |  | YES | ignore |
| Positioning Data Unavailable | O |  | ENUMERATED( no data available, user consent not granted, …) |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>