3GPP TSG-RAN WG3 Meeting #129															R3-255508

Bengaluru, India, 25th - 29th Aug, 2025

Agenda Item:	14.3

Source:	Huawei, Ericsson, Thales, Jio Platforms, Deutsche Telekom

Title:	(TP to BL CR for TS 38.300) Hard FLSO and AMF management

Document for:	discussion

# 1	Introduction

In this paper, we continue to discuss the NG suspend/resume indication by clarifying few aspects on AMF management which seems to be not well understood, then need to further clarified and probably captured in stage 2, TS 38.300 for the NTN regenerative payload.

# 2	Discussion

## 2.1. Discussion history

For the feeder link switch over (FLSO), RAN3 already made the following agreements.

Feeder link switch issue:

RAN3 assumes that during the feeder link switch, AMF is not changed for the UEs. Whether there has any standard impact needs to be further checked.

The current mechanisms including, e.g. multiple TNLA association, are sufficient for TNL management during feeder link switch.

In the RAN3#127bis meeting, it was agreed that no new NGAP suspend/resume procedures are needed in R19 as follows:

- No new NGAP suspend/resume procedures in R19.

## 2.2. AMF awareness of hard FLSO

There are multiple ways for the AMF to be aware the the exact moment when the Hard FLSO occurs:

- The AMF can calculate the moment of Hard FLSO based on some information from OAM, such as the satellite ephemeris, ground station position, and the minimum elevation angle of the ground station.

- The AMF can use some information from the underlying connection, such as when the SCTP connection of NGAP is shutdown, the AMF could know that the connection to the ground station has been disconnected.

- The AMF can obtain some information associated with the FLSO from the NTN Control function, such as when to suspend data or when to resume transmission.

With regards to the argumentation developed last meeting RAN3 #128, on “Since AMF cannot know the actual time for hard FLSO”, due to meteorology or other conditions, we will refer to [1], observations 8 to 10, where a complete argumentation is developed to demonstrate that arguments are not relevant. We would also remind online comment of RAN3#128 that the NTN System is resilient and robust, then even in difficult conditions, the hard FLSO is known some minutes before the FLSO.

We propose then RAN3 to clarify this aspect in Stage 2.

- RAN3 to suggest to introduce the following into the stage 2 for hard feeder link switchover

- The point in time when the hard feeder link switch over is performed at the AMF is pre-configured or determined by the NTN control function.

## 2.3. No AMF change during FLSO

The previous meeting also had the following agreement, to specify that the AMF is not changed during the FLSO.

RAN3 assumes that during the feeder link switch, AMF is not changed for the UEs. Whether there has any standard impact needs to be further checked.

We consider this is an important agreement applicable for all RRC state UEs, which should be captured in the stage 2.

- RAN3 to agree the Stage 2 TP to capture that during the feeder link switch, AMF is not changed for the UEs (including idle/inactive/connected UEs).

# 3	Conclusion

This contribution discusses the NR NTN regenerative payload required clarification. It is proposed:

- RAN3 to suggest to introduce the following into the stage 2 for hard feeder link switchover

- The point in time when the hard feeder link switch over is performed at the AMF is pre-configured or determined by the NTN control function.

- RAN3 to agree the Stage 2 TP to capture that during the feeder link switch, AMF is not changed for the UEs (including idle/inactive/connected UEs).

The TP are proposed in Annex A

# 4	References

- R3-25xxxx, Downlink NG Transmission Suspend/Resume, Ericsson , Thales, Huawei et al.

# Annex A: TP for TS 38.300 against For AMF Management

<<<<<<<<<<<<<<<<<<<< Change Begins >>>>>>>>>>>>>>>>>>>>

#### 16.14.4.2	Assumptions

A feeder link switch over may result in transferring the established connection for the affected UEs between two gNBs.

For soft feeder link switch over, an NTN payload is able to connect to more than one NTN Gateway during a given period, i.e. a temporary overlap can be ensured during the transition between the feeder links.

For hard feeder link switch over, an NTN payload connects to only one NTN Gateway at any given time, i.e. a radio link interruption and/or NG link interruption may occur during the transition between the feeder links.

In this version of the specification, for regenerative payload, the AMF is not changed for an NTN payload serving UEs in all RRC state during the feeder link switch over.

#### 16.14.4.3	Procedures

The NTN Control function (see Annex B.4) determines the point in time when the feeder link switch over is performed. The transfer of the affected UE(s) at feeder link switch over is performed by means of handover or TNL functionality, and it depends on the gNBs' implementation and configuration information provided to the gNBs by the NTN Control function.

NOTE:	In case of TNL address change due to feeder link switchover, existing procedures (e.g., Path Switch Request procedure, PDU Session Resource Modify Indication procedure) may be used.

For regenerative payload, an indication to suspend and resume data and signaling procedures over NG interface may be sent to AMF from gNB in the RAN Configuration Update procedure when the hard feeder link switch over occurs.

Editor’s Note: FFS whether this text is needed.

The point in time when the hard feeder link switch over is performed at the AMF may be configured or determined by the NTN control function.

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>