3GPP TSG-RAN WG3 Meeting #129															R3-255512

Bengaluru, India, 25th - 29th Aug, 2025

Agenda Item:	15.2

Source:	Huawei, Jio Platforms

Title:	(TP for TS 36.300) Full eNB as Regenerative Payload - TNL aspetcs

Document for:	Other

# 1	Introduction

This contribution proposes TP to TS 36.300 on the TNL aspects during the feeder link switch for IoT NTN Regenerative payload to clarify IoT NTN.

# 2	Discussion

At the previous RAN3-127bis meeting [1], given that the AMF is not changed during the feeder link switch over for the regenerative payload, it was agreed that the transfer of the affected UE(s) could be performed by means of handover or TNL functionality for NR NTN.

Accordingly, the following texts are introduced in the section 16.14.4 Switchover in the BLCR [2].

#### 16.14.4.3 Procedures The NTN Control function (see Annex B.4) determines the point in time when the feeder link switch over is performed. The transfer of the affected UE(s) at feeder link switch over is performed by means of handover or TNL functionality, and it depends on the gNBs' implementation and configuration information provided to the gNBs by the NTN Control function.

For the full eNB as regenerative payload, the same agreement can be taken for the transfer of the affected UEs. The stage 2 BLCR for IoT NTN already made the following changes, but the key aspect on the TNL functionality is still missing.

#### 23.21.5.3 Procedures The NTN control function determines the point in time when the feeder link switchover is performed. For BL UEs and UEs in enhanced coverage, the transfer of the affected UEs at feeder link switchover is performed by means of handover, and it depends on the eNBs' implementation and configuration information provided to the eNBs by the NTN Control function.

Indeed multiple SCTP is not supported on LTE, but as SA2 already pointed the TR 23.700-29, S1 can use the UE Retention Information IE. Also, according to the TS 38.412, the SCTP multi-homing could be used for the transport network redundancy.

- RAN3 to agree that for eNB as regenerative payload, the transfer of the affected UE(s) at feeder link switch over is performed by means of handover or TNL functionality, and agree the stage 2 TP.

# 3	Conclusion

This paper provides analysis for the eNB regenerative payload for FLSO, the following proposal is made:

- RAN3 to agree that for eNB as regenerative payload, the transfer of the affected UE(s) at feeder link switch over is performed by means of handover or TNL functionality, and agree the stage 2 TP.

# 4 Reference

- RAN3 #127bis chairman notes.

- R3-252523 Support for Regenerative Payload and MBS broadcast in NR NTN, Ericsson et al.

# 5	TP for TS 36.300 against R3-253973

<<<<<<<<<<<<<<<<<<<< Change Begins >>>>>>>>>>>>>>>>>>>>

### 23.21.5	Switchover

#### 23.21.5.1	Definitions

A feeder link switchover is the procedure where the feeder link is changed from a source NTN Gateway to a target NTN Gateway for an NTN payload. The feeder link switchover is a Transport Network Layer procedure.

Both hard and soft feeder link switchover are supported in transparent or regenerative NTN payloads.

#### 23.21.5.2	Assumptions

A feeder link switchover may result in transferring the established connection for the affected UEs between two eNBs.

For soft feeder link switchover, an NTN payload is able to connect to more than one NTN Gateway during a given period i.e. a temporary overlap can be ensured during the transition between the feeder links.

For hard feeder link switchover, an NTN payload only connects to one NTN Gateway at any given time i.e. a radio link interruption and/or S1 link interruption may occur during the transition between the feeder links.

#### 23.21.5.3	Procedures

The NTN control function determines the point in time when the feeder link switchover is performed. For BL UEs and UEs in enhanced coverage, the transfer of the affected UEs at feeder link switchover is performed by means of handover or TNL functionality, and it depends on the eNBs' implementation and configuration information provided to the eNBs by the NTN Control function.

NOTE:	In case of TNL address change due to feeder link switchover, existing procedures (e.g., Path Switch Request procedure, E-RAB Modification Indication procedure) may be used.

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>