3GPP TSG-RAN WG3 Meeting #129															R3-255513

Bengaluru, India, 25th - 29th Aug, 2025

Agenda Item:	15.2

Source:	Huawei, Jio Platforms, Ericsson

Title:	(TP for TS 36.300) Discussion on S&F mode transition

Document for:	Discussion

# 1	Introduction

In this paper, we discuss the the S&F mode transition and propose to clarify some aspects.

# 2	S1 S&F mode transition

Previous RAN2 meetings discussed the S&F operation mode transition with the following agreements.

RAN2-129 agreements: 1. RAN2 currently assumes that a S&F mode changes does not necessarily imply a MME change 2. The already agreed time information on transition from S&F operation mode to normal mode is provided in SIB31 (can work on signalling optimizations in case the new time information and T-service would carry the same information) 3. Use absolute UTC time to broadcast the (S&F-> normal mode) time information. When this information is broadcast and indicates a time in the future the UE can assume the NW is currently operating in S&F mode (until the indicated S&F-> normal mode transition time). 4. A change to the (S&F-> normal mode) time indication shall not result in system information change notification. Legacy rules for acquiring SIB31 apply RAN2-129bis agreements: 1. For UEs supporting S&F, the UE AS indicates the information on transition time (if any) from current "S&F operation mode" to "normal mode" (if received) to the upper layers. Whether/how it is used by the upper layers is up to CT1. 2. We introduce an indication in system information for the normal mode to S&F mode transition, at least for NAS use. FFS on the details (e.g. whether we can link this to other existing information). The information on transition time for the normal mode to S&F mode transition is sent from AS to NAS 3. We don’t introduce any new release cause related to S&F operation RAN2-130 meeting 1. When the S&F operation indication is present, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the normal mode will start; otherwise, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the S&F mode will start. 2. In a S&F network deployment which also exhibits discontinuous coverage, existing mechanisms to handle discontinuous coverage can be leveraged (e.g. satellite assistance information, UE not needing to perform idle mode tasks when the UE determines that is out of coverage, etc.). There is no need to modify existing discontinuous coverage features due to the addition of S&F Satellite operation. FFS if we clarify in discontinuous coverage procedure in idle mode that the UE also takes into account the information about NAS configured S&F monitoring list. 3. Clarify in Stage 2 from Rel-18 (so in general, not only for S&F) that the values used in “satelliteId-r17” are expected to correspond to the values used in “satelliteId-r18” in other SIBs (e.g. the value used to identify the serving satellite in SIB33, SIB32 and SIB31 should be the same). Come back in the next meeting with an actual Rel-18 CR for this. FFS if in Rel-19 we need to say anything about the correspondence between the “satelliteId” IE and “S&F Monitoring List”. Working Assumption: 1. In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not (FFS if we also include the transition

According to the above, RAN2 assumes that a S&F mode transition does not necessarily imply an MME change.

RAN2 has assumed that a S&F mode change does not necessarily imply an MME change.

Building on this assumption, further analysis is provided from RAN3 perspective as follows.

First, for the transition from the S&F mode to normal mode, for those served UEs served by the on-board eNB before the mode transition, they were already served by the on-board CN. Clearly there is no strong motivation to update the MMEs. There is no need for the eNB to trigger the S1 based handover to update the MME for connected UEs. And there is no need to update the broadcast TAC list to trigger the UE to perform TAU update. Otherwise, this would lead to a lot of signalling overload, which is not favourable for the satellite communication.  Hence given the interfaces between the on-board eNB and on-board CN are still maintained, there is no need to update the MME for all RRC state UEs.

Similarly, for the transition from the normal mode to the S&F mode, for those UEs already served by the on-board eNB before the mode transition, it is better to select the on-board CN. Otherwise, the same issues as analysed above also apply here.

Therefore, when the S&F mode changes, there is no need to switch down/on the on-board CN and change the UE’s serving MME. Then the following proposals are made.

- RAN3 to agree that during the S&F mode transition (i.e., from the S&F mode to the normal mode or vice versa), the MME is not changed for its served UEs (for all connected/idle UEs).

- RAN3 to agree the stage 2 TP to capture the above and some further clean-ups given this is last R19 WI meeting (see Annex).

Given that this agreement may have SA2 impact considering the MME change also involves SA2 work, and SA2 is also discussing the S&F mode transition, it is better to inform SA2 our agreements.

- RAN3 to send an LS to SA2 to inform the agreements as described in Proposal 1 in LS R3-255516.

# 3	Conclusion

This paper provides analysis for the eNB regenerative payload, the following proposals are made:

- RAN2 has assumed that a S&F mode change does not necessarily imply an MME change.

- RAN3 to agree that during the S&F mode transition (i.e., from the S&F mode to the normal mode or vice versa), the MME is not changed for its served UEs (for all connected/inactive/idle UEs).

- RAN3 to agree the stage 2 TP to capture the above and some further clean-ups given this is last R19 WI meeting (see Annex).

- RAN3 to send an LS to SA2 to inform the agreements as described in Proposal 1 in LS R3-255516

# 4. Reference

- RAN3 #127-128 chairman notes.

- R3-247016 Reply LS on Support of Regenerative-based Satellite Access, SA2

- RAN3 #127bis chairman notes.

# 5. Annex1: TP for TS 36.300 against R3-250861

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the following terms and definitions apply.

A2X communication: A communication to support A2X services leveraging PC5 reference points. A2X services are realized by various types of A2X applications, e.g. BRID or DAA.

>>Unchanged part skipped<<

FeMBMS: further enhanced multimedia broadcast multicast service.

FeMBMS/Unicast-mixed cell: cell supporting MBMS transmission and unicast transmission as SCell.

Feeder link: wireless transport link between the NTN Gateway and the NTN payload.

>>Unchanged part skipped<<

Hybrid cell: a cell broadcasting a CSG indicator set to false and a specific CSG identity. This cell is accessible as a CSG cell by UEs which are members of the CSG and as a normal cell by all other UEs.

Inter-satellite link: wireless transport link between two regenerative NTN payloads.

>>Unchanged part skipped<<

NTN payload: a network node, embarked on board a satellite or high altitude platform station, providing connectivity functions, between the service link and the feeder link. Considering E-UTRAN functions, the NTN payload may be a TNL node (transparent payload), or an eNB (regeneative payload).

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 4.12	Support of Non-Terrestrial Networks

E-UTRAN supports radio access over non-terrestrial networks for BL UEs, UEs in enhanced coverage and NB-IoT UEs. Support for non-terrestrial networks encompasses platforms that provide radio access through Geosynchronous orbits (GSO), Non-Geosynchronous Orbit (NGSO), which includes Low-Earth Orbit (LEO) and Medium Earth Orbit (MEO) or High Altitude Platform Systems (HAPS).

The Figures 4.12-1 and  4.12-X below illustrate examples of a Non-Terrestrial Network (NTN) providing non-terrestrial access by means of an NTN payload and an NTN Gateway, depicting a service link between the NTN payload and a UE, and a feeder link between the NTN Gateway and the NTN payload. In Figure 4.12.1, the payload is transparent; in Figure 4.12-X, the payload is regenerative and hosts an eNB.

Figure 4.12-1: Overall illustration of an NTN with transparent payload

Figure 4.12-X: Overall illustration of an NTN with regenerative payload hosting an eNB

NOTE:	Figures 4.12-1 and Figure 4.12-X illustrate an NTN; RAN4 aspects are out of scope.

The transparent NTN payload transparently forwards the radio protocol received from the UE (via the service link) to the NTN Gateway (via the feeder link) and vice-versa. The regenerative payload terminates the Uu interface (via the service link), S1 and X2 intefaces. The following connectivity is supported by the NTN payload:

-	A NTN Gateway may serve multiple transparent or regenerative NTN payloads;

-	A transparent or regenerative NTN payload may be served by multiple eNBs;

-	A regenerative NTN payload may terminate one or more inter-satellite links toward other regenerative payloads.

NOTE:	In this release, the transparent NTN-payload may change the carrier frequency, before re-transmitting it on the service link, and vice versa (respectively on the feeder link).

For NTN, the following applies in addition to Network entity related Identities as described in clause 8.2:

-	A Tracking Area corresponds to a fixed geographical area. Any respective mapping is configured in the RAN;

-	A Mapped Cell ID as specified in clause 23.21.5.

Three types of service links are supported:

-	Earth-fixed: provisioned by beam(s) continuously covering the same geographical areas all the time (e.g., the case of GSO satellites);

-	Quasi-Earth-fixed: provisioned by beam(s) covering one geographic area for a limited period of time and a different geographic area during another period of time (e.g., the case of NGSO satellites generating steerable beams);

-	Earth-moving: provisioned by beam(s) whose coverage area slides over the Earth surface (e.g., the case of NGSO satellites generating fixed or non-steerable beams).

With NGSO satellites, the eNB can provide either quasi-Earth-fixed cell coverage or Earth-moving cell coverage, while eNB operating with GSO satellites can provide Earth fixed cell coverage or quasi-Earth-fixed cell coverage.

Store and Forward satellite operation is supported as described in TS 23.401 [17].

NOTE:	In case of the Store and Forward mode transition, the MME(s) is not changed for the NTN regenerative payload served UEs.

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>>