3GPP TSG-RAN WG3 Meeting #129	R3-255521

Bengaluru, India, 25 – 29 August 2025

Agenda item:	8.1

Source:	Nokia

Title:	Handling of Radio Capability for Paging

WID/SID:	FS_NewRAT-Core, 5GS_ph1, TEI17

Document for:	Discussion and Decision

# 1	Introduction

At RAN3#129 the LS in [1] has been received:

RAN3 would like to ask SA2 that when the NG-RAN node receives the UE Radio Capability for Paging IE in the INITIAL UE CONTEXT SETUP REQUEST message, and if the UE Radio Capability for Paging Information does not contain paging information on all NG-RAN RATs that the UE supports, then should the NG-RAN node send to the AMF the paging information on the missing NG-RAN RATs or all NG-RAN RATs that the UE supports in the UE RADIO CAPABILITY INFO INDICATION message?

SA2 Answer 1: SA2 considers that “UE Radio Capability for Paging” is an Information Element that will be stored in AMF and transferred between NG-RAN and AMF. The AMF never interprets the information in it, nor processes it. Hence SA2 believe that the RAN needs to send the information as a single IE. AMF replaces the old IE, if there is one stored, with the new received IE. NG-RAN is responsible to check and upload the proper information (e.g. gNB may include NR and additionally E-UTRA information but does not include NB-IoT) to AMF. NG-RAN shall not just provide “missing NG-RAN RAT”.

This paper describes this issue and proposes a way forward.

# 2	Discussion

The topic was started two meetings ago in RAN3#127bis around the interpretation of the following gNB requirement from TS 23.501 clause 5.4.4.3:

#### 5.4.4.3	Paging assistance information

The paging assistance information contains UE radio related information that assists the RAN for efficient paging. The Paging assistance information contains:

a)	UE radio capability for paging information:

-	The UE Radio Capability for Paging Information contains information derived by the NG-RAN node (e.g. band support information) from the UE Radio Capability information. The AMF stores this information without needing to understand its contents.

As the AMF only infrequently (e.g. at Initial Registration) prompts the NG-RAN to retrieve and upload the UE radio capabilities i.e. UE Radio Capability information to the AMF and the AMF may be connected to more than one NG-RAN RAT, it is the responsibility of the NG-RAN to ensure that UE Radio Capability for Paging Information (which is derived by the NG-RAN node) contains information on all NG-RAN RATs that the UE supports in that PLMN. To assist the NG-RAN in this task, as specified in TS 38.413 [34], the AMF provides its stored UE Radio Capability for Paging Information in every NG-AP INITIAL CONTEXT SETUP REQUEST message sent to the NG-RAN.

At RAN3#127bis, RAN3 sent the LSout to SA2 in R3-252276 [2] to ask clarification on the radio capability for paging between two interpretations:

Interpretation 1

The AMF provides the UE Radio Capability for Paging IE in the INITIAL UE CONTEXT SETUP REQUEST message and if the UE Radio Capability for Paging Information does not contain paging information on all NG-RAN RATs that the UE supports, then the NG-RAN node send to the AMF the paging information on the missing NG-RAN RATs.

or

Interpretation 2

The AMF provides the UE Radio Capability for Paging IE in the INITIAL UE CONTEXT SETUP REQUEST message and if the UE Radio Capability for Paging Information does not contain paging information on all NG-RAN RATs that the UE supports, then the NG-RAN node send to the AMF the paging information on all NG-RAN RATs that the UE supports in the UE RADIO CAPABILITY INFO INDICATION message.

Interpretation 1 would mean that AMF receives in the UE Radio Capability Information message only the information on missing RAT compared to the current content of UE Radio Capability for Paging IE container stored in the AMF. This means that AMF would have to open the stored UE Radio Capability for Paging IE container, open the UE Radio Capability for Paging IE received in the UE RADIO CAPABILITY INFO INDICATION message and combine both.

This obviously goes against SA2 explanation that AMF should remain transparent to this container and not process it, as clearly stated in SA2 reply LS in S2-256082:

SA2 Answer 1: SA2 considers that “UE Radio Capability for Paging” is an Information Element that will be stored in AMF and transferred between NG-RAN and AMF. The AMF never interprets the information in it, nor processes it. Hence SA2 believe that the RAN needs to send the information as a single IE. AMF replaces the old IE, if there is one stored, with the new received IE. NG-RAN is responsible to check and upload the proper information (e.g. gNB may include NR and additionally E-UTRA information but does not include NB-IoT) to AMF. NG-RAN shall not just provide “missing NG-RAN RAT”.

SA2 in [1] clearly says that when the new UE Radio Capability for Paging IE is received in the UE RADIO CAPABILITY INFO INDICATION message, the AMF replaces the currently stored IE by the new one.

RAN3 should therefore agree interpretation 2.

Proposal 1: RAN3 to agree that when the AMF receives the new UE Radio Capability for Paging IE in the UE RADIO CAPABILITY INFO INDICATION message, the AMF replaces the currently stored IE by the new one.

It is beneficial that such clarification is made in our RAN3 TS 38.413. Indeed, if not clarified it is prone to interoperability issue between gNB vendor and AMF vendor if some gNB assumes that only missing RATs needs to be sent. It is proposed to agree corresponding CR for TS 38.413 and reply accordingly to SA2.

Proposal 2: RAN3 the CR to TS 38.413 in [3] and reply LS in [4].

# 3	Conclusion

This paper has investigated the handling of radio capability for paging in the light of received SA2 reply LS and proposed to align with SA2 as follows:

Proposal 1: RAN3 to agree that when the AMF receives the new UE Radio Capability for Paging IE in the UE RADIO CAPABILITY INFO INDICATION message, the AMF replaces the currently stored IE by the new one.

Proposal 2: RAN3 the CR to TS 38.413 in [3] and reply LS in [4].

# 4	References

- R3-255025, Reply LS on Handling of UE Radio Capability for Paging (S2-2506082)

- R3-252276, LS on Handling of UE Radio Capability for Paging

- R3-255524, Correction of Handling of UE Radio capability for paging

- R3-255522, Response LS on Handling of UE Radio capability for paging