3GPP TSG-RAN WG3 Meeting #129	R3-255537

Bengaluru, India, 25th – 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | - | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Stage 2 correction for Layer 2 reset at inter-gNB-DU LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, CATT, Jio Platforms |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In case of inter-gNB-DU LTM cell switch execution, the new serving gNB-DU does not know the ID of the old serving cell and cannot compare the L2 reset IDs of the new and old serving cell. Without this comparison, the new serving gNB-DU cannot be aware whether the UE performed Layer 2 reset. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update step 20 of the inter-gNB-DU LTM signalling procedure, where the gNB-CU can send the Old Serving Cell ID in the CU-DU CELL SWITCH NOTIFICATION F1AP message, to inform the new serving gNB-DU about the old cell serving the UE before the LTM cell switch. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In case of inter-DU cell switch execution, the new serving gNB-DU may have a different view compared to the UE in terms of L2 reset handling. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.473 CR 1581 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

#### 8.2.1.5	Inter-gNB-DU LTM

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for LTM. Figure 8.2.1.5-1 shows the inter-gNB-DU LTM procedure for intra-NR.

Figure 8.2.1.5-1: Inter-gNB-DU LTM

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB-DU containing measurements of neighbouring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2.	The gNB-CU determines to initiate LTM configuration.

3.	The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU(s) for each candidate cell, containing one candidate cell ID and the CSI resource configuration for subsequent LTM. The gNB-CU may provide the LTM configuration ID mapping list to the candidate gNB-DU(s). The gNB-CU may request PRACH resources from the candidate gNB-DU(s). The gNB-CU may request the candidate gNB-DU(s) to provide the lower layer configuration for the purpose of generating the reference configuration or provide the lower layer part of the reference configuration to the candidate gNB-DU(s).

4.	If the candidate gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configurations for the accepted target candidate cell.

NOTE 1:	The CU-initiated UE Context Modification procedure may be initiated for preparing candidate cells in the source gNB-DU as specified in step 3 and 4 in 8.2.1.4 Intra-gNB-DU LTM.

5.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including the information related to early sync and the LTM configuration ID mapping list for the accepted target candidate cell(s). The gNB-CU may send the updated CSI resource configuration to the source gNB-DU. The gNB-CU may inform the source gNB-DU about intra-DU L2 reset configuration.

6.	The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which includes an updated lower layer configuration, e.g., containing the updated CSI report configuration of the source cell.

7.	The gNB-CU may send a UE CONTEXT MODIFICATION REQUEST message for each candidate cell accepted in the candidate gNB-DU(s), containing the information for subsequent LTM or for updating the configurations of candidate cells. The gNB-CU may also provide the lower layer part of the reference configuration to the candidate gNB-DU(s). The gNB-CU may inform the candidate gNB-DU(s) about intra-DU L2 reset configuration.

8.	The candidate gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message including the updated lower layer configuration, e.g., containing the updated CSI report configuration of the requested candidate cell.

NOTE 2: Step 7 may also be triggered after step 19, or after step 22 by implementation for subsequent LTM.

9.	The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

10.	The source gNB-DU forwards the received RRCReconfiguration message to the UE.

11.	The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

12.	The source gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

12a. If LTM is triggered based on L3 measurement, gNB-CU sends a CU-DU MOBILITY INITIATION REQUEST message to the gNB-DU to trigger early synchronization and/or cell switch.

13.	Early TA acquisition to the candidate cell(s) may be performed as specified in TS 38.300 [2].

14.The candidate gNB-DU sends a DU-CU TA INFORMATION TRANSFER message to the gNB-CU, which includes the TA values, and the associated PRACH resource information.

15.	The gNB-CU forwards the TA value, and the associated PRACH resource information to the source gNB-DU in the CU-DU TA INFORMATION TRANSFER message.

16/ 17.	The UE sends the L1 measurement result to the source gNB-DU if L1 measurement is configured to UE. The source gNB-DU decides to execute LTM to a target cell.

18.	The source gNB-DU sends the Cell Switch Command to the UE.

19.	The source gNB-DU sends the DU-CU CELL SWITCH NOTIFICATION message to the gNB-CU to indicate the initiation of the Cell Switch Command to the UE, including the target cell ID and the TCI state ID(s). The TA value(s) related information applicable for subsequent LTM may also be included.

20.	The gNB-CU forwards in the CU-DU CELL SWITCH NOTIFICATION message to the target gNB-DU the target cell ID, the TCI state ID(s), and the TA value(s) related information received in step 19. The gNB-CU may also provide the old serving cell ID, and the target gNB-DU uses it for L2 reset handling.

21.	The target gNB-DU detects the UE access as specified in TS 38.300 [2].

22.	The target gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target cell ID.

23.	The UE sends an RRCReconfigurationComplete message to the target gNB-DU.

24.	The target gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

25.	The gNB-CU may send the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of prepared cells.

26.	The source gNB-DU responds with a UE CONTEXT RELEASE COMPLETE message.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>