3GPP TSG RAN WG3 Meeting #129	R3-255552

Bengaluru, India, 25 - 29 August, 2025

Agenda item:	8.1

Source:	Ericsson

Title:	Discussion on the LS from SA2 on emergency call back and paging

Document for:	Discussion

# 1	Introduction

This paper discusses the received LS from SA2 on emergency call back and paging [1].

# 2	Discussion

## SA2 LS questions:

SA2 provides the following questions to RAN3 in [1]:

- SA2 Q1： Is it possible to have different behaviour in NG-RAN node depending on the presence of the emergency PDU session to decide when to release RRC connection, compared to when there is no emergency PDU session? - SA2 Q2: If yes, when the emergency PDU session exist, whether it is required to configure the NG-RAN to not release RRC connection until the emergency PDU session is released by the SMF?

RAN3 has previously captured in RAN3#127bis (Wuhan April 2025) meeting a NOTE to TS 38.300 section 9.2.10 stipulating that the gNB which has identified the emergency PDU session should not configure eDRX for a UE released to RRC_INACTIVE and sent a LS to SA2 [2].

NOTE:	If emergency PDU session resources are established, the gNB should not configure the Extended DRX when sending the UE to RRC_INACTIVE state. The gNB recognizes the emergency PDU session resources based on special ARP value of a QoS flow as specified in TS 23.501 [3].

RAN2 have further replied on this topic to SA2 and RAN3 that the UE can also be kept in RRC_CONNECTED for fast emergency callback [3].

It is up to the gNB implementation whether to release the UE to RRC_IDLE or RRC_INACTIVE or keep the UE in RRC_CONNECTED state for fast emergency callback.

It would have been more future proof to keep the UE only in RRC_CONNECTED state during the duration of the emergency PDU session to avoid issues related to paging loss and latency (e.g., when using Rel-19 PEI or LP-WUS), but since the behaviour of NW during emergency PDU session differs from vendor implementation to another, it is not possible to mandate such configuration.

However, the current NOTE in TS 38.300 section 9.2.10 is misleading and incomplete as it is missing the RAN2 agreement that a UE could also be kept in RRC_CONNECTED for fast emergency callback [3]. Therefore, it is proposed to update the previous TS 38.300 NOTE as follows:

NOTE: If emergency PDU session resources are established, the gNB should either keep the UE in RRC_CONNECTED state until the emergency PDU session is released by the SMF or not configure the Extended DRX when sending the UE to RRC_INACTIVE state. The gNB recognizes the emergency PDU session resources based on special ARP value of a QoS flow as specified in TS 23.501 [3].

Observation 1: RAN3 has previously clarified that when the UE has an emergency PDU session the gNB should not configure eDRX for that UE in RRC_INACTIVE. RAN2 have further replied that the UE can also be kept in RRC_CONNECTED for fast emergency callback.

Proposal 1: Revise the NOTE in TS 38.300 SECTION 9.2.10 to also capture RAN2 agreement.

TPs to TS 38.300 Rel-17 and Rel-18 specs are provided in [4] and [5], respectively.

As answer to SA2, RAN3 can provide the following feedback:

- SA2 Q1： Is it possible to have different behaviour in NG-RAN node depending on the presence of the emergency PDU session to decide when to release RRC connection, compared to when there is no emergency PDU session? RAN3 answer: As long as NG-RAN is aware of the Emergency PDU session, it should either keep the UE in RRC_CONNECTED state until the emergency PDU session is released by the SMF or not configure the Extended DRX when sending the UE to RRC_INACTIVE state. - SA2 Q2: If yes, when the emergency PDU session exist, whether it is required to configure the NG-RAN to not release RRC connection until the emergency PDU session is released by the SMF? RAN3 answer: When the NG-RAN is aware of the emergency PDU session, the NG-RAN can be configured to not release the RRC connection by implementation.

Proposal 2: RAN3 answers SA2 with following:

- Q1 answer: As long as NG-RAN is aware of the Emergency PDU session, it should either keep the UE in RRC_CONNECTED state until the emergency PDU session is released by the SMF or not configure the Extended DRX when sending the UE to RRC_INACTIVE state.

- Q2: answer: When the NG-RAN is aware of the emergency PDU session, the NG-RAN can be configured to not release the RRC connection by implementation.

A LS reply is provided in [6].

# 3	Conclusion

This contribution has provided the following observations and conclusions:

Observation 1: RAN3 has previously clarified that when the UE has an emergency PDU session the gNB should not configure eDRX for that UE in RRC_INACTIVE. RAN2 have further replied that the UE can also be kept in RRC_CONNECTED for fast emergency callback.

Proposal 1: Revise the NOTE in TS 38.300 SECTION 9.2.10 to also capture RAN2 agreement.

Proposal 2: RAN3 answers SA2 with following:

- Q1 answer: As long as NG-RAN is aware of the Emergency PDU session, it should either keep the UE in RRC_CONNECTED state until the emergency PDU session is released by the SMF or not configure the Extended DRX when sending the UE to RRC_INACTIVE state.

- Q2: answer: When the NG-RAN is aware of the emergency PDU session, the NG-RAN can be configured to not release the RRC connection by implementation.

# 4	Reference

- R3-255022, Reply LS on emergency call back and paging

- R3-252402 Reply LS to SA2, RAN2 on emergency call back and paging

- R2-2504847, Reply LS on emergency call back and paging, RAN2

- R3-255553, Clarification on emergency call back and UE RRC state, Draft CR to TS 38.300 (Rel-17)

- R3-255554, Clarification on emergency call back and UE RRC state, Draft CR to TS 38.300 (Rel-18)

- R3-255555, Draft Reply LS to SA2 (Ericsson)