3GPP TSG-RAN WG3 Meeting #129	R3-255563

Bangalore, India, 25 – 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.455 | CR | 0194 | rev |  | Current version: | 16.15.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correcting the IE extension definition for bandwidth SRS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Jio Platforms, China Telecom, CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2025-08-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are irregularities in current type definition for the BandwidthSRS type definition. TS 38.455 v16.15.0 defines the type FR1-Bandwith as follows (Rel-16 additions in green): BandwidthSRS ::= CHOICE { fR1 ENUMERATED {mHz5, mHz10, mHz20, mHz40, mHz50, mHz80, mHz100, ..., mHz15, mHz25, mHz30, mHz60}, fR2 ENUMERATED {mHz50, mHz100, mHz200, mHz400, ...}, choice-extension ProtocolIE-Single-Container { { BandwidthSRS-ExtIEs } } } TS 38.455 v17.10.0 defines the type FR1-Bandwith as follows (Rel-17 additions in blue): BandwidthSRS ::= CHOICE { fR1 ENUMERATED {mHz5, mHz10, mHz20, mHz40, mHz50, mHz80, mHz100, ..., mHz15, mHz25, mHz30, mHz60, mHz35, mHz45, mHz70, mHz90}, fR2 ENUMERATED {mHz50, mHz100, mHz200, mHz400, ..., mhz800, mHz1600, mHz2000 }, choice-extension ProtocolIE-Single-Container { { BandwidthSRS-ExtIEs } } } TS 38.455v18.6.0 defines the type FR1-Bandwidth as follows (Rel-18 additions in red) BandwidthSRS ::= CHOICE { fR1 ENUMERATED {mHz5, mHz10, mHz20, mHz40, mHz50, mHz80, mHz100, ... ,mHz160, mHz200, mHz15, mHz25, mHz30, mHz60, mHz35, mHz45, mHz70, mHz90}, fR2 ENUMERATED {mHz50, mHz100, mHz200, mHz400, ..., mHz600, mhz800, mHz1600, mHz2000 }, choice-extension ProtocolIE-Single-Container { { BandwidthSRS-ExtIEs } } } There will be interoperability problems when a sender node that has implemented TS 38.455 v18.6.0 sends, e.g., mHz160, mHz200 in FR1 IE and mHz600 in FR2 IE, which would be interpreted as mHz15, mHz25 as FR1 IE and mHz800 as FR2 IE, respectively, by a node that has implemented different releases of the specification. It is proposed to correct Rel-16 encoding by adding dummy values (purple pseudo revision marks) to avoid interoperability with nodes that have implemented other releases of the specifications: BandwidthSRS ::= CHOICE { fR1 ENUMERATED {mHz5, mHz10, mHz20, mHz40, mHz50, mHz80, mHz100, ... , rel-18-dummy1, rel-18-dummy2, mHz15, mHz25, mHz30, mHz60}, fR2 ENUMERATED {mHz50, mHz100, mHz200, mHz400, ...}, choice-extension ProtocolIE-Single-Container { { BandwidthSRS-ExtIEs } } } Impact Analysis Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it impacts encoding of the FR1 bandwith. This CR is functional BC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add dummy values to the encoding of FR1 IE in the Requested SRS Transmission Characteristics IE to ensure interoperability with nodes that have implemented future releases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interoperability issue may occur with nodes that have implemented future releases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.27, 9.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------- CHANGES START HERE --------------------------------------

### 9.2.27	Requested SRS Transmission Characteristics

This IE contains the requested SRS configuration for the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Number Of Periodic Transmissions | C-ifResourceTypePeriodic |  | INTEGER (0..500,…) | The number of periodic SRS transmissions requested. The value of ‘0’ represents an infinite number of periodic SRS transmissions. |  |  |
| Resource Type | M |  | ENUMERATED (periodic, semi-persistent, aperiodic, …) |  |  |  |
| CHOICE Bandwidth | M |  |  |  |  |  |
| >FR1 |  |  | ENUMERATED (5mHz, 10mHz, 20mHz, 40mHz, 50mHz, 80mHz, 100mHz, ... , rel-18-dummy1, rel-18-dummy2, 15mHz, 25mHz, 30mHz, 60mHz) | The codepoints ‘rel-18-dummy1’ and ‘rel-18-dummy2’ are ignored. |  |  |
| >FR2 |  |  | ENUMERATED (50mHz, 100mHz, 200mHz, 400mHz,…) |  |  |  |
| SRS Resource Set List |  | 0.. 1 |  |  |  |  |
| >SRS Resource Set Item |  | 1..< maxnoSRS-ResourceSets> |  |  |  |  |
| >>Number of SRS Resources Per Set | O |  | INTEGER (1..16,...) | The number of SRS Resources per resource set for SRS transmission. |  |  |
| >>Periodicity List |  | 0.. 1 |  |  |  |  |
| >>>Periodicity List Item |  | 1..<maxnoSRS-ResourcePerSet> |  |  |  |  |
| >>>>PeriodicitySRS | M |  | ENUMERATED (0.125, 0.25, 0.5, 0.625, 1, 1.25, 2, 2.5, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240, …) | Milli-seconds |  |  |
| >>Spatial Relation Information | O |  | 9.2.34 | This IE is ignored if the Spatial Relation Information per SRS Resource IE is present. |  |  |
| >>Pathloss Reference Information | O |  | 9.2.53 |  |  |  |
| >>Spatial Relation Information per SRS Resource | O |  | 9.2.60 |  |  |  |
| SSB Information | O |  | 9.2.54 |  |  |  |
| SRS Frequency | O |  | INTEGER(0..3279165) | NR ARFCN The carrier frequency of SRS transmission bandwidth. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifResourceTypePeriodic | This IE shall be present if the Resource Type IE is set to the value "Periodic". |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-ResourceSets | Maximum no of requested SRS Resource Sets for SRS transmission. Value is 16. |
| maxnoSRS-ResourcePerSet | Maximum no of SRS Resources per set. Value is 16. |

-------------------------------------- NEXT CHANGE --------------------------------------

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

-------------------------------------- SKIPPED TEXT UNCHANGED --------------------------------------

#### -- B

BandwidthSRS ::= CHOICE {

fR1		ENUMERATED {mHz5, mHz10, mHz20, mHz40, mHz50, mHz80, mHz100, ..., rel-18-dummy1, rel-18-dummy2, mHz15, mHz25, mHz30, mHz60},

fR2		ENUMERATED {mHz50, mHz100, mHz200, mHz400, ...},

choice-extension		ProtocolIE-Single-Container { { BandwidthSRS-ExtIEs } }

}

BandwidthSRS-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

-------------------------------------- END OF CHANGES --------------------------------------