3GPP TSG RAN WG3 Meeting #129	R3-255571

Bengaluru, India, 25 - 29 August, 2025

Agenda item:	20.2

Source:	Ericsson

Title:	Way forward for Proactive (NGAP-based solution) for AI/ML NG-RAN assisted positioning

Document for:	Discussion, approval

# 1	Introduction

This paper discusses how to proceed with the NGAP based solution for data collection in NG-RAN to support case 3a considering the agreements made in Rel-19.

# 2	Discussion

SA2 state in their reply LS [1] that “solution 2” described by RAN3 as the NGAP-based signalling solution is not possible in Rel-19 timeframe because their specifications are frozen.

This is unfortunate as it leads to an incomplete AI/ML framework. In fact, there is agreement in SA2 which allows LMF as model host to initiate data collection for its model anytime. There is also an agreement in RAN2 which allows UE as model host to initiate data collection for its model anytime. However, for case 3a the NG-RAN as model host cannot trigger data collection for its own model without relying upon a positioning session being already initiated by LMF.

Observation 1: Based on RAN2 and SA2 agreements, the LMF and UE, as model hosts, can initiate data collection for their own model anytime. However, the NG-RAN is lacking this freedom and must rely on a positioning session being already opened by LMF to ask for training data.

We believe that RAN3 should not prevent the NG-RAN from having in place a flexible mechanism for data collection and training. This was also in line with the agreements taken by RAN3 in RAN3#127 meeting:

  - The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.

  - For case 3a, it should be possible to collect data in both an “opportunistic” (gNB collects available data from ongoing positioning sessions) and “proactive” (gNB triggers data collection at the LMF, e.g., which may trigger positioning) manner.

  - For case 3a, it should be possible for the gNB to collect data for both served and non-served UEs.

Observation 2: RAN3 has previously agreed that the proactive solution (i.e., gNB triggers data collection at the LMF, e.g., which may trigger positioning) should be supported.

Hence, we propose to continue discussing the proactive solution (i.e., NGAP-based solution) in RAN3. Since SA2 have claimed that the solution cannot be supported in Rel-19 timeframe, it is proposed to capture that the NGAP proactive solution will be considered in Rel-20, as leftover of the Rel-19 AI/ML for PHY WID (as a RAN3 only objective) or as RAN3-led TEI20.

Proposal 1: RAN3 to work on NGAP proactive solution in Rel-20, as leftover of the Rel-19 AI/ML for PHY WID (as a RAN3 only objective) or as RAN3-led TEI20.

A LS reply to SA2 and RAN is provided in annex of this document.

# 3	Conclusions

In this paper, we have made the following observations and proposals:

Observation 1: Based on RAN2 and SA2 agreements, the LMF and UE, as model hosts, can initiate data collection for their own model anytime. However, the NG-RAN is lacking this freedom and must rely on a positioning session being already opened by LMF to ask for training data.

Observation 2: RAN3 has previously agreed that the proactive solution (i.e., gNB triggers data collection at the LMF, e.g., which may trigger positioning) should be supported.

Proposal 1: RAN3 to work on NGAP proactive solution in Rel-20, as leftover of the Rel-19 AI/ML for PHY WID (as a RAN3 only objective) or as RAN3-led TEI20.

A LS reply to SA2 and RAN is provided in annex of this document.

# 4	References

- R3-255023, Reply LS on AI/ML Positioning Case 3a, SA2(Nokia)

# 5	Draft LS to RAN, SA2

3GPP TSG RAN WG3 Meeting #129	R3-25xxxx

Bengaluru, India, 25 - 29 August, 2025

# Title:	[DRAFT] Reply LS on AI/ML Positioning Case 3a

# Response to:	Reply LS on AI/ML Positioning Case 3a (R3-255023/ S2-2505943)

# Release:	Release 19

# Work Item:	NR_AIML_air-Core

Source:	Ericsson (to be RAN3)

To:	SA2, RAN

Cc:	-

Contact Person:

Name:	Yazid Lyazidi

Tel. Number:

E-mail Address:	Yazid.lyazidi@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

RAN3 thanks SA2 for their LS response with feedback to RAN3 solutions on case 3a (NW-sided model, AI/ML-assisted positioning).

RAN3 has previously agreed to support a gNB hosting an AI/ML model for assisted positioning to trigger data collection from the LMF (e.g., which may trigger a positioning session). Since SA2 has concluded that this solution is not feasible in current frozen Rel-19, RAN3 will re-discuss the solution in Rel-20 timeframe as a leftover of Rel19.

2. Actions:

To SA2:

ACTION: 	RAN3 kindly asks SA2 and RAN to take the above RAN3 agreement into account .

3. Date of Next TSG RAN3 Meetings:

TSG-RAN3 Meeting #129bis	13-17 October 2025	Pragues, CZ

TSG-RAN3 Meeting #130	17-21 November 2025	Dallas, Texas (US)