3GPP TSG-RAN WG3 Meeting #129	R3-255577

Bengaluru, India, August 25th–29th, 2025

Agenda Item:	17.2

Source:	Ericsson

Title:	On-demand SSB SCell operation

Document for:	Discussion, Decision

# 1	Introduction

The revised WID for “Enhancements of network energy savings for NR” [1] in Rel-19 states the following objectives:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

This paper discusses the “on-demand SSB SCell operation” topic (Objective 1).

A complete list of RAN1 and RAN2 agreements on this objective is provided at the end of the paper.

# 2	Discussion

## 2.1	Observations and current state of discussion

In our previous paper [2], we observed the following:

- On-demand SSB SCell operation is a new SCell operation mode where SSB transmissions on an SCell are triggered when needed and indicated to the UE via RRC or MAC CE based signaling.

- On-demand SSB parameters are preconfigured to the UE via RRC based signaling.

- On-demand SSB transmission is tightly connected to SCell operation at the gNB-DU, and it is triggered by the gNB-DU.

- On-demand SSB transmission enables energy-efficient SCell operation (with sparse or no always-on SSB) in cases where SSB-less SCell operation is not possible.

Besides, in the summary of the offline discussion on on-demand SSB SCell operation at the RAN3#127 meeting [3], the following was noted:

For the allowing On-demand SSB SCell, we discussed the two way-forward as follows:

- WF1: No new List from the gNB-CU to the gNB-DU for “allowing OD SSB SCell” is needed, instead we can reuse, if needed, the Rel-18 existing “Cells Allowed to be Deactivated List” (CU to DU). Potential rewording of the procedure may be needed to make the “list” more general.

- WF2: It is the gNB-DU to decide how and when to perform “deactivation” or “OD SSB”.

But the above is not agreed, there are two concerns raised during the discussion:

- One concern is that gNB-DU does not always need the “CU to DU” assistance in order to go to the OD-SSB SCell operation.

- Another concern is how and whether to handle the transition from PCell normal operation to OD-SSB SCell only operation.

To be continued at the next meeting on the above basis.

During the RAN3#127-bis meeting, there was an initial understanding to move forward with WF1, as it was noted in the summary of the offline discussion on on-demand SSB SCell operation [4]:

It is proposed to take the following as baseline and to be discussed at the next meeting (no need to capture into the Chairman notes).

Take the following texts as baseline:

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may take it into account to enable the on-demand SSB SCell operation for the indicated cells when they are operating as SCells.

However, during the RAN3#128 meeting, there was still no consensus in how to move forward [5] in relation to the impact of OD-SSB SCell operation on the F1 interface.

On the other hand, there is a general agreement that OD-SSB SCell operation does not have any impacts on the Xn interface, as we explained in our previous paper [6].

In what follows, we first discuss the potential impact (of on-demand SSB SCell operation) on the F1 interface.

## 2.2	Discussion on potential F1 impact

In past RAN3 meetings, it was discussed whether the gNB-DU can autonomously decide on configuring on-demand SSB SCell operation or whether it needs some form of assistance from the gNB-CU, e.g., a list of cells in which on-demand SSB transmission is allowed.

We share the view that the gNB-DU does not always need assistance from the gNB-CU to apply on-demand SSB SCell operation, and we see no reason to limit the use of on-demand SSB SCell operation at the gNB-DU. It is understood that the gNB-DU configures (via RRC based signaling) and triggers (via RRC or MAC CE based signaling) on-demand SSB. On-demand SSB transmissions are additional SSB transmissions, in addition to legacy SSB transmissions, if any. Hence, the gNB-DU should be free to apply on-demand SSB as it sees fit, since it is the entity that is in control of SCell operation and that observes the effect on CA performance and NW energy consumption. The gNB-DU should not require permission from the gNB-CU for on-demand SSB SCell operation in a cell, similar to legacy, where the gNB-DU does not need assistance from the gNB-CU for SSB-less SCell operation (cf. [2]).

In legacy SSB-less SCell operation, the gNB-DU does not need assistance or permission from the gNB-CU for SSB transmission omission.

RAN3 to agree that the gNB-DU does not require permission from the gNB-CU for on-demand SSB SCell operation in a cell.

In this sense, we believe that the following text (proposed at a previous RAN3 meeting) is appropriate with “may” statement:

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may take it into account to enable the on-demand SSB SCell operation for the indicated cells when they are operating as SCells.

According to RAN1 agreements, L3 measurements on on-demand SSB are supported. However, further work on this topic is left up to RAN2 and RAN4. The usability of L3 measurements on on-demand SSB at the gNB-CU is limited. Since on-demand SSB is only for configured SCell, not for neighbor cell, L3 measurements on on-demand SSB can only be used to decide on SCell removal, not to decide on SCell addition (or swap). Thus, L3 measurements on on-demand SSB are not applicable to SCell candidates. For UE mobility decisions, in particular CA related mobility decisions, the gNB-CU either relies on L3 measurements on always-on SSB, Case #2, or NW deployment knowledge (i.e., blind SCell configuration), Case #1.

The usability of L3 measurements on on-demand SSB at the gNB-CU, e.g., for UE mobility decisions, in particular CA related mobility decisions, is limited.

Considering the limitations of on-demand SSB transmissions in Rel-19, we do not see sufficient motivation to introduce signaling over F1 to enable the gNB-CU to request the gNB-DU to transmit on-demand SSB, at least not in this release. On-demand SSB is transmitted by the gNB-DU in connection to SCell activation (solely controlled by the gNB-DU), e.g., to speed up the SCell activation process. The gNB-DU does not need an indication from the gNB-CU to trigger on-demand SSB transmission for this purpose.

Moreover, we see no reason for the gNB-DU to inform the gNB-CU about transmission of on-demand SSB since on-demand SSB transmission in a cell of a gNB-DU is only visible to and impacts (Rel-19 NES-capable) UEs served by that gNB-DU with that cell as an SCell. It is thus not obvious what the gNB-CU would do with such information. It was suggested by one company in [7] that the gNB-CU would know to expect L3 measurements. However, whether the UE reports L3 measurements depends on the reporting configuration (e.g., measurement events). Besides, it is not clear why the gNB-CU would need such a pre-warning for receiving L3 measurements. Instead, L3 measurements can be sent to the gNB-CU as in legacy.

Not to introduce further signaling over F1 for requesting on-demand SSB transmission (gNB-CU to gNB-DU) or informing about on-demand SSB transmission (gNB-DU to gNB-CU) in the current release.

We think that the on-demand SSB related configuration is best included in the gNB-DU-owned part of the UE configuration (e.g., in the cell config). As such, the legacy F1 signaling, e.g., the existing UE context management procedures, would suffice to enable the gNB-DU to configure the on-demand SSB related parameters and indicate the on-demand SSB transmission to the UE, like the de-/activation state of the on-demand SSB configuration (which is already agreed in RAN3). In that case, there would be no obvious RAN3 impact from on-demand SSB SCell operation.

RAN3 to assume introduction of the on-demand SSB related configuration in the gNB-DU-owned part of the UE configuration (but it is up to RAN2 to discuss and decide).

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	In legacy SSB-less SCell operation, the gNB-DU does not need assistance or permission from the gNB-CU for SSB transmission omission.

Observation 2	The usability of L3 measurements on on-demand SSB at the gNB-CU, e.g., for UE mobility decisions, in particular CA related mobility decisions, is limited.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN3 to agree that the gNB-DU does not require permission from the gNB-CU for on-demand SSB SCell operation in a cell.

Proposal 2	Not to introduce further signaling over F1 for requesting on-demand SSB transmission (gNB-CU to gNB-DU) or informing about on-demand SSB transmission (gNB-DU to gNB-CU) in the current release.

Proposal 3	RAN3 to assume introduction of the on-demand SSB related configuration in the gNB-DU-owned part of the UE configuration (but it is up to RAN2 to discuss and decide).

# 4	References

RP-251678, Revised WID on enhancements of network energy savings for NR

R3-250551, On-demand SSB SCell operation and other Energy Saving topics, Ericsson

R3-250800, Summary of Offline Discussion on on-demand SSB SCell operation, Huawei (Moderator)

R3-252357, Summary of offline discussions: OndemandSSB, Huawei (Moderator)

R3-253780, Network Energy Saving Summary of offline discussion, Ericsson (Moderator)

R3-253549, On-demand SSB SCell operation, Ericsson

R3-250129, (TP to BLCR for TS 38.473) Discussion on on-demand SSB SCell operation, Huawei

# 5	RAN1 and RAN2 agreements for Objective 1

  - RAN1: On-demand SSB for SCell operation

    - RAN1#116

Agreement

Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study:

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

- FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted.

FFS: Which scenario the above applies for

Agreement

RAN1 to strive for a common design for on-demand SSB operation considering all applicable CA configurations.

Agreement

For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios.

- Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321)

- Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321)

  - This does not preclude SCell for which activation is completed

  - FFS: The case where SCell activation is completed

FFS: Application timing between NW triggering message and on demand SSB transmission

Agreement

Support on-demand SSB SCell operation triggered by gNB.

- FFS Details of associated signaling/indication/configuration provided to UE

Agreement

- For SSB burst(s) triggered by on-demand SSB SCell operation, study at least the following options.

  - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A.

  - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB

  - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B.

  - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted.

  - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B.

  - FFS: The combination of above options

  - FFS: How to define time instance A/B and the value of N per option

  - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement)

    - RAN1#116bis

Agreement

For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases:

- Scenario #2 and Case #1

- Scenario #2 and Case #2

- Scenario #2A and Case #1

- Scenario #2A and Case #2

- FFS: Scenario #3A and Case #1

- FFS: Scenario #3A and Case #2

- FFS: Scenario #3B and Case #1

- FFS: Scenario #3B and Case #2

- For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner.

  - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered.

- Notes:

  - Scenario #2A refers to

    - “When UE receives SCell activation command (e.g., as defined in TS 38.321)”

  - Scenario #3A refers to

    - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed”

  - Scenario #3B refers to

    - “When SCell activation is completed and SCell is activated” or

    - “After SCell activation is completed and SCell is activated”

  - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications.

  - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.

Agreement

- For a cell supporting on-demand SSB SCell operation,

  - Note: It is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not.

  - For on-demand SSB on the cell, downselect between the following alternatives

    - Alt-1: It is up to gNB implementation whether on-demand SSB is cell-defining SSB or not.

    - Alt-2: On-demand SSB is limited to non-cell-defining SSB.

      - FFS: Further limitations to on-demand SSB

Agreement

- For a cell supporting on-demand SSB SCell operation,

  - L1 and/or L3 measurement based on on-demand SSB is supported for the cell.

    - FFS further details on L1 and/or L3 measurement

Agreement

The following agreement from RAN1#116 is modified (in red)

- For SSB burst(s) triggeredindicated by on-demand SSB SCell operation, study at least the following options.

  - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A.

  - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB

  - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B.

  - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted.

  - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B.

  - FFS: The combination of above options

  - FFS: How to define time instance A/B and the value of N per option

  - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement)

Agreement

For a cell supporting on-demand SSB SCell operation, further study the following options.

- Option 1: Separate signaling between legacy/existing signaling (e.g., RRC, MAC CE) providing SCell activation/deactivation and signaling providing On-demand SSB transmission indication.

- Option 2: A single signaling in which both SCell activation/deactivation and On-demand SSB transmission indication are provided.

  - FFS: Details of the signaling

- Other options are not precluded.

- FFS: Details on On-demand SSB transmission indication

    - RAN1#117

Agreement

- For a cell supporting on-demand SSB SCell operation,

  - Support RRC based signaling to indicate on-demand SSB transmission on the cell.

  - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell.

  - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell.

    - This DCI signaling does not provide SCell activation/deactivation.

    - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc.

  - FFS: Scenarios where the above signalings are applicable

Agreement

- For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported.

  - Frequency of the on-demand SSB

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst

  - Periodicity of the on-demand SSB

  - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE

  - FFS: Whether the RRC is newly introduced or existing RRC is reused

Agreement

- At least support L1 measurement based on on-demand SSB

  - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported.

    - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB

Agreement

For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows.

- Alt 3-1: Time instance A is [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission

  - The SSB time domain positions of on-demand SSB burst are configured by gNB.

- FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components

- Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation

- FFS: Whether the value of T is predefined or indicated/configured by gNB

- FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB”

Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

Agreement

- For a cell supporting on-demand SSB SCell operation, at least the followings for on-demand SSB are known to UE.

  - Sub-carrier spacing of the on-demand SSB

  - Physical Cell ID of the on-demand SSB

  - Location of on-demand SSB burst

  - Downlink transmit power of on-demand SSB

  - FFS: Other parameters

  - FFS: Whether each of above parameters is configured/indicated explicitly or not

    - RAN1#118

Agreement

- Update the previous RAN1 agreement as follows.

  - At least support L1 measurement based on on-demand SSB

    - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported.

      - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB

      - The support of LTM is a separate discussion point

Agreement

For a cell supporting on-demand SSB SCell operation,

- Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration.

  - FFS: Whether to support RRC based signaling for other cases.

- Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A.

Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed.

Agreement

For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell.

- Periodicity of the on-demand SSB

- FFS: Any other relevant parameters

Agreement

For a cell supporting on-demand SSB SCell operation, at least the following is supported

- On-demand SSB on the cell is not located on synchronization raster.

- On-demand SSB on the cell is non-cell-defining SSB

FFS: Additional support of OD-SSB for CD-SSB located on sync-raster

Agreement

Support L3 measurement based on on-demand SSB

- Further work on L3 measurement is up to RAN2/RAN4

Agreement

LS to RAN2 for on-demand SSB SCell operation is agreed. Final LS in R1-2407438.

Agreement

The previous RAN1 agreement made in RAN1#117 is revised as follows.

- For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows.

  - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] of on-demand SSB burst [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is at least T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission

    - The SSB time domain positions of on-demand SSB burst are configured by gNB.

  - FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components

  - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation

  - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification.

    - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ+1

  - FFS: Whether the value of T is predefined or indicated/configured by gNB

  - (Working assumption) T=T_min

  - FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB”

- Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

Agreement

LS on timeline for On-demand SSB operation on SCell is agreed in R1-2407565.

    - RAN1#118bis

Agreement

- For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options.

  - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication

  - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication

  - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

  - Option 3: Configuration/indication of the duration of on-demand SSB transmission window

  - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell

  - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired

  - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed

  - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI

  - FFS: Each option is applicable to which Cases or Scenarios

  - FFS: Details related to each of the above options

Agreement

- For a cell supporting on-demand SSB SCell operation, support to provide at least the following parameters for on-demand SSB configuration by RRC at least for Case #1.

  - Sub-carrier spacing of the on-demand SSB

    - FFS if this can be absent

  - Physical Cell ID of the on-demand SSB

  - FFS: Time domain location of on-demand SSB burst such as SFN offset and half frame index

  - Downlink transmit power of on-demand SSB

  - FFS: The number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

  - FFS whether the above parameters are configured by reusing legacy RRC parameters or new RRC parameters

Agreement

- For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), consider only one or both of the following options for UE to perform L1 measurement based on on-demand SSB.

  - Option 1: A CSI report configuration is associated with both of on-demand SSB and always-on SSB

  - Option 2: A CSI report configuration is associated with one of always-on SSB and on-demand SSB

  - FFS: Whether OD-SSB and always on SSB have same beam or not

Conclusion

No consensus on the support of on-demand SSB SCell operation triggered by UE.

Agreement

The previous RAN1 agreement is partly confirmed and further revised as follows.

- For SSB burst(s) indicated by on-demand SSB SCell operation via a MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows.

  - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] ofwithin the first “possible” on-demand SSB burst which is at least T slots after the slot where UE receives a signalling from gNB to indicate on-demand SSB transmission

    - The SSB time domain positions of on-demand SSB burst are configured by gNB.

      - The location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB

  - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation

  - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification.

    - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ+1

  - (Working assumption) T=T_min

- Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

Agreement

For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), study at least the following Mux-Cases.

- Mux-Case #1: No time-domain overlap between always-on SSB and on-demand SSB

- Mux-Case #2: Always-on SSB and on-demand SSB overlap at least in time or frequency domain

    - RAN1#119

Agreement

Response to Q1 (What is the relation in terms of periodicity between always-on SSB and OD-SSB?) of Obj.1:

- The periodicity of on-demand SSB is one of 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms.

- The periodicity of on-demand SSB can be configured separately from the periodicity of always-on SSB.

- RAN1 is discussing what is the relation between periodicity of always-on SSB and periodicity of on-demand SSB and it has been identified that the main use case is that the periodicity of on-demand SSB is equal to or smaller than that of always-on SSB.

Further update to be made based on RAN1#119 progress.

Agreement

Response to Q3 (What is the relation in terms of frequency location between the always-on SSB and OD-SSB?) of Obj.1:

- The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. RAN1 is discussing the frequency location of OD-SSB for the case where always-on SSB is CD-SSB.

Agreement

Response to Q4 (What is the spatial relation between the always-on SSB and OD-SSB?) of Obj.1:

- SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters.

  - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same

- When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications

  - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same

- At least the case where SSB indices within on-demand SSB burst are identical to SSB indices within always-on SSB burst is supported. RAN1 is discussing whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst.

Agreement

- For a cell supporting on-demand SSB SCell operation, support to configure time domain location of on-demand SSB per on-demand SSB periodicity by RRC for both Case #1 and Case #2.

  - For Case #1 (i.e., No always-on SSB on the cell),

    - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index

      - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0

      - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0.

      - If half frame index parameter is NOT configured, UE assumes half frame index set to 0.

      - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced.

      - The value range of half frame index is 0 or 1.

  - For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), down-select one of the following alternatives.

    - Alt A: Same as for Case #1

    - Alt B: Based on a single parameter which is to indicate the time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset)

Agreement

- New periodicity value for on-demand SSB other than the legacy values (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms) is NOT introduced in Rel-19.

Agreement

Down-select at least one of the following alternatives.

- Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB.

- Alt 2: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB

- Alt 3: Do not support the case where always-on SSB is CD-SSB on a synchronization raster.

Down-select at least one of the following alternatives.

- Alt A: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB can be same or different from the frequency location of always-on SSB, subject to its configuration.

- Alt B: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB

- Alt C: Do not support the case where always-on SSB is CD-SSB and not on a synchronization raster.

Agreement

Response to Q2 (What is the relation in terms of time location between always-on SSB and OD-SSB?) of Obj.1:

- RAN1 understands the time location of OD-SSB in Q2 refers to the time location of possible OD-SSB burst

- RAN1 is still discussing the relation in terms of time location between always-on SSB and OD-SSB

Agreement

For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective.

- Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication

  - Deactivation by RRC is up to RAN2

  - FFS: Which scenario Option 1 is used

- Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

  - FFS: Whether Option 4, 4a is needed in addition to Option 2

  - FFS: Whether the value of N can be implicitly determined using a timer

    - RAN1#120

Agreement

Regarding the relation in terms of time location between the always-on SSB and on-demand SSB, at least for the case when the center frequency locations of always-on SSB and on-demand SSB are same, down-select one of the following.

- Alt Time-A: It is not allowed that time location of an SSB within on-demand SSB burst overlaps with time location of any SSB within always-on SSB burst.

  - NOTE: It is assumed that the periodicity of always-on SSB is equal to or larger than that of on-demand SSB.

- Alt Time-B: Time-domain locations of always-on SSB are a subset of time-domain locations of on-demand SSB.

  - NOTE: It is assumed that the periodicity of always-on SSB is equal to or larger than that of on-demand SSB.

- Alt Time-C: The specification has no restriction with regards to overlapping

Agreement

- For a cell supporting on-demand SSB SCell operation, to provide time domain location of on-demand SSB by RRC,

  - For Case #2, adopt Alt A in the previous RAN1 agreement, i.e.,

    - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index

      - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0

      - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0.

      - If half frame index parameter is NOT configured, UE assumes half frame index set to 0.

      - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced.

      - The value range of half frame index is 0 or 1.

    - FFS: Whether/how to indicate time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset)

      - By defining reference point as SFN where AO-SSB burst exists

Agreement

- For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameter (i.e., od-ssb-config) for on-demand SSB configuration, for both Case #1 and Case #2.

- For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameters to configure each of the followings, for both Case #1 and Case #2.

  - Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency)

    - FFS: Whether this parameter can be absent for Case #2

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst)

    - FFS: Whether this parameter can be absent for Case #2

  - Periodicity of the on-demand SSB (i.e., od-ssb-Periodicity)

  - Sub-carrier spacing of the on-demand SSB (i.e., od-ssbSubcarrierSpacing)

    - For Case #2, this parameter is absent and sub-carrier spagcing of on-demand SSB is the same as that of always-on SSB.

  - Physical Cell ID of the on-demand SSB (i.e., od-ssb-physCellId)

    - For Case #2, this parameter is absent and physical cell ID of on-demand SSB is the same as that of always-on SSB.

  - Time location of on-demand SSB burst (i.e., od-ssb-sfn-Offset and od-ssb-halfFrameIndex)

  - Downlink transmit power of on-demand SSB (i.e., od-ss-PBCH-BlockPower)

    - For Case #2, this parameter is absent and downlink transmit power of on-demand SSB is the same as that of always-on SSB.

  - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofTx)

  - FFS: Which of the above parameters are a list

Agreement

For RRC based OD-SSB indication, UE expects on-demand SSB is transmitted from the first on-demand SSB burst after receiving RRC carrying indication of OD-SSB transmission

Conclusion

The following combination of scenarios and cases for indicating OD-SSB are not supported in Rel-19

- Scenario #3A and Case #1

- Scenario #3A and Case #2

Above does not impact discussion on SSB periodicity adaptation in time domain

Agreement

Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB,

- Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB.

  - On-demand SSB is not on sync raster

  - AO-SSB and OD-SSB are located in the same BWP

  - FFS: Additional conditions

  - Subject to separate UE capability

  - Note: UE is not required to measure both AO-SSB and OD-SSB

Agreement

Regarding the relation in terms of time location between the always-on SSB and on-demand SSB,

- For the case when the center frequency locations of always-on SSB and on-demand SSB are same,

  - Alt Time-C: RAN1 specification has no restriction with regards to overlapping

  - From RAN1 perspective,

    - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification).

    - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported.

  - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2

  - At least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB

    - FFS: Whether half frame index is the same or different for AO-SSB and OD-SSB

- For the case when the center frequency locations of always-on SSB and on-demand SSB are different,

  - Alt Time-C: RAN1 specification has no restriction with regards to overlapping

  - UE assumes that frequency resources of always-on SSB are not overlapped with those of on-demand SSB in frequency domain.

  - AO-SSB and OD-SSB are located in the same BWP

  - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB

- NOTE: AO-SSB periodicity is not adapted

Send an LS to RAN4 to inform them of the above agreement. Final LS in R1-2501633.

[Active] Proposal #6-3a (SCS of T):

For the case where SCell with on demand SSB transmission and cell with signalling transmission have different numerology, when UE determines time instance A, the SCS to determine the value of T is down selected among the following options

- Option 1: the SCS of the active DL BWP where UE receives MAC CE for on-demand SSB transmission indication

- Option 2: the minimum of “the SCS of the active DL BWP where UE receives MAC CE for on-demand SSB transmission indication” and “the SCS of the active DL BWP where UE receives on-demand SSB”

- Option 3: the SCS of the active UL BWP where the UE transmits ACK corresponding to the MAC-CE for on-demand SSB transmission indication

    - RAN1#120bis

Agreement

For the case where SCell with on demand SSB transmission and cell with signalling transmission have different numerology, adopt Option 3 to determine SCS for Nslotsubframe,μ for determining T

- Option 3: the SCS of the active UL BWP where the UE transmits ACK corresponding to the MAC-CE for on-demand SSB transmission indication

Agreement

For the case when the center frequency locations of always-on SSB and on-demand SSB are different, at least the following is supported for QCL between AO-SSB and OD-SSB

- SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters.

- When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications

Agreement

For a cell supporting on-demand SSB SCell operation, for configuring the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofBurst),

- Alt 2: The value range of od-ssb-nrofBurst is {N2 integer values}

  - N2= [8]

  - If od-ssb-nrofBurst for an on-demand SSB is NOT configured, the on-demand SSB is deactivated via MAC CE.

  - If od-ssb-nrofBurst for an on-demand SSB is configured, the on-demand SSB is deactivated based on the configured value for od-ssb-nrofBurst [or via MAC CE].

Agreement

Upon the reception of MAC CE for deactivating on-demand SSB, UE expects that on-demand SSB is NOT transmitted from time instance B.

- Time instance B’ is T slots after the slot where UE receives a signalling from gNB to deactivate on-demand SSB. If time instance B’ falls within an on-demand SSB burst, time instance B is the ending of the slot containing the last actually transmitted SSB index within the on-demand SSB burst, otherwise, time instance B = time instance B’.

  - As agreed in previous RAN1 meeting, the SSB time domain positions of on-demand SSB burst are configured by gNB.

    - As agreed in previous RAN1 meeting, the location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB

  - (Working assumption) T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to deactivate on-demand SSB ending in slot n, and Nslotsubframe,μ is as defined in current specification.

  - (Working assumption) T=T_min

  - RAN4 to confirm the above two working assumptions

  - Nslotsubframe,μ is based on the SCS of the active UL BWP where the UE transmits ACK corresponding to the MAC-CE for on-demand SSB transmission indication

Agreement

For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s) (in addition to agreed ones), multiple candidate values can be configured (includes the case where no candidate values are configured) by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell.

- SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) for the following cases

  - The case where center frequency of AO-SSB and OD-SSB are different

  - Case 1

- Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb- nrofBurst)

FFS: Additional restrictions

Agreement

LS to RAN4 on time instance B for on-demand SSB SCell operation is agreed. Final LS in R1-2503108.

Agreement

For a cell supporting on-demand SSB SCell operation,

- Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency) can be absent for Case #2

  - If absent, the center frequency of on-demand SSB is the same as that of always-on SSB.

- od-ssb-PositionsInBurst, if provided, has the same bitmap structure as ssb-PositionsInBurst provided in ServingCellConfigCommon, that is od-ssb-PositionsInBurst is choice of 4-bit shortBitmap, 8-bit mediumBitmap, or 64-bit longBitmap.

- od-ssb-PositionsInBurst can be absent for Case #2

  - If absent, od-ssb-PositionsInBurst is the same as ssb-PositionsInBurst provided in ServingCellConfigCommon.

Agreement

For a cell supporting on-demand SSB SCell operation, the following combinations of scenarios and cases are supported for indicating OD-SSB using a MAC-CE.

- Scenario #3B and Case #1

  - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

- Scenario #3B and Case #2

  - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

Agreement

For a cell supporting on-demand SSB SCell operation, for Case #1 (i.e., No always-on SSB on the cell)

- UE does not expect the OD-SSB transmission indicated by RRC/MAC-CE to be deactivated while the SCell is activated.

Agreement

For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell),

- A CSI report configuration can be associated with

  - both of AO-SSB and OD-SSB

    - Which SSB to measure is up to UE implementation as long as RAN4 requirements are met

    - Applies at least for the case where AO-SSB and OD-SSB have the same center frequency and the same positions in the same SSB burst

    - FFS: It is up to UE to measure both SSBs or one of them if the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”.

  - OD-SSB only

    - FFS: Applies at least for the case where AO-SSB and OD-SSB have the different center frequencies

    - RAN1#121

Agreement

For a cell supporting on-demand SSB SCell operation,

- The following cases are NOT supported in Rel-19.

  - AO-SSB is CD-SSB and not on a synchronization raster.

  - OD-SSB is CD-SSB and on sync-raster.

  - When AO-SSB is not CD-SSB, the center frequency of AO-SSB is different from the center frequency of OD-SSB.

- When the center frequency locations of always-on SSB and on-demand SSB are different, OD-SSB is not CD-SSB.

Agreement

For a cell supporting on-demand SSB SCell operation, the following is supported for QCL between different OD-SSB configurations

- SS/PBCH blocks with the same SSB indexes according to OD-SSB configuration and indication are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters.

- When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index according to OD-SSB configuration and indication with the same QCL parameters according to existing specifications.

- FFS: Case where there is different SSB indexes before and after OD-SSB transmission adaptation

Conclusion

For a cell supporting on-demand SSB SCell operation,

- LTM based on OD-SSB is NOT supported in Rel-19.

- L1 measurement based on OD-SSB for a serving cell in Scenario #2 is NOT supported in Rel-19.

  - Note: As per previous RAN1 agreement, Scenario #2 is when SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321).

- L1 measurement based on OD-SSB for a non-serving/neighbor cell is NOT supported in Rel-19.

Agreement

For a cell supporting on-demand SSB SCell operation,

- If a PDCCH candidate and a currently being transmitted OD-SSB are overlapped in at least one RE, the UE is not required to monitor the PDCCH candidate.

Agreement

For a UE supporting on-demand SSB SCell operation,

- When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI, MCCH-RNTI, Multicast MCCH-RNTI or PDSCHs with SPS.

  - At least the UE shall assume that the PRBs containing currently being transmitted OD-SSB resources are not available for PDSCH in the OFDM symbols where OD-SSB is currently being transmitted on the cell.

Agreement

For a cell supporting on-demand SSB SCell operation and for Case #1 (i.e., No always-on SSB on the cell),

- For RO validation before SSB-to-RO mapping,

  - Alt 3: RO validation rule is determined based on all configured OD-SSB configuration(s) in RRC.

- For SSB-to-RO mapping,

  - SSB-to-RO mapping rule is determined based on all the SSB indices configured by all configured value(s) of od-ssb-PositionsInBurst in RRC.

Conclusion

For a cell supporting on-demand SSB SCell operation,

- Whether or not to assume quasi co-location between different SSB indices is up to UE implementation

Agreement

For a cell supporting on-demand SSB SCell operation,

- For OD-SSB, the SS/PBCH block in TS 38.213 Clause 11.1 can be replaced by the SS/PBCH block that is transmitted according to an on-demand SSB transmission operation in the cell.

Note: Whether/How to capture the above is up to the editor.

Agreement

For a cell supporting on-demand SSB SCell operation, it is supported to use OD-SSB as

- PL-RS,

- Reference signal for QCL/TCI configuration, or

- Candidate beam detection for beam failure recovery.

Note: Whether/How to capture the above is up to the editor.

Agreement

For a cell supporting on-demand SSB SCell operation, the following combinations are supported.

- For OD-SSB transmission activation (OD-Tact) and OD-SSB transmission adaptation (OD-TA),

  - Case A1: RRC-based OD-Tact without N (i.e., od-ssb-nrofBurst) configured + MAC CE-based OD-TA;

    - Subject to UE capability

  - Case B1: MAC CE-based OD-Tact without N configured + MAC CE-based OD-TA;

  - Case B2: MAC CE-based OD-Tact with N configured + MAC CE-based OD-TA.

- For OD-SSB transmission deactivation (OD-TD),

  - Case X1: RRC-based OD-Tact without N configured + MAC CE-based OD-TD;

    - Subject to UE capability

  - Case Y1: MAC CE-based OD-Tact or OD-TA without N configured + MAC CE-based OD-TD;

  - Case Y2: MAC CE-based OD-Tact or OD-TA with N configured + implicit OD-TD;

  - Case Y3: MAC CE-based OD-Tact or OD-TA with N configured + MAC CE-based OD-TD.

- Conclusion: There is no RAN1 consensus to support RRC activation of OD-SSB transmission configuring od-ssb-nrofBurst.

- Note: “Implicit OD-TD” above implies that the on-demand SSB is deactivated based on the value for od-ssb-nrofBurst according to NW indication.

Agreement

For a cell supporting on-demand SSB SCell operation, for configuring od-ssb-nrofBurst of which the value range is {N2 integer values},

- N2= 8

  - Note: This is updated from the previous RAN1 agreement.

- The following values for od-ssb-nrofBurst are taken as the starting point and to be confirmed in RAN1#122

  - For FR1, the value range of od-ssb-nrofBurst is {5, 10, 15, 20, 25, 30, 40, 50}.

  - For FR2, the value range of od-ssb-nrofBurst is {25, 30, 40, 50, 75, 100, 150, 200}.

Agreement

For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell),

- For RO validation before SSB-to-RO mapping,

  - Alt 3: RO validation rule is determined based on all configured AO-SSB and OD-SSB configuration(s) in RRC.

- For SSB-to-RO mapping,

  - SSB-to-RO mapping rule is determined based on all the SSB indices configured by all configured value(s) of od-ssb-PositionsInBurst and SSB indices configured for AO-SSB in RRC.

- NW to ensure that the SSB to RO mapping is the same between different UEs under the same serving cell.

Agreement

- For a cell supporting on-demand SSB SCell operation,

  - For Case #1 (i.e., No always-on SSB on the cell),

    - A CSI report configuration is associated with OD-SSB only,

      - UE reports SSBRI based on SSB-index corresponding to the currently being transmitted OD-SSB.

  - For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell),

    - For the case where AO-SSB and OD-SSB have the same center frequency,

      - A CSI report configuration is associated with both of AO-SSB and OD-SSB,

      - UE reports SSBRI based on SSB-index corresponding to AO-SSB and/or based on SSB-index corresponding to the currently being transmitted OD-SSB. Which SSB(s) is(are) used is up to UE implementation as long as RAN4 requirements are met.

    - For the case where AO-SSB and OD-SSB have the different center frequencies,

      - Option 2: A CSI report configuration is associated with both of AO-SSB and OD-SSB.

        - UE reports SSBRI based on SSB-index corresponding to AO-SSB and/or based on SSB-index corresponding to the currently being transmitted OD-SSB. Which SSB(s) is(are) used is up to UE implementation as long as RAN4 requirements are met.

  - SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of the associated csi-SSB-ResourceList in the corresponding CSI-SSB-ResourceSet.

- Note: The CSI report in the above agreement applies only to what is supported up to Rel-18.

Agreement

For a cell supporting on-demand SSB SCell operation and for L1 measurement based on on-demand SSB,

- For a CSI report configuration configured with reportConfigType set to periodic,

  - Do not support periodic CSI reporting based on OD-SSB, for Case #1 and Case #2

  - RAN2: On-demand SSB for SCell operation

    - RAN2#125bis

- No discussion in RAN2 this meeting

    - RAN2#126

- No discussion in RAN2 this meeting

    - RAN2#127

- Scenarios on OD-SSB:

  - RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

- OD-SSB transmission indication:

  - RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

  - New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

- Measurement on OD-SSB:

  - Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

  - For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

  - RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

R2-2407414: Proposal 3: RAN2 WG to discuss the issue of false measurement report triggering due to no SSB transmission when on-demand SSB is deactivated.

- RAN2 does not handle this issue in OD-SSB

  - RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

    - RAN2#127bis

- OD-SSB transmission indication:

  - No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

  - Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

  - NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

    - RAN2#128

- Not discussed

    - RAN2#129

- Agreements on OD-SSB

  - RAN2 leave it to RAN4 to conclude whether always-on SSB and/or OD-SSB are measured when both are transmitted in OD-SSB case 2.

    - RAN2#129bis

- Agreements on OD-SSB

  - For explicit activation/deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE).

  - A/D bit: “1” means activation, “0” means deactivation for explicit deactivation case.

  - For explicit activation/deactivation, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

  - OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB.

  - RAN2 aims to define single MAC CE format for both explicit and implicit OD-SSB deactivations. Details are FFS.

  - Working assumption: When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. FFS if we allow multiple OD-SSBs with the different frequencies for a given SCell.

    - RAN2#130

- Agreements on OD-SSB

  - (RRC-1) The configuration index is associated with a set of parameters, i.e., od-ssb-Periodicity, od-ssb-PositionsInBurst and od-ssb-nrofBurst.

  - The OD-SSB MAC CE is a variable size MAC CE. For each activated OD-SSB SCell, an additional field is added to indicate the applicable OD-SSB configuration index.

  - When UE receives MAC CE with Cj bit set to 0 for an SCell for which OD-SSB is currently activated:

    - OD-SSB is deactivated for the jth SCell, irrespective of whether od-ssb-nrofBurst is included or not included in configuration applied for the activated OD-SSB. Running timer/counter for implicit deactivation (if any) for the SCell is stopped.

  - For implicit deactivation of an OD-SSB transmission after the transmission of N occasions, reuse to SCell deactivation timer behaviour: UE (re)-applies the indicated N value in the MAC CE for each activated OD-SSB transmission upon reception of the OD-SSB Activation/Deactivation MAC CE.

  - In L3 measurement in OD-SSB case 1, if MAC-CE/RRC based activation / deactivation:

    - The UE starts L3 measurement towards the activated OD-SSB based on configured servingCellMO after reception of the activation

    - The UE stops L3 measurements after it determines the OD-SSB is deactivated implicitly or explicitly.

  - When the AO-SSB and OD-SSB have the same frequency, they share the same servingCellMO.

  - The UE applies the OD-SSB specific SMTC when the OD-SSB is activated and SCell is activated. Decision above does not impact RAN4 discussion whether both OD-SSB and AO-SSB can be measured.

  - Will send a LS to RAN4 to check if they have any concern.

  - RAN2 confirms the working consumption that when AO-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB, as agreement.

  - RAN2 understands multiple OD-SSBs with the different frequencies for a given SCell is not supported.

  - When OD-SSB is activated, UE uses servingCellMO-OD to measure serving cell; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell.

  - RAN2 will leave whether to introduce OD-SSB MAC CE triggered MR or not to RAN4.