3GPP TSG-RAN WG3 Meeting #129	R3-255582

Bengaluru, India, August 25th–29th, 2025

Agenda Item:	8.3

Source:	Ericsson, CATT

Title:	Support Aerial UE Flight Information Reporting to CN

Document for:	Discussion

# Introduction

At the last RAN3 meetings, we have worked on the incoming SA2 LSs (R3-250022 and R3-253020).

SA2 is specifying a new feature “Pre-flight Planning and In-flight Monitoring for UAVs” and SA2 agrees that the UAV altitude can be received from NG-RAN, assuming to reuse the existing feature like eventH1/H2 reporting together with additional trigger from the core network as specified in Rel-19. Time Stamp on when altitude was reported from the UE is also useful.

The only remaining issue is how to NG-RAN node reports the failure or when the UAV altitude reporting is stopped.

This meeting we should make a decision and complete the task.

# Discussion

In [1], two options are captured for failure handling:

- ”Aerial UE flight information reporting failed” indication in the Location Reporting Failure message. (FFS Option 1 for Failure Handling)

- Aerial UE flight information reporting failed” indication in the Location Report message and new cause value. (FFS Option 2 for Failure Handling)

The argument against Option 1 is that the Failure message is used for the failure when the request is initiated, and it cannot be used for failure when the reporting procedure has been ongoing, for example, an ongoing reporting is stopped by the reporter.

The argument against Option 2 is that there will be two changes to the procedure, and the cause value is not mandatory to be implemented. If for example, a NG-RAN node uses a general “Unspecified” CV, the CN could not tell if it is due to NG-RAN node do not support the UAV reporting, or other reasons.

Further, in the current system, it is already possible to use Location Reporting Failure procedure for the already initiated Location reporting, for example, during handover. Further, it is understood that we “borrow the Location Reporting procedure” for UAV reporting. Thus the argument against Option 1 does not hold.

Although both options would work, we consider Option 1 is clearer, has less impact and it is more robust. Option 1, using “Aerial UE flight information reporting failed” indication in the Location Reporting Failure message to indicate to CN is our preference.

Proposal 1: It is proposed that RAN3 discuss and conclude on the UAV Reporting failure handling at this meeting. Our preference is Option 1, using “Aerial UE flight information reporting failed” indication in the Location Reporting Failure message.

Proposal 2: It is proposed RAN3 to agree CR R3-255520.

# Proposals

Proposal 1: It is proposed that RAN3 discuss and conclude on the UAV Reporting failure handling at this meeting. Our preference is Option 1, using “Aerial UE flight information reporting failed” indication in the Location Reporting Failure message.

Proposal 2: It is proposed RAN3 to agree CR R3-255520.

# References

R3-255135, Support Aerial UE Flight Information Reporting to CN, Ericsson, CMCC, ZTE, CATT, Nokia, Huawei

R3-255520, Support Aerial UE Flight Information Failure Reporting in FAILURE INDICATION message, Ericsson, CMCC, CATT