3GPP TSG-RAN WG3 Meeting #129	R3-255584

Bengaluru, India, 25 – 29 August, 2025

Agenda item:	8.1

Source:	Nokia

Title:	Discussion on MBS Communication Service Type for QMC

Document for:	Decision

# 1	Introduction

We here provide discussion on LS received from SA4 in R3-255028 [1].

# 2	Discussion

In their LS, SA4 informs RAN3 about their agreements relative to the attribute @communicationServiceType which is part of the application layer measurement configuration defined in TS 26.247. This configuration is transparently forwarded by the gNB to the UE. SA4 explains that the @communicationServiceType is a superset of @mbsCommunicationServiceType. @communicationServiceType indicates a list of communication service type(s) for which the collection and reporting of QoE metrics is requested, and shall contain one or more of the following values:

- The value unicast refers to the common unicast communication type carrying media streaming services.

- The value mbsMulticast refers to the MBS Multicast communication service  per clause 21.1 of TS 38.300.

- The value mbsBroadcast refers to the MBS Broadcast communication service per clause 21.1 of TS 38.300.

- The value all refers to all communication service types.

As per Rel-18 RAN specification, a given QMC configuration may apply for one of the following communication service types:

- unicast

- broadcast

- multicast

The current specification therefore doesn’t support any combination of communication service types, and SA4 specification and RAN3 specification are per today not aligned (Rel-18). However, NGAP and XnAP signalling could as such be enhanced to support combinations of service types, either as correction in Rel-18 or in later releases.

On the other side, the solution retained in Rel-18 is based on particular handling of QMC sessions configured for MBS broadcast service. The configuration for such sessions is transferred to the UE and stored in the UE in idle mode and retrieved by the network when the UE reconnects. When the gNB receives such configuration from the UE, it will as per current specification assume that the corresponding QMC session applies to MBS broadcast service only. One option here could be to update TS 38.300 to specify that QMC configurations uploaded from the UE apply to e.g. all communication services (including unicast), but support of a configurable list of communication services seems to require additional information retrieved by the gNB from the UE. Such additional information could be, e.g. a list of the communication service types that are configured for the QMC session.

How to support the configuration mechanism described by SA4 would therefore in our view need further analysis in RAN2.

Proposal 1: Send a reply LS to RAN2, SA4 cc SA5 describing solution where the gNB receives additional configuration information from the UE so that QMC sessions applicable to any combination of communication service types can be supported.

Proposal 2: In the LS, ask feedback from RAN2 whether such solution is feasible, including in which 3GPP release.

# 3	Conclusion

We have made the following proposals:

Proposal 1: Send a reply LS to RAN2, SA4 cc SA5 describing solution where the gNB receives additional configuration information from the UE so that QMC sessions applicable to any combination of communication service types can be supported.

Proposal 2: In the LS, ask feedback from RAN2 whether such solution is feasible, including in which 3GPP release.

# References

[1]		R3-255028 – S4-251096, LS to RAN3 on MBS Communication Service Type, SA4