3GPP TSG-RAN WG3 Meeting #129	R3-255592

# Bangaluru, India, 25 – 29 August 2025

Agenda Item:	10.2

Source:	Ericsson, Huawei, Jio Platforms

Title:	Signaling of MRO for LTM between target DU and source DU

Document for:	Discussion and Decision

# 1	Introduction

During previous RAN3 meetings, 3 MRO for LTM use-cases were agreed:

- Beam Failure Recovery (BFR) after successful LTM Cell Switch to wrong beam (case 1)

- RACH- less LTM Cell Switch failure due to wrong beam (case 2)

- Cell Switch failure due to outdated TA (case 3)

A common stage-3 solution was designed and agreed, re-using the ACCESS AND MOBILITY INDICATION and ACCESS and DU-CU MOBILITY INDICATION F1AP messages.

However, some questions and FFSs were remaining. This discussion paper presents solutions to answer these questions. It is also proposed to simplify the structure that was agreed during RAN3#129 as well as some corrections.

# 2	Discussion

During RAN3#129, the following stage-3 structure was agreed to support the 3 agreed MRO for LTM use-cases:

#### 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| […] |  |  |  |  |  |  |
| Beam Failure Recovery Information | O |  |  |  | YES | ignore |
| >Recovery TCI State | M |  | OCTET STRING | Includes the TCI-StateId IE used at Beam Failure Recovery, as defined in TS 38.331 [8]. | - |  |
| >gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | - |  |
| LTM Cell Switch Failure due to Wrong Beam Information | O |  |  |  | YES | ignore |
| >Target Beam Information | M |  | FFS | FFS if it corresponds to the target beam index or TCI state ID. | - |  |
| >gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | - |  |
| TA Information (name is FFS) | O |  |  |  | YES | ignore |
| >TA Value | M |  | INTEGER (0..4095) | Indicates the TA value used at successful Random Access, as defined in TS 38.213 [31]. | - |  |
| >gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | - |  |

Editor’s note: FFS if a common structure is needed for UE F1AP IDs.

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports. Value is 64. |

Next change

#### 9.2.10.2	DU-CU ACCESS AND MOBILITY INDICATION

This message is sent by the gNB-DU to provide access and mobility information to the gNB-CU.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| […] |  |  |  |  |  |  |
| Beam Failure Recovery Information | O |  |  |  | YES | ignore |
| >Recovery TCI State | M |  | OCTET STRING | Includes the TCI-StateId IE used at Beam Failure Recovery, as defined in TS 38.331 [8]. | - |  |
| >gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | - |  |
| LTM Cell Switch Failure due to Wrong Beam Information | O |  |  |  | YES | ignore |
| >Target Beam Information | M |  | FFS | FFS if it corresponds to the target beam index or TCI state ID | - |  |
| >gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | - |  |
| TA Information (name is FFS) | O |  |  |  | YES | ignore |
| >TA Value | M |  | INTEGER (0..4095) | Indicates the TA value used at successful Random Access, as defined in TS 38.213 [31]. | - |  |
| >gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | - |  |

Editor’s note: FFS if a common structure is needed for UE F1AP IDs.

| Range bound | Explanation |
| --- | --- |
| maxnoofLBTFailureInformation | Maximum no. of UEs for which LBT Failure Information is provided, the maximum value is 64. |

In the following sections, a new structure is proposed to deal with the Editor’s notes. Some further enhancements regarding the beam index information are also discussed, as well as some corrections and stage-2 proposals.

## 2.1. F1AP tabular corrections and enhancements

In both F1AP messages used for the the MRO for LTM signalling, the UE F1AP IDs are common for all 3 use-cases. Therefore, it would simplify the structure of the messages if a common optional IE is introduced, instead of one IE per use-case. It would also allow the target DU to signal multiple failure/near-failure at the same time.

Proposal 1: Agree on a common optional IE for cases 1, 2 and 3.

Another aspect to consider is the type of information used to identify the beam (e.g. reconnect or re-established or recovery beam) after successful BFR or LTM Cell Switch failure. From a target DU point-of-view, retrieving the SSB index is easier than retrieving the full configuration (i.e. TCI state) related to it. Also, with the SSB index, the source DU can retrieve the TCI state if it needs more information to perform optimization. Therefore, it is proposed to signal the SSB Index of the reconnect or re-established or recovery beam after successful BFR or after LTM Cell Switch Failure due to Wrong Beam.

Proposal 2: Signal the SSB Index of the recovery beam after successful BFR and change semantics description accordingly.

Proposal 3: Signal the SSB Index of the reconnect or re-established or recovery beam after LTM Cell Switch Failure due to Wrong Beam and change semantics description accordingly.

Another FFS captured during RAN3#129 is related to the name of the IE used for case 3 (Cell Switch failure due to outdated TA). The goal of this IE is to convey the TA value that was used at RRC re-establishment (RACH-based) after an LTM Cell Switch failure in the same beam. In that case it is proposed to use the name “TA Value at re-establishment” and to clarify the use-case in the semantics description.

Proposal 4: Change the name of the IE for case 3 into “TA Value at re-establishment”, clarify the semantics description and remove FFS.

It is therefore proposed to use the following structure for the DU-CU ACCESS AND MOBILITY INDICATION message (from target DU to CU):

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| […] |  |  |  |  |  |  |
| MRO for LTM Information | O |  |  |  | YES | ignore |
| >gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | - |  |
| >BFR SSB Index | O |  | INTEGER (0..63) | SSB Index of the recovery beam used at successful Beam Failure Recovery. | - |  |
| >Target SSB Index after Cell Switch Failure | O |  | INTEGER (0..63) | SSB Index of the re-established or recovery beam after LTM Cell Switch Failure due to Wrong Beam. | - |  |
| >TA Value at re-establishment | O |  | INTEGER (0..4095) | Indicates the TA value, as defined in TS 38.213 [31], used at successful Random Access after a Cell Switch failure in same beam. | - |  |

Correspondingly, it is also proposed to use the following structure for the ACCESS AND MOBILITY INDICATION message (from CU to source DU):

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| […] |  |  |  |  |  |  |
| MRO for LTM Information | O |  |  |  | YES | ignore |
| >gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | - |  |
| >BFR SSB Index | O |  | INTEGER (0..63) | SSB Index of the recovery beam used at successful Beam Failure Recovery. | - |  |
| >Target SSB Index after Cell Switch Failure | O |  | INTEGER (0..63) | SSB Index of the re-established or recovery beam after LTM Cell Switch Failure due to Wrong Beam. | - |  |
| >TA Value at re-establishment | O |  | INTEGER (0..4095) | Indicates the TA value, as defined in TS 38.213 [31], used at successful Random Access after a Cell Switch failure in same beam. | - |  |

Proposal 5: Capture the new tabular structure into TS 38.473.

## 2.2 Procedural text corrections and stage-2 description

### 2.2.1 CU to source DU – ACCESS AND MOBILITY INDICATION message

The following procedural text corresponding to the previous tabular structure was captured in the F1AP BL CR:

If the ACCESS AND MOBILITY INDICATION message contains the Beam Failure Recovery Information IE, the gNB-DU shall, if supported, consider that a Beam Failure Recovery after successful LTM Cell Switch has occurred in the target DU for the UE identified by the gNB-DU UE F1AP ID IE, and may take it into account for optimisation of target beam selection.

If the ACCESS AND MOBILITY INDICATION message contains the LTM Cell Switch Failure due to Wrong Beam Information IE, the gNB-DU shall, if supported, consider that an LTM Cell Switch Failure due to Wrong Beam has occurred in the target DU for the UE identified by the gNB-DU UE F1AP ID IE, and may take it into account for optimisation of target beam selection.

If the ACCESS AND MOBILITY INDICATION message contains the TA Information IE (name is FFS), the gNB-DU may take it into account for optimisation of Timing Advance related parameters for the UE identified by the gNB-DU UE F1AP ID IE.

This text needs to be updated to follow the new structure proposed in the previous section. Without changing the meaning of the information signalled between CU and source DU, this could lead to the following proposal:

If the ACCESS AND MOBILITY INDICATION message contains the BFR SSB Index IE within the MRO for LTM Information IE, the gNB-DU shall, if supported, consider that a Beam Failure Recovery after successful LTM Cell Switch has occurred in the target DU for the UE identified by the gNB-DU UE F1AP ID IE, and may take it into account for optimisation of target beam selection.

If the ACCESS AND MOBILITY INDICATION message contains the Target SSB Index after Cell Switch Failure IE within the MRO for LTM Information IE, the gNB-DU shall, if supported, consider that an LTM Cell Switch Failure due to Wrong Beam has occurred in the target DU for the UE identified by the gNB-DU UE F1AP ID IE, and may take it into account for optimisation of target beam selection.

If the ACCESS AND MOBILITY INDICATION message contains the TA Value at re-establishment IE within the MRO for LTM Information IE, the gNB-DU may take it into account for optimisation of Timing Advance related parameters for the UE identified by the gNB-DU UE F1AP ID IE.

Proposal 6: Capture procedural text changes into TS 38.473.

### 2.2.2 Target DU to CU – DU-CU ACCESS AND MOBILITY INDICATION message and stage-2

The following procedural text was captured in the F1AP BL CR:

Interaction with the Access and Mobility Indication procedure:

If the Beam Failure Recovery Information IE or the LTM Cell Switch Failure due to Wrong Beam Information IE or the TA Information IE (name is FFS) is included in the DU-CU ACCESS AND MOBILITY INDICATION message, the gNB-CU shall, if supported, forward this information to the source gNB-DU via the ACCESS AND MOBILITY INDICATION message including the Beam Failure Recovery Information IE, the LTM Cell Switch Failure due to Wrong Beam Information IE and the TA Information IE (name is FFS) respectively.

Editor’s note: FFS how to capture the translation of the UE F1AP IDs.

The interaction with other procedures’ section can only be used for the same interface instance. This is not the case here. Therefore, this description shall be moved to stage-2 (i.e. TS 38.401).

Proposal 7: Capture interaction between target DU, CU and source DU into stage-2 (i.e TS 38.401).

For cases 2 and 3, the stage-2 text includes the case where the UE reconnect to the target cell after being in IDLE. However, in that case, the target DU is not able to identify if the UE that is reconnecting is the same as the UE which failed. Therefore, cases 2 and 3 cannot apply to reconnection after IDLE, and it is proposed to remove it from the stage-2 description.

Proposal 8: Remove the reconnection cases from stage-2.

As a result, the following stage-2 description is proposed:

## 7.X	MRO support for LTM

The gNB-CU receives a RLF report associated to an LTM mobility event from the UE or via the Failure indication message over Xn. The gNB-CU performs initial analysis and in case of failure due to inappropriate cell switch triggering may forward the RLF report to the last serving gNB-DU in case of too late LTM, or to the source gNB-DU in case of too early LTM or LTM to wrong cell.

The target gNB-DU identifies that a Beam Failure Recovery (BFR) has happened in the UE shortly after a successful LTM cell switch by detecting a time gap between the successful LTM cell switch and the BFR in the same cell is smaller than the configured threshold (e.g. Tstore_UE_cntxt).The target gNB-DU performs initial analysis and may send the recovery beam information to the source gNB-DU via the gNB-CU using MRO for LTM information contains in DU-CU ACCESS AND MOBILITY INDICATION and ACCESS AND MOBILITY INDICATION messages.

The target gNB-DU identifies that the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell but to a beam different from the beam used during the failed LTM cell switch execution. The target gNB-DU may send the re-established or recovery beam information to the source gNB-DU via the gNB-CU using MRO for LTM information contains in DU-CU ACCESS AND MOBILITY INDICATION and ACCESS AND MOBILITY INDICATION messages. The source gNB-DU performs root cause analysis to detect RACH-less LTM failure caused by wrong beam selection.

The target gNB-DU identifies that the UE successfully performed a RACH-based access while re-establishing or recovering to the same target cell and same target beam but with TA value different from the one which was included in CU-DU CELL SWITCH NOTIFICATION message received from gNB-CU at LTM Cell Switch execution. The target gNB-DU sends the TA value used at successful RACH-based access to the source gNB-DU via the gNB-CU using MRO for LTM information contains in DU-CU ACCESS AND MOBILITY INDICATION and ACCESS AND MOBILITY INDICATION messages.

# 3	Conclusion

Stage-2 and stage-3 implementation of MRO for LTM use-cases have been discussed and the following proposals were made:

Proposal 1: Agree on a common optional IE for cases 1, 2 and 3.

Proposal 2: Signal the SSB Index of the recovery beam after successful BFR and change semantics description accordingly.

Proposal 3: Signal the SSB Index of the reconnect or re-established or recovery beam after LTM Cell Switch Failure due to Wrong Beam and change semantics description accordingly.

Proposal 4: Change the name of the IE for case 3 into “TA Value at re-establishment”, clarify the semantics description and remove FFS.

Proposal 5: Capture the new tabular structure into TS 38.473.

Proposal 6: Capture procedural text changes into TS 38.473.

Proposal 7: Capture interaction between target DU, CU and source DU into stage-2 (i.e TS 38.401).

Proposal 8: Remove the reconnection cases from stage-2.