3GPP TSG-RAN WG3 #129	R3-255596

Bengaluru, India, August 25-29, 2025

Agenda Item:	10.2

Source:	Ericsson

Title:	(TP for BL CR to 38.473 for SON) Ping-pong avoidance for LTM

Document for:	Agreement

# Annex:	 TP for BL CR to 38.473 for SON

First change

### 8.3.9	DU-CU Cell Switch Notification

#### 8.3.9.1	General

The purpose of the DU-CU Cell Switch Notification procedure is to enable the gNB-DU to inform the gNB-CU about the initiation of the cell switch command to the UE. This procedure is also used to transfer the selected TCI state from the gNB-DU to the gNB-CU. The procedure uses UE-associated signalling.

#### 8.3.9.2	Successful Operation

Figure 8.3.9.2-1: DU-CU Cell Switch Notification procedure. Successful operation.

The gNB-DU initiates the procedure by sending a DU-CU CELL SWITCH NOTIFICATION message.

Upon reception of the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall, if supported, consider that a cell switch command was sent to the UE where the target cell is indicated by the included Cell ID IE.

Interaction with CU-DU Cell Switch Notification procedure:

If the LTM Cell Switch Information IE is included in the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall, if supported, forward it to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message.

If the TA Information List IE is included in the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall, if supported, forward it to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message to transfer the TA value(s).

If the LTM ping-pong detection IE is included in the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall, if supported, consider that a ping-pong has occurred between the cells contained in the L1 visited cell list IE, and may take that into account for mitigating the ping-pong.

Next change

#### 9.2.2.16	CU-DU CELL SWITCH NOTIFICATION

This message is sent by the gNB-CU to inform the gNB-DU about the initiation of the cell switch command to the UE.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |
| LTM ping-pong detection |  | 0..1 |  |  | YES | ignore |
| >L1 visited cell list |  | 0.. <maxnoofL1VisitedCells> |  |  | EACH | ignore |
| >>Previous Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>Time UE Stayed in Previous Cell | M |  | INTEGER (0..4095) | The duration of time the UE stayed in the previous cell, or set of previous NR cells with the same NR ARFCN for reference point A, in seconds. If the duration is more than 4095s, this IE is set to 4095. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |
| maxnoofL1VisitedCells | Maximum number of L1 visited cells to be sent, the maximum value is 8. |

Next change

### 9.4.4	PDU Definitions

<SNIP>

F1U-PathFailure,

LTMResetInformation,

LTMPingPongDetectionList

<SNIP>

id-LTMResetInformation,

id-PreconfiguredSRSInformation,

id-LTMPingPongDetectionList

<SNIP>

-- **************************************************************

--

-- CU-DU CELL SWITCH NOTIFICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- CU-DU Cell Switch Notification

--

-- **************************************************************

CUDUCellSwitchNotification ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ CUDUCellSwitchNotificationIEs}},

...

}

CUDUCellSwitchNotificationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-NRCGI					CRITICALITY reject	TYPE NRCGI							PRESENCE mandatory	}|

{ ID id-LTMCellSwitchInformation	 CRITICALITY ignore	TYPE LTMCellSwitchInformation	PRESENCE optional	}|

{ ID id-TAInformation-List			CRITICALITY ignore	TYPE TAInformation-List			PRESENCE optional		}|

{ ID id-LTMPingPongDetectionList	CRITICALITY ignore	TYPE LTMPingPongDetectionList	PRESENCE optional	},

...

}

Next change

### 9.4.5	Information Element Definitions

<SNIP>

LTMPingPongDetectionList ::= SEQUENCE (SIZE(1..maxnoofL1VisitedCells)) OF LTMPingPongDetection-Item

LTMPingPongDetection-Item		::= SEQUENCE {

previousCellId										NRCGI,

timeUEStayedInPreviousCell						INTEGER (0..4095),

iE-Extensions		ProtocolExtensionContainer { { LTMPingPongDetection-Item-ExtIEs} } OPTIONAL,

...

}

LTMPingPongDetection-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

<SNIP>

maxnoofSSBIndices						INTEGER ::= 64

maxnoofL1VisitedCells							INTEGER ::= 8

<SNIP>

id-SRSPosPeriodicConfigHyperSFNIndex				ProtocolIE-ID ::= 856

id-PreconfiguredSRSInformation						ProtocolIE-ID ::= 857

id-LTMPingPongDetectionList							ProtocolIE-ID ::= xxx

End of changes