3GPP TSG-RAN WG3 Meeting #129	R3-255600

# Bengaluru, India, 25th – 29th Aug, 2025

Title: 	Enhanced Rejected Slice Reporting for Dynamic Slice Optimization

Source: 	Jio Platforms

Agenda item:	10.3.2

Document Type:	Discussion and decision

# 1	Introduction

During RAN3#128, discussions highlighted the need for enhancements in reporting NAS-level rejected slice requests to NG-RAN nodes:

Whether to do enhancement on reporting rejected slice from CN to NG-RAN based on the request from NG-RAN?

This contribution proposes a mechanisms for gNB to request rejected slice information from AMF along with a dynamic slice adaptation and proactive OAM adjustments.

Discussion key focus is on Enhanced reporting of rejected slices to enable real-time slice optimization and introduction of UE-assisted feedback for slice demand prediction.

# 2	Slice Support Overview

##### 2.1 Observations on UE Slice Request Behaviour

Following 3GPP TS 38.413 §9.2.6.1 NG SETUP REQUEST and §9.2.6.4 RAN CONFIGURATION UPDATE messages sent from gNB to AMF, they contain the list of slices supported per Tracking Area, as initially configured in OAM for the gNB.

During RAN3#128, discussions revealed ambiguity regarding whether a UE can request services over a network slice that was not included in its initial registration. Specifically, it was unclear if a UE could later request slices outside its current Allowed, Rejected, or Partially Allowed NSSAI lists.

To address this, we examine the UE’s process for requesting slice-based services. Each UE is provisioned with a Configured NSSAI per PLMN. To access a slice service, the UE must first register to that slice, as specified in TS 23.501 §5.15.5.2.1. During registration, the UE provides a Requested NSSAI containing the S-NSSAIs it wishes to use, unless they are in the Pending NSSAI.

Observation 1: The UE is not required to register on all configured slices upfront. It may request services from a subset and later register for additional slices as needed.

Following TS 23.501 §5.15.4.1.1, the UE includes only PLMN-applicable S-NSSAIs in the Requested NSSAI.

If an S-NSSAI is not in the Allowed, Partially Allowed, or Rejected NSSAI lists, the UE may attempt registration again in subsequent requests. Moreover TS 23.501 §5.15.5.2.2 and TS 23.502 §4.2.2.2.2 clarify that a UE can initiate a Mobility Registration Update to modify its permitted slices while in RM-REGISTERED state.

Observation 2: The UE can dynamically trigger a registration procedure to request access to new slices, even if they were previously unrequested or outside its current NSSAI lists.

##### 2.2 Current Limitations in Slice Rejection Reporting

A critical gap exists in current NG-RAN operation: while AMF communicates Allowed, Partially Allowed, and Target NSSAIs to gNBs through INITIAL CONTEXT SETUP REQUEST and DOWNLINK NAS TRANSPORT messages, the Rejected NSSAI list is notably absent from these communications. This creates an information asymmetry where the RAN node remains unaware of which specific slices were requested but denied by the AMF, particularly when rejections occur due to tracking area unsupport or policy restrictions. Furthermore the gNB possesses cell-level location knowledge but lacks slice rejection context, while the AMF has slice decision-making data but lacks precise cell-level information. This disconnect prevents optimal slice management.

Based on previous analysis there are some key Consequences. Without rejection awareness, gNBs may continue resource preparation for slices that will never be authorized. Moreover the RAN OAM systems cannot correlate rejection patterns with specific cells neither identify underserved areas requiring slice deployment adjustments to respond dynamically to emerging slice demand patterns

Observation 3: The absence of rejected slice information at the gNB level breaks what should be a closed-loop optimization process between core network decisions and RAN configuration.

Current implementations force OAM systems to operate without visibility into actual slice demand patterns, particularly for new service areas (e.g., enterprise deployments), mobile scenarios (e.g., vehicular slice handovers) and temporal demand variations (e.g., event-driven slice needs)

Observation 4: When a UE's slice request is rejected due to TA limitations (a) The AMF makes this determination based on configured policies; (b) The gNB receives no notification of this rejection; (c) Subsequent UE attempts trigger repeated signaling; (d) OAM systems remain unaware of unmet demand

This missing feedback loop represents a significant barrier to truly dynamic network slicing implementation, where real-time demand should inform real-time resource allocation.

##### 2.3 Conclusions on Enhancing UE Slice Request Behaviour

The current 5G network slicing architecture operates under the assumption that slice requests are relatively static after initial registration. However, Observation 1 reveals a critical gap: UEs are not required to register for all configured slices upfront. Instead, they may activate slices dynamically based on real-time service demands. This behavior has profound implications for slice management and optimization.

The Configured NSSAI allows UEs to register for slices on an as-needed basis. As an example use case, a smartphone might initially request only an eMBB slice for general internet access but later require a URLLC slice for a latency-sensitive application like cloud gaming or industrial control. Traditional static slice provisioning may lead to underutilization or unexpected rejections when UEs attempt to access previously unused slices

Observation 5: Since UEs can request new slices at any time, the network must be able to anticipate and adapt to these dynamic requests.

Each rejected slice request carries valuable information about unmet demand. By analyzing these rejections, operators can identify patterns—such as frequent requests for a specific slice in a particular area—and adjust slice deployment accordingly.

Proposal 1: Use Rejection Data as a Demand Signal to identify patterns for slice adjustments.

As a second example use case, if multiple UEs in a factory environment are rejected when requesting an industrial IoT slice, the OAM system can prioritize deploying that slice in the affected cells. This approach transforms rejection reporting from a reactive mechanism into a proactive optimization tool.

Proposal 2: Use rejection reporting for Proactive Optimization.

Proposal 3: allow for dynamic rejection tracking, by enhancing NGAP signaling to include timestamps and UE context (e.g., service type) with rejection reports, enabling more granular demand analysis.

Following the previous analysis, Observation 2 underscores the UE's ability to dynamically trigger registration for new slices, even those outside its current NSSAI lists. This capability renders the gNB's traditional "blind spot" to rejected slices a significant limitation in slice management. The gNB, as the entity managing radio resources, needs visibility into slice rejections to make informed decisions. For example, if a UE's request for a high-priority slice (e.g., emergency services) is rejected due to coverage gaps, the gNB could prioritize resource allocation or trigger handovers to cells where the slice is available.

Proposal 4: Integrate rejection data with machine learning models to forecast slice demand and pre-emptively allocate resources.

Rejections may occur for various reasons, such as temporary congestion or permanent lack of support. Without this context, the RAN cannot take corrective actions, leading to suboptimal user experiences. For example, repeated rejections of a URLLC slice in a specific area might indicate the need for additional infrastructure or slice reconfiguration.

Observation 6: By correlating rejected slices with cell-level data, the gNB can identify coverage holes or capacity shortages.

Current 5G networks lack mechanisms for NG-RAN (gNB) to learn about slices rejected by the core network (AMF). This blind spot forces gNBs to wastefully reserve resources and trigger signaling for unsupported slices, degrading network efficiency.

Observation 7: Awareness of rejected slices allows the gNB to avoid unnecessary signaling or resource reservations for slices that are unlikely to be supported, improving overall network efficiency.

Proposal 5: Introduce new IEs in the INITIAL CONTEXT SETUP REQUEST and DOWNLINK NAS TRANSPORT messages to convey rejection reasons (e.g., "slice not supported," "temporary congestion") from the AMF to the gNB.

# 3	Conclusion

Follows the summary of Observations and Proposals:

Observation 1: The UE is not required to register on all configured slices upfront. It may request services from a subset and later register for additional slices as needed.

Observation 2: The UE can dynamically trigger a registration procedure to request access to new slices, even if they were previously unrequested or outside its current NSSAI lists.

Observation 3: The absence of rejected slice information at the gNB level breaks what should be a closed-loop optimization process between core network decisions and RAN configuration.

Observation 4: When a UE's slice request is rejected due to TA limitations (a) The AMF makes this determination based on configured policies; (b) The gNB receives no notification of this rejection; (c) Subsequent UE attempts trigger repeated signaling; (d) OAM systems remain unaware of unmet demand

Observation 5: Since UEs can request new slices at any time, the network must be able to anticipate and adapt to these dynamic requests.

Proposal 1: Use Rejection Data as a Demand Signal to identify patterns for slice adjustments.

Proposal 2: Use rejection reporting for Proactive Optimization.

Proposal 3: allow for dynamic rejection tracking, by enhancing NGAP signaling to include timestamps and UE context (e.g., service type) with rejection reports, enabling more granular demand analysis.

Proposal 4: Integrate rejection data with machine learning models to forecast slice demand and pre-emptively allocate resources.

Observation 6: By correlating rejected slices with cell-level data, the gNB can identify coverage holes or capacity shortages.

Observation 7: Awareness of rejected slices allows the gNB to avoid unnecessary signaling or resource reservations for slices that are unlikely to be supported, improving overall network efficiency.

Proposal 5: Introduce new IEs in the INITIAL CONTEXT SETUP REQUEST and DOWNLINK NAS TRANSPORT messages to convey rejection reasons (e.g., "slice not supported," "temporary congestion") from the AMF to the gNB.