3GPP TSG-RAN WG3 Meeting #129	R3-255601

# Bengaluru, India, 25th – 29th Aug, 2025

Title: 	Inter-CU LTM Robustness Enhancements

Source: 	Jio Platforms

Agenda item:	13.2

Document Type:	Discussion and decision

# 1	Introduction

During the last RAN3#128 meeting, the following meeting minutes were captured:

From RAN3#128:

Inter-CU LTM:

WA: RAN3 agrees that, for both F1AP and XnAP, the activation and deactivation of CSI-RS transmission in LTM candidate cells are performed at the level of individual CSI-RS Resource IDs.

Add description in Stage 2 TS 38.401 for describing that CU can request Candidate DU to provide CSI-RS configuration in UE CONTEXT SETUP REQUEST message, and Candidate DU signals the CSI-RS configuration in UE CONTEXT SETUP RESPONSE message.

For Inter-CU LTM, LTM CONFIGURATION UPDATE procedure is per node level basis with a list of cells, and security key is per cell..”

As part of ongoing refinements to the Low-Latency Target Cell Mobility (LTM) framework in multi-node architectures, it has become evident that both CSI-RS resource coordination and PRACH resource lifecycle management are critical enablers of reliable RACH-less LTM. While prior contributions focused on the preparation and execution phases of LTM, additional standardization is needed to ensure consistency with recent agreements from RAN#129, particularly regarding per-CSI-RS resource activation, node-level configuration updates, and per-cell security key management.

This document extends the previous discussion from RAN3#128 by proposing concrete enhancements to both XnAP and F1AP procedures to maintain alignment with the updated working assumptions, while preserving robustness of the LTM mechanism..

# 2	Discussion on Inter-CU Robustness Enhancements.

In Inter-CU LTM preparation, the source gNB initiates a request for PRACH and CSI-RS resources from one or more candidate gNBs to enable future execution of RACH-less mobility. Candidate gNBs provision PRACH pools and CSI-RS resource IDs, which the source gNB can reference in subsequent PDCCH orders or configuration messages.

However, two key reliability risks remain:

- CSI-RS and PRACH resource deallocation by the candidate node may occur without timely notification to the source node.

- Ambiguities in LTM execution arise if the resources originally configured for one UE are reused without explicit synchronization or acknowledgement.

##### 2.1 CSI-RS and PRACH resource deallocation

During Inter-CU Low-Latency Mobility (LTM) preparation, the source gNB (typically the current serving CU) initiates a resource provisioning request toward one or more candidate gNBs (i.e., target CUs) to facilitate the seamless execution of RACH-less mobility procedures. This preparation involves the explicit allocation of uplink PRACH resources and the configuration of CSI-RS transmission points, which are critical for enabling the UE to maintain time and frequency alignment with the candidate cell prior to mobility execution.

In response to the preparation request, each candidate gNB may allocate one or more CSI-RS Resource IDs along with associated transmission parameters (e.g., periodicity, offset, power, and antenna port configuration), as well as reserve PRACH resources within a defined temporal and spatial window. These resources are subsequently referenced by the source gNB when constructing mobility-triggering signaling—such as the downlink PDCCH order or dedicated RRC reconfiguration—intended for the UE during the execution phase.

However, a significant reliability risk arises from the potential deallocation of these provisioned resources by the candidate gNB prior to or during the execution phase, without any explicit notification or synchronization with the source gNB. Such uncoordinated deallocation may be driven by local radio resource management (RRM) decisions, PRACH congestion, cell load fluctuations, or resource repurposing. Without timely signaling to the source node, the latter may continue to rely on stale or invalid CSI-RS and PRACH configurations, which can result in execution failures, misalignment, or even RACH-less handover rejection by the target cell.

Observation 1: The current LTM preparation procedures allow for the allocation of CSI-RS and PRACH resources by candidate gNBs for use during RACH-less mobility. However, the deallocation of these resources by the candidate node (e.g., due to local scheduling, load, or RRM changes) is not necessarily coordinated with the source node, leading to potential inconsistency at execution time.

Proposal 1: RAN3 to consider the need for a notification mechanism (e.g., via XnAP or F1AP) that enables the candidate gNB to inform the source gNB of CSI-RS or PRACH resource deallocation before the execution phase, ensuring the source node maintains a valid resource view.

This concern becomes more acute in multi-candidate scenarios, where multiple target gNBs are pre-configured in parallel. In such cases, the source gNB maintains references to multiple PRACH and CSI-RS resources across candidates, with no implicit guarantee of ongoing validity unless proactive status updates or expiry controls are enforced. Importantly, the issue is not just the loss of resource availability, but the absence of a defined signaling mechanism to revoke or modify earlier resource allocations—particularly problematic when the preparation-to-execution latency exceeds typical reallocation thresholds on the candidate side.

Observation 2: In multi-candidate gNB scenarios, multiple CSI-RS and PRACH resources may be allocated in parallel, but the source node has no standardized method to determine whether a given resource is still valid at execution time. This undermines reliability and can lead to failed RACH-less mobility attempts.

Proposal 2: RAN3 to discuss the introduction of a resource validity timer or expiry indication in the LTM preparation response, allowing source nodes to discard stale resources and re-trigger preparation if needed.

RAN3 has acknowledged this granularity in its Working Assumption that, for both F1AP and XnAP, CSI-RS activation and deactivation procedures must operate at the level of individual CSI-RS Resource IDs, rather than assuming cell-wide or configuration-wide semantics. This highlights the importance of resource-level lifecycle management, where both the source and candidate gNBs maintain synchronized views of each active CSI-RS resource’s status throughout the LTM cycle.

Observation 3: The RAN3 working assumption states that CSI-RS activation and deactivation should occur at the level of individual Resource IDs. However, there is currently no specification text ensuring that both allocation and deallocation of CSI-RS resources are synchronized between source and candidate gNBs, particularly in asynchronous inter-CU scenarios.

Proposal 3: RAN3 to evaluate whether the granularity of CSI-RS resource-level activation/deactivation needs to be explicitly modeled in XnAP/F1AP LTM-related messages, possibly by including lifecycle state (e.g., active, deallocated, expiring) for each CSI-RS Resource ID.

Extending this principle, a similar level of granularity and synchronization is needed for PRACH resource lifecycle management. Without it, the source gNB risks issuing a PDCCH order referencing a PRACH resource that has been silently deallocated, leading to undefined behavior on the UE side and a loss of mobility robustness.

In summary, while current specifications support the allocation of CSI-RS and PRACH resources during LTM preparation, they do not yet prescribe a deterministic or mandatory feedback loop for resource invalidation. This results in temporal blind spots, especially in asynchronous or long-delay inter-CU LTM scenarios. To address this gap, future specification work should consider introducing lightweight signaling extensions (e.g., CSI-RS/PRACH Deallocation Notification or Resource Expiry Indication) to allow candidate gNBs to inform source gNBs of any relevant changes, thus preserving consistency and enabling proactive fallback or re-selection strategies.

Observation 4: There is no explicit fallback mechanism defined in case the originally allocated PRACH/CSI-RS resources become unavailable at the time of execution, potentially impacting UE behavior or mobility success rates.

Proposal 4: RAN3 to consider defining optional support for alternative or secondary resource sets during LTM preparation (e.g., a list of CSI-RS/PRACH fallback resources), along with a validity priority or ordering scheme to guide execution-stage selection.

##### 2.2 Ambiguities in LTM due to synchronization or acknowledgement

In the context of Inter-CU LTM (Last Triggered Mobility) preparation, the source gNB proactively coordinates with one or more candidate gNBs by initiating requests for the allocation of dedicated PRACH configurations and CSI-RS resource IDs. These resources are intended to enable RACH-less mobility execution at a future point in time, thereby reducing handover latency and improving mobility robustness. Upon receiving such a request, each candidate gNB may reserve a set of PRACH pools and CSI-RS resources, which are subsequently signaled back to the source node for association with a particular UE context.

However, a critical reliability risk arises when resource reuse occurs without clear, synchronized state management between the involved gNBs. Specifically, once a candidate gNB allocates a CSI-RS or PRACH resource set for a particular UE, there is no standardized mechanism to tie that allocation exclusively to the UE or to ensure its validity remains unchanged until execution. If the candidate gNB, due to internal optimization or resource limitations, decides to reassign or reuse the same CSI-RS or PRACH resource ID for another UE or purpose—without an explicit update or acknowledgement exchange with the original source gNB—ambiguities may emerge during LTM execution.

Observation 5: In the current specification, there is no defined mechanism to synchronize or acknowledge the reuse or reallocation of previously provisioned CSI-RS or PRACH resources between candidate and source gNBs in the Inter-CU LTM context. This may result in the source gNB triggering LTM procedures with outdated or invalid resource references, leading to failure in RACH-less mobility execution.

Proposal 5: RAN3 to consider introducing a resource state coordination mechanism over XnAP, allowing candidate gNBs to inform source gNBs upon reallocation or deactivation of previously allocated CSI-RS/PRACH resources for a given UE. This may involve a lightweight update or explicit resource invalidation message tied to a UE context ID.

This can lead to a number of problematic scenarios:

- The source gNB may issue a PDCCH order or UE configuration message based on now-invalid resource assumptions, directing the UE to attempt RACH-less handover using CSI-RS or PRACH resources that have been reassigned or deactivated.

- If the resource is now shared among multiple UEs, collision or interference may occur, leading to failed access attempts or degraded channel estimation.

- The candidate gNB may no longer be in a state to receive or process the expected transmission, resulting in missed scheduling opportunities or fallback to reactive handover procedures.

These outcomes not only compromise LTM reliability but also undermine the overall purpose of RACH-less mobility, which is to provide a deterministic and low-latency handover path. The root cause of this issue is the lack of a synchronized, transactional resource state model between the source and candidate gNBs. Unlike traditional handover procedures, LTM relies on pre-configured and static assumptions about future resource availability. Without an agreed mechanism for resource ownership, reservation lifetimes, and update triggers, both nodes may operate under outdated or conflicting views.

Observation 6: The specification does not currently define the validity duration or lifetime model for CSI-RS or PRACH resources once allocated during LTM preparation. Without an expiration timer or usage confirmation, resources may persist indefinitely or be reused prematurely, causing inconsistencies in execution.

Proposal 6: RAN3 to explore defining a time-based or usage-based validity model for LTM-related resource reservations, such that resource bindings automatically expire after a configurable timeout or after a single successful execution. Optional resource refresh signaling from the source gNB could be allowed to extend the validity period as needed.

To mitigate these risks, there is a strong need for explicit synchronization mechanisms, such as:

- Acknowledgement of resource consumption upon LTM execution,

- Notification procedures when resources are internally reassigned,

- UE-to-resource binding models that prevent reuse until resource deallocation is confirmed,

- Or, time-bound resource validity models that expire unless refreshed or extended.

RAN3 may consider defining a more deterministic lifecycle model for CSI-RS and PRACH resources within the LTM framework, thereby ensuring that once a resource is allocated for a specific UE, it cannot be reused or modified without a coordinated update to the originating gNB. Such enhancements would significantly improve execution consistency and reduce ambiguity in high-mobility or load-sensitive deployments.

Observation 7: The specification does not currently define the validity duration or lifetime model for CSI-RS or PRACH resources once allocated during LTM preparation. Without an expiration timer or usage confirmation, resources may persist indefinitely or be reused prematurely, causing inconsistencies in execution.

Proposal 7: RAN3 to explore defining a time-based or usage-based validity model for LTM-related resource reservations, such that resource bindings automatically expire after a configurable timeout or after a single successful execution. Optional resource refresh signaling from the source gNB could be allowed to extend the validity period as needed.

##### 2.3 Overall Discussions and Proposals on Signalling IE Changes

With the introduction of a node-level LTM CONFIGURATION UPDATE procedure—capable of encompassing a list of multiple target cells—a more modular and scalable mobility management framework is enabled. However, this added flexibility also introduces complexity, particularly in the context of per-cell security key generation and validation processes, which require precise resource state awareness.

Observation 8: With the adoption of a node-level LTM CONFIGURATION UPDATE procedure (containing a list of cells), and per-cell security key management, it becomes increasingly important to avoid implicit resource assumptions. Any dynamic change in resource availability (e.g., PRACH deallocation or CSI-RS deactivation) should be explicitly communicated to maintain secure and deterministic LTM behavior.

Implicit assumptions about the availability or persistence of resources like PRACH occasions or CSI-RS configurations may lead to synchronization mismatches or invalid mobility triggering. To ensure secure and deterministic behavior of LTM (L1-based Triggered Mobility), any reconfiguration, deactivation, or deallocation of such mobility-critical resources must be signaled explicitly to the involved nodes. This includes notifying the source gNB if any of the referenced cells modify or remove their mobility-related resources post-update. Otherwise, stale resource references may compromise handover robustness or violate security compliance (e.g., by triggering LTM with outdated or invalid keys). Therefore, a consistent and transparent resource lifecycle management approach—aligned with security and performance requirements—is crucial to fully realize the benefits of node-level LTM configuration.

Proposal 8: For Inter-CU LTM, when a candidate gNB deallocates any previously assigned PRACH Preamble Index or CSI-RS Resource ID associated with LTM, it shall notify the source gNB using an explicit signaling message or extension of the existing LTM CONFIGURATION UPDATE procedure over XnAP.

Proposal 9: For Intra-CU LTM, a similar mechanism shall be applied: if a candidate gNB-DU deallocates PRACH or CSI-RS resources previously granted for LTM, the source gNB-DU shall be informed via F1AP signaling. This ensures consistent behavior across distributed and centralized deployments.

To ensure robust and deterministic execution of Inter-CU L1-assisted mobility (LTM), particularly in RACH-less handover scenarios, it is proposed to enhance the existing signaling model defined in TS 38.401 for UE CONTEXT SETUP REQUEST/RESPONSE procedures between source Central Unit (CU) and candidate Distributed Unit (DU). These enhancements target the CSI-RS configuration exchange and the lifetime management of associated resources used during LTM preparation.

Indeed in current RAN architecture, when a source gNB-CU initiates an LTM preparation with one or more candidate gNBs, the CSI-RS resources required for future RACH-less mobility execution are provisioned by the candidate DU. However, the signaling interfaces do not formally capture:

- That the resource request pertains specifically to LTM use,

- Any constraints on the validity duration or usage scoping of the resources,

- Or the lifecycle expectations for such configurations (e.g., single-use vs. reusable).

This lack of explicit scoping leads to ambiguity and potential resource conflicts, especially when candidate DUs reuse CSI-RS resources for different UEs or sessions without coordination. Such reuse may cause execution failures or unintended collisions during LTM-triggered PDCCH orders.

The following updates to TS 38.401, Clause 9.3.2 (UE Context Setup Procedure) are proposed:

- LTM-Specific Resource Request Field:

- The UE CONTEXT SETUP REQUEST message shall be extended to optionally include an LTM Indication Flag, signaling that the source CU is requesting CSI-RS configuration explicitly for future LTM execution and not for immediate serving cell purposes.

- This indication helps the candidate DU distinguish LTM-related CSI-RS needs from those required for conventional mobility or addition procedures.

- CSI-RS Configuration Request Block:

- The same request message may optionally carry a CSI-RS Configuration Request block, indicating parameters such as preferred CSI-RS type (e.g., periodic/tracking/beamformed), desired density or spatial coverage, and a suggested validity period.

- If omitted, the candidate DU is free to assign default CSI-RS configurations according to local policies.

- Candidate DU Response and Resource Scoping:

- The UE CONTEXT SETUP RESPONSE from the candidate DU shall contain the CSI-RS Configuration Information allocated for the requesting UE.

This block may also contain optional fields indicating:

- Resource Scope (e.g., UE-specific, session-specific, or reusable),

- Validity Timer (absolute expiration or relative time from allocation),

- Usage Count or “Single-Use” marker indicating whether the resource should be invalidated after one LTM execution.

- Resource Identifier Tagging:

- To support later references by the source CU during LTM execution, the CSI-RS resource(s) shall be tagged with a Resource ID or index that can be consistently referenced in PDCCH Orders or downlink control signaling when triggering the LTM.

Proposal 10: Update TS 38.401 (Stage 2) to specify that during Inter-CU LTM, the UE CONTEXT SETUP REQUEST message may include a request from the source CU to the candidate DU to provide CSI-RS configuration, and that the UE CONTEXT SETUP RESPONSE from the candidate DU shall contain the corresponding CSI-RS configuration. These exchanges shall also reflect whether the resource is intended for LTM and may carry lifetime or validity parameters.

To strengthen the robustness, clarity, and determinism of L1-assisted mobility procedures across disaggregated gNB architectures, particularly in support of Inter-CU LTM (L1-assisted Target Measurements), we propose to introduce coordinated Change Requests (CRs) to TS 38.423 (F1 interface) and TS 38.473 (Xn interface) to address current signaling gaps in resource lifecycle management and to align Stage 3 procedures with recent agreements and baseline behaviours endorsed at RAN#129.

During LTM preparation, candidate gNBs allocate CSI-RS and/or PRACH resources in response to configuration requests from the source gNB. These resources are intended for later usage during the RACH-less handover execution or beam redirection. However, several issues currently undermine the reliability and manageability of this mechanism:

- There is no standardized signaling for deallocation or expiration of these resources, leading to prolonged or unnecessary resource occupancy in candidate nodes.

- The activation model for CSI-RS resources, recently updated at RAN#129 to a per-resource activation paradigm, is not yet reflected in the associated XnAP and F1AP procedures.

- The LTM CONFIGURATION UPDATE message lacks flexibility to express fine-grained control over resource lifecycles (e.g., delayed activation, time-bounded validity).

- Residual Editor’s Notes and optionality flags in TS 38.300 and related BL CRs still refer to obsolete procedural steps, creating ambiguity for implementation.

It Is then important to define explicit fields or message elements in F1AP (TS 38.473) and XnAP (TS 38.423) that allow the source CU to signal cancellation or release of previously allocated PRACH occasions and CSI-RS resources.

Proposal 11: Introduce Change Requests (CRs) targeting TS 38.423 and 38.473 to:

- Include signaling support for deallocation notification of PRACH and CSI-RS resources assigned for LTM.

- Align the message flows with the per-CSI-RS resource activation model as agreed at RAN#129.

- Extend the LTM CONFIGURATION UPDATE procedure with optional resource lifecycle control fields (activation/deactivation, validity window, etc.).

- Remove outdated or unclear optionality notes (e.g., the prior Editor’s Note on steps 6 and 7 in TS 38.300 BL CR), ensuring consistency with new baseline behavior.

# 3	Conclusion

Follows the summary of Observations and Proposals:

Observation 1: The current LTM preparation procedures allow for the allocation of CSI-RS and PRACH resources by candidate gNBs for use during RACH-less mobility. However, the deallocation of these resources by the candidate node (e.g., due to local scheduling, load, or RRM changes) is not necessarily coordinated with the source node, leading to potential inconsistency at execution time.

Proposal 1: RAN3 to consider the need for a notification mechanism (e.g., via XnAP or F1AP) that enables the candidate gNB to inform the source gNB of CSI-RS or PRACH resource deallocation before the execution phase, ensuring the source node maintains a valid resource view.

Observation 2: In multi-candidate gNB scenarios, multiple CSI-RS and PRACH resources may be allocated in parallel, but the source node has no standardized method to determine whether a given resource is still valid at execution time. This undermines reliability and can lead to failed RACH-less mobility attempts.

Proposal 2: RAN3 to discuss the introduction of a resource validity timer or expiry indication in the LTM preparation response, allowing source nodes to discard stale resources and re-trigger preparation if needed.

Observation 3: The RAN3 working assumption states that CSI-RS activation and deactivation should occur at the level of individual Resource IDs. However, there is currently no specification text ensuring that both allocation and deallocation of CSI-RS resources are synchronized between source and candidate gNBs, particularly in asynchronous inter-CU scenarios.

Proposal 3: RAN3 to evaluate whether the granularity of CSI-RS resource-level activation/deactivation needs to be explicitly modeled in XnAP/F1AP LTM-related messages, possibly by including lifecycle state (e.g., active, deallocated, expiring) for each CSI-RS Resource ID.

Observation 4: There is no explicit fallback mechanism defined in case the originally allocated PRACH/CSI-RS resources become unavailable at the time of execution, potentially impacting UE behavior or mobility success rates.

Proposal 4: RAN3 to consider defining optional support for alternative or secondary resource sets during LTM preparation (e.g., a list of CSI-RS/PRACH fallback resources), along with a validity priority or ordering scheme to guide execution-stage selection.

Observation 5: In the current specification, there is no defined mechanism to synchronize or acknowledge the reuse or reallocation of previously provisioned CSI-RS or PRACH resources between candidate and source gNBs in the Inter-CU LTM context. This may result in the source gNB triggering LTM procedures with outdated or invalid resource references, leading to failure in RACH-less mobility execution.

Proposal 5: RAN3 to consider introducing a resource state coordination mechanism over XnAP, allowing candidate gNBs to inform source gNBs upon reallocation or deactivation of previously allocated CSI-RS/PRACH resources for a given UE. This may involve a lightweight update or explicit resource invalidation message tied to a UE context ID.

Observation 6: The specification does not currently define the validity duration or lifetime model for CSI-RS or PRACH resources once allocated during LTM preparation. Without an expiration timer or usage confirmation, resources may persist indefinitely or be reused prematurely, causing inconsistencies in execution.

Proposal 6: RAN3 to explore defining a time-based or usage-based validity model for LTM-related resource reservations, such that resource bindings automatically expire after a configurable timeout or after a single successful execution. Optional resource refresh signaling from the source gNB could be allowed to extend the validity period as needed.

Observation 7: The specification does not currently define the validity duration or lifetime model for CSI-RS or PRACH resources once allocated during LTM preparation. Without an expiration timer or usage confirmation, resources may persist indefinitely or be reused prematurely, causing inconsistencies in execution.

Proposal 7: RAN3 to explore defining a time-based or usage-based validity model for LTM-related resource reservations, such that resource bindings automatically expire after a configurable timeout or after a single successful execution. Optional resource refresh signaling from the source gNB could be allowed to extend the validity period as needed.

Observation 8: With the adoption of a node-level LTM CONFIGURATION UPDATE procedure (containing a list of cells), and per-cell security key management, it becomes increasingly important to avoid implicit resource assumptions. Any dynamic change in resource availability (e.g., PRACH deallocation or CSI-RS deactivation) should be explicitly communicated to maintain secure and deterministic LTM behavior.

Proposal 8: For Inter-CU LTM, when a candidate gNB deallocates any previously assigned PRACH Preamble Index or CSI-RS Resource ID associated with LTM, it shall notify the source gNB using an explicit signaling message or extension of the existing LTM CONFIGURATION UPDATE procedure over XnAP.

Proposal 9: For Intra-CU LTM, a similar mechanism shall be applied: if a candidate gNB-DU deallocates PRACH or CSI-RS resources previously granted for LTM, the source gNB-DU shall be informed via F1AP signaling. This ensures consistent behavior across distributed and centralized deployments.

Proposal 10: Update TS 38.401 (Stage 2) to specify that during Inter-CU LTM, the UE CONTEXT SETUP REQUEST message may include a request from the source CU to the candidate DU to provide CSI-RS configuration, and that the UE CONTEXT SETUP RESPONSE from the candidate DU shall contain the corresponding CSI-RS configuration. These exchanges shall also reflect whether the resource is intended for LTM and may carry lifetime or validity parameters.

Proposal 11: Introduce Change Requests (CRs) targeting TS 38.423 and 38.473 to:

- Include signaling support for deallocation notification of PRACH and CSI-RS resources assigned for LTM.

- Align the message flows with the per-CSI-RS resource activation model as agreed at RAN#129.

- Extend the LTM CONFIGURATION UPDATE procedure with optional resource lifecycle control fields (activation/deactivation, validity window, etc.).

- Remove outdated or unclear optionality notes (e.g., the prior Editor’s Note on steps 6 and 7 in TS 38.300 BL CR), ensuring consistency with new baseline behavior.