3GPP TSG-RAN WG3 Meeting #129	R3-255602

# Bengaluru, India, 25th – 29th Aug, 2025

Title: 	Reconsideration on NG Suspend/Resume Signaling for FLSO

Source: 	Jio Platforms

Agenda item:	14.3

Document Type:	Discussion and decision

# 1	Introduction

During RAN3#128 the following WA was considered:

WA: For regenerative payload, an indication to suspend or resume NG connection may be sent to AMF from gNB in the RAN Configuration Update procedure.

For the continuation of the subject it was also proposed:

Check the use cases including hard feeder link switch; other use cases to be further justified.

Throughout Rel-19, the technical part of this discussion would have benefited from increased time allocation. We provide here a more comprehensive analysis of the arguments in favor of NG suspend/resume from supporting companies so far. We provide this as a basis for continued discussion in order to resolve the many outstanding technical issues with this proposal.

This contribution addresses the proposal to include a suspend/resume indicator in the RAN Configuration Update message from gNB to AMF in the context of regenerative NTN architectures with hard Feeder Link Switch Over (FLSO). We recognize that the FFS discussions at RAN3 #127bis explored this as a possible solution to transient link disruptions during FLSO. However, based on architectural analysis, system impact, and signaling implications, we recommend against such an addition and propose alternative interpretations aligned with system efficiency and 3GPP principles.

# 2	NGAP Suspend/Resume proposal

##### 2.1 FLSO with Suspend/Resume procedure opposition

Following the Work Agreement reached at RAN3 meeting #128, enhancements to the NG interface behavior during hard feeder link switch-over (FLSO) in regenerative payload scenarios are being addressed in ongoing Change Request R3-254012. Specifically, the proposed procedure allows the gNB to inform the AMF, via the RAN Configuration Update message, to temporarily suspend or resume both control and user plane traffic over the NG interface when a feeder link handover is triggered.

This signaling is enabled through the inclusion of a newly defined optional information element—Downlink NG Transmission Control—which utilizes an extendable ENUMERATED type with “suspend” and “resume” values. The gNB's inclusion of this IE in the update message prompts the AMF to initiate the corresponding action, provided the feature is supported.

In regenerative NTN systems, a hard feeder link switch is a deterministic, non-random event, controlled and scheduled by the NTN control function. Such switches are typically designed to minimize service disruption and, where necessary, leverage predictive offloading or buffering mechanisms at the gNB level.

Observation 1: Feeder link switch over (FLSO)  is a deterministic event easily predicted by regenerative NTN gNB or AMF.

The RAN Configuration Update procedure in NGAP is designed for gNB-level configuration changes (e.g., TAI updates, PLMN modifications), not for manipulating per-UE PDU session states. Introducing suspend/resume functionality via this procedure would:

- Break protocol layering: Mixing session-level control with node-level updates violates the separation of concerns in NGAP.

- Create ambiguity: The AMF may misinterpret such messages as node-wide rather than per-session actions, leading to misconfigurations.

- Increase standardization risk: Future NGAP extensions could be complicated by this precedent.

As a conclusion a new, dedicated procedure would be needed—but given the transient nature of FLSO, such complexity is unjustified.

Observation 2: NGAP suspend/resume over RAN Configuration Update Procedure for FLSO would violate the NGAP Layering Principles

Unlike unexpected failures (e.g., fiber cuts), FLSO is scheduled (via ephemeris data) and lasts <1 second. Thus there is no need for 5GC involvement, the gNB can preemptively buffer or reroute traffic without signaling. Moreover the TNL (Transport Network Layer) retransmissions (e.g., SCTP) handle successfully brief interruptions.

Furthermore NGAP carries far less traffic than UP. Instead of suspending all NG traffic:

- gNB can buffer NGAP messages (e.g., Path Switch Requests).

- AMF can tolerate minor delays (existing timers handle this).

Observation 3: Explicit suspend/resume signaling introduces latency that may exceed the FLSO duration itself, hence overhead outweighs benefit

Observation 4: The suspend/resume proposal solves a non-existent problem—FLSO does not require 5GC coordination.

Proposal 1: RAN3 do not further consider the suspend/resume proposal for the FLSO functionality

##### 2.2 Alternative solution to FLSO without suspend/resume

The proposed suspend/resume mechanism for handling FLSO in regenerative NTN gNBs introduces unnecessary complexity, given the predictable and ultra-short nature of FLSO interruptions (typically <1 ms). Such brief disruptions can be effectively managed at the transport layer (e.g., via SCTP retransmissions and buffering) without requiring explicit NGAP signaling to the 5GC.

The average medium throughput in NTN scenarios further supports this approach, as transport-layer resilience (e.g., TNL retransmissions, packet buffering) is sufficient to mitigate any transient packet loss. Given that FLSO events are scheduled and last only milliseconds, involving the core network via suspend/resume procedures introduces excessive control-plane overhead without tangible performance benefits. Instead, the gNB can autonomously manage FLSO transitions using existing protocol mechanisms, avoiding additional signaling latency that may exceed the interruption duration itself.

Since most user-plane traffic tolerates sub-second delays, and control-plane messages can be buffered locally, there is no demonstrated need for 5GC coordination. The proposal’s reliance on NGAP modifications also contradicts prior RAN3 decisions, where similar suspend/resume concepts were rejected due to scalability concerns. Given these factors, the most efficient solution is to rely on lower-layer recovery mechanisms rather than introducing new core network signaling for an event that is both predictable and fleeting. Some proposed alternative methods follow to efficiently support Hard Feeder Link Switch Over (FLSO) in 5G NTN regenerative payload architecture, ensuring minimal interruption for both Control Plane (CP) and User Plane (UP) without requiring suspend/resume signaling over NG interfaces

Leveraging the deterministic motion of LEO satellites using GNSS data and orbital ephemeris, allows the NTN Control Function (NTN-CF) or the gNB to predict exact FLSO time and pre-schedule traffic offloading, buffer uplink and downlink traffic and/or notify UEs indirectly using existing signaling

Proposal 2: Use Predictive Pre-Scheduling Based on Ephemeris Awareness to handle the FLSO short interruption time issues

Another proposed approach/solution (typically vendor proprietary implementation) is to design the regenerative payload to maintain simultaneous or overlapping connectivity with two gateways during FLSO. Hence during overlap the switch is seamless and no link loss is perceived at the gNB. The benefit is obvious, practically zero downtime if soft-switch handover is supported; requires architectural enhancement at payload and GWs.

Proposal 3: Allow the NTN solution to support Multi-Feeder Redundant Connectivity (Soft Diversity)

Another proposal is to align the feeder switch to a TTI boundary with guard period to avoid ongoing HARQ or scheduling windows. In such a way the system ensures all MAC and PHY processes as completed before switch and gNB scheduler ensures that FLSO happens during natural TTI gaps or empty scheduling opportunities. Consequently there is no CP/UP loss or delay, since it is fully handled in MAC scheduler.

Proposal 4: Slot-Level TTI-Aware FLSO Coordination

# 3	Conclusion

Follows the summary of Observations and Proposals:

Observation 1: Feeder link switch over (FLSO)  is a deterministic event easily predicted by regenerative NTN gNB or AMF.

Observation 2: NGAP suspend/resume over RAN Configuration Update Procedure for FLSO would violate the NGAP Layering Principles

Observation 3: Explicit suspend/resume signaling introduces latency that may exceed the FLSO duration itself, hence overhead outweighs benefit

Observation 4: The suspend/resume proposal solves a non-existent problem—FLSO does not require 5GC coordination.

Proposal 1: RAN3 do not further consider the suspend/resume proposal for the FLSO functionality

Proposal 2: Use Predictive Pre-Scheduling Based on Ephemeris Awareness to handle the FLSO short interruption time issues

Proposal 3: Allow the NTN solution to support Multi-Feeder Redundant Connectivity (Soft Diversity)

Proposal 4: Slot-Level TTI-Aware FLSO Coordination