3GPP TSG-RAN WG3 Meeting #129	R3-255603

Bengaluru, India, 25th – 29th August, 2025

Title: 	Packet loss metric Observability in UE performance

Source: 	Jio Platforms

Agenda item:	11.2

Document Type:	Discussion and agreement

# 1. Introduction

This contribution addresses key aspects of the Release-19 WI on AI/ML enhancements for NG-RAN, building on prior study work summarized in TR 38.743, as captured also in meeting minutes from R3#128:

«Specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Slicing. [RAN3]»

This document focuses on examining limitations in the current Packet Loss metric and proposes improvements to support more reliable UE performance evaluations in Release-19.

# 2. Discussion

In the context of Release-19, RAN3 continues to develop the Work Item (WI) “Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (RP-251245), which builds upon earlier study efforts (RP-240323) and the findings captured in TR 38.743. A key objective of the WI is to define enhancements for AI/ML deployment in NG-RAN by refining data collection and signaling mechanisms, with applicability across both split and non-split architectures. These enhancements aim to support intelligent features such as AI/ML-driven Network Slicing and mobility optimization.

An important element in this process is the treatment of UE performance metrics, including Average DL/UL Throughput, Average DL/UL Packet Delay, and particularly Average Packet Loss in the DL direction. These metrics are gathered at the target gNB following a handover, and reported back to the source gNB once specific exit conditions are met. This feedback loop enables the source gNB to evaluate post-handover performance and potentially fine-tune AI/ML-based mobility models. The feedback can be used either to adjust decision-making in future mobility scenarios or to support offline model retraining and validation.

However, Release-18-based frameworks have demonstrated limitations in extracting meaningful insights from such metrics—especially Packet Loss—which motivates further refinement in Release-19. This contribution discusses proposed enhancements to improve the accuracy and utility of UE measurement reporting. Additionally, it addresses how these metrics can support both real-time AI/ML-based decisions and training data generation when model-based actions are not yet deployed. These developments contribute to a broader effort to enhance RAN intelligence and ensure robust, performance-driven mobility management in evolving NG-RAN systems.

## Challenges and Limitations of Packet Loss Rate Metrics

In response to the liaison from SA5 concerning the availability of PDCP-level metrics for evaluating packet loss in the UL and DL directions, it is acknowledged that 3GPP TS 28.552 defines the following relevant measurements within the context of split gNB deployments:

- UL PDCP SDU Loss Rate (clause 5.1.3.1.1), which quantifies the fraction of PDCP SDUs not successfully received at the gNB-CU-UP, accounting for losses over the radio interface, internal gNB functions, and F1-U transport;

- DL PDCP SDU Drop Rate in gNB-CU-UP (clause 5.1.3.2.1), which represents the ratio of PDCP SDUs discarded in the downlink due to congestion or internal queuing policies within the CU-UP, prior to transmission over any user-plane interface.

These measurements are offered at the per-DRB level and can be optionally broken down further by QoS class identifiers (e.g., 5QI) and supported S-NSSAIs, thus providing granularity suitable for slice-level monitoring. However, there are limitations to directly mapping these metrics to UE-level performance evaluations, especially for the purpose of signaling Average DL/UL Packet Loss per UE, as envisioned in AI/ML-assisted mobility procedures.

Observation 1: While per-DRB and per-S-NSSAI granularity enables slice-level traffic monitoring, this level of abstraction does not capture individual UE behavior, which is critical for mobility performance tuning and for training per-UE AI/ML models. There is a growing need to correlate such aggregated metrics with per-UE experience indicators.

Firstly, the metrics defined in TS 28.552 are scoped for NG-RAN-wide performance reporting and are not intended to reflect per-UE statistics in their native form. In contrast, UE-level KPIs are explicitly addressed in TS 28.558, which supports UE-centric granularity needed for mobility learning frameworks.

Secondly, the applicability of these PDCP-layer measurements is constrained to architectures where gNB functions are functionally split, particularly where a CU-UP is deployed. In scenarios involving non-split gNB deployments—where user-plane and control-plane functions are co-located—the PDCP-layer drop metrics as defined in TS 28.552 become inapplicable or undefined, as the functional boundary required for such measurements does not exist.

Given these considerations, while the existing SA5 metrics provide a useful basis for monitoring PDCP performance under split architectures, their suitability for generalized per-UE packet loss tracking—especially for training or validating AI/ML-based mobility models—remains limited. For a more unified and architecture-agnostic approach, enhancements to the measurement framework may be needed to support consistent per-UE packet loss metrics across both split and non-split configurations, enabling their effective use in AI/ML inference and feedback loops.

Observation 2: The current dependency of PDCP-layer loss measurements on split architecture limits their applicability in real-world deployments where non-split gNBs remain prevalent. This architectural inconsistency poses challenges for standardized metric collection across diverse RAN deployments and hinders uniform AI/ML model training and validation.

## Packet Loss Rate Metrics Proposals

As AI/ML-based mobility strategies become increasingly integrated into NG-RAN operations under Release-19, the ability to monitor and report packet loss rates with high accuracy and fidelity is critical. The latest version of the XnAP Backward Link Context Reporting (BLCR) specification introduces Downlink and Uplink Average Slice Packet Loss metrics as encoded integers, provisionally ranging from 0 to 10⁷. However, the final determination of this encoding range remains open for discussion, as acknowledged by the FFS (“For Further Study”) marker currently associated with the information element type and reference.

This encoding was initially proposed in R3-253431, which highlighted deficiencies in the Release-18 definition—particularly its inability to represent low packet loss rates with sufficient resolution for AI/ML use cases. The absence of explicit granularity in the current signaling structure limits the interpretability of packet loss values reported after mobility actions. This is especially problematic in scenarios where UE performance post-handover must be evaluated to refine or retrain AI/ML-based decision models.

Observation 3: The current encoding does not indicate the resolution at which packet loss values are measured or reported. Without an agreed granularity, the receiving node cannot reconstruct the actual percentage of packet loss experienced, undermining the usefulness of the metric in high-precision applications such as URLLC or AI-based handover validation.

To address this, R3-253431 suggests adopting a measurement granularity of 10⁻⁸, allowing representation of loss rates as small as 0.00000001%. This aligns well with the needs of services with stringent QoS requirements (e.g., 5QI classes with guaranteed loss bounds of 10⁻⁷ to 10⁻⁸). Nevertheless, there are alternative viewpoints suggesting that 10⁻⁹ granularity may provide even finer resolution, accommodating observed packet loss behaviors in extremely low-loss contexts (e.g., mmWave with small packets and strong link adaptation).

Observation 4: While 10⁻⁹ granularity may appear advantageous in capturing minute variations, it may introduce overhead in implementation due to larger numeric representation and reduced encoding efficiency. On the other hand, 10⁻⁸ offers a balance between precision and practical signaling size, and is already sufficient to support a broad range of use cases with ultra-low packet loss requirements.

Proposal 1: RAN3 is invited to agree on removing the FFS status associated with the “IE type and reference” for DL/UL Average Slice Packet Loss metrics in XnAP BLCR, and to formalize an encoding scheme that includes both the value range and measurement granularity.

Proposal 2:To ensure consistency with the variety of packet loss profiles observed across QoS flows, the value range for packet loss rate encoding should span from 10⁻¹ to 10⁻¹⁰ (0.0000000001%), allowing support for both best-effort and highly reliable applications.

A clearly defined range and granularity in packet loss metrics not only improves the interpretability of feedback used in AI/ML model tuning but also enables harmonized behavior across vendor implementations, supporting end-to-end consistency in multi-vendor and multi-node deployments.

Proposal 3: The selected measurement granularity should be set at 10⁻⁸, providing a precise yet manageable level of detail that meets the requirements of critical services while maintaining signaling efficiency and alignment with known QoS profiles.

A TP reflecting the main proposals in this paper is presented in R3-253432.

# 3. Conclusions

In summary, Release-19 presents an opportunity to enhance the practical usability of packet loss metrics within XnAP signaling, by formalizing value scaling and granularity. This evolution is key to enabling accurate post-handover performance assessment and feedback-driven AI/ML mobility optimization.

This paper makes the following observations and proposals:

Observation 1: While per-DRB and per-S-NSSAI granularity enables slice-level traffic monitoring, this level of abstraction does not capture individual UE behavior, which is critical for mobility performance tuning and for training per-UE AI/ML models. There is a growing need to correlate such aggregated metrics with per-UE experience indicators.

Observation 2: The current dependency of PDCP-layer loss measurements on split architecture limits their applicability in real-world deployments where non-split gNBs remain prevalent. This architectural inconsistency poses challenges for standardized metric collection across diverse RAN deployments and hinders uniform AI/ML model training and validation.Proposal 3: The new definition of packet loss rate for UE performance shall accommodate a granularity that is detailed enough to use for applications with very low packet loss rates.

Observation 3: The current encoding does not indicate the resolution at which packet loss values are measured or reported. Without an agreed granularity, the receiving node cannot reconstruct the actual percentage of packet loss experienced, undermining the usefulness of the metric in high-precision applications such as URLLC or AI-based handover validation.

Observation 4: While 10⁻⁹ granularity may appear advantageous in capturing minute variations, it may introduce overhead in implementation due to larger numeric representation and reduced encoding efficiency. On the other hand, 10⁻⁸ offers a balance between precision and practical signaling size, and is already sufficient to support a broad range of use cases with ultra-low packet loss requirements.

Proposal 1: RAN3 is invited to agree on removing the FFS status associated with the “IE type and reference” for DL/UL Average Slice Packet Loss metrics in XnAP BLCR, and to formalize an encoding scheme that includes both the value range and measurement granularity.

Proposal 2:To ensure consistency with the variety of packet loss profiles observed across QoS flows, the value range for packet loss rate encoding should span from 10⁻¹ to 10⁻¹⁰ (0.0000000001%), allowing support for both best-effort and highly reliable applications.

Proposal 3: The selected measurement granularity should be set at 10⁻⁸, providing a precise yet manageable level of detail that meets the requirements of critical services while maintaining signaling efficiency and alignment with known QoS profiles.