3GPP TSG-RAN WG3 Meeting #129	R3-255605

Bengaluru, India, 25 – 29 August 2025

Agenda Item:	13.2

Source:	CATT

Title:	(TP for 37.340) Discussion for Inter-CU LTM in DC

Document for:	Decision

- Introduction

In this paper, we will analyse the issues in inter-CU LTM.

Update the online agreement to: CU can request Candidate DU to provide CSI-RS configuration in UE CONTEXT SETUP REQUEST and UE CONTEXT MODIFICATION REQUEST message, and Candidate DU signals the CSI-RS configuration in UE CONTEXT SETUP RESPONSE and UE CONTEXT MODIFICATION RESPONSE message.

Reuse the CSI-RS coordination procedure over F1AP and XnAP for source gNB/gNB-DU to activate or deactivate the SP CSI-RS resource for CSI acquisition in candidate cell.

In Xn interface, candidate gNB provides the LTM CFRA Resource Configuration of each candidate cell to source gNB for LTM cell switch command generation.

Source gNB generate the UE Based TA Measurement Configuration, and transfer it to all candidate gNB(s) via LTM Configuration Update message.

Include the Rel-19 set IDs of source cell and each candidate cell(s) in UE Context Modification Request message. Introduce Rel-19 set IDs into LTM Security Information IE.

FFS on the Rel-19 Set ID(s) assignment among CUs, down select from Option1 and Option2:

Option 1: Source gNB sends the Rel-19 Set ID(s) or Rel-19 set ID range assigned to the candidate gNB in the HANDOVER REQUEST message, then candidate gNB assigns Rel-19 set ID(s) to its own candidate cells and feedback via HANDOVER REQUEST ACKNOWLEDGE message.

Option 2: Candidate gNB provides Rel-19 set ID per candidate cell in HANDOVER REQUEST ACKNOWLEDGE message, then source gNB may update the Rel-19 set ID to ensure that the Rel-19 set IDs under different candidate gNB-CU are different.

FFS on the procedure to be used for source gNB to transfer Rel-19 set ID per candidate cell to the candidate gNB.

FFS on whether gNB-DU/gNB provides the report type (periodic or semi-persistent) of the CSI-RS resources in both F1AP and XnAP.

FFS on whether the TCI State/QCL-info List needs to be included in CSI-RS COORDINATION procedure.

FFS on whether to include the SP CSI-RS and SSB mapping in the HANDOVER REQUEST ACKKNOLEDGE message and UE Context Modification Request message.

FFS on whether to add a new IE for SP CSI-RS resource for CSI acquisition in the corresponding procedure of SP CSI-RS resource for L1 RSRP measurement in F1AP and XnAP.

The suggested PSCell list should be explicitly included in the SN Change Required message.

The maximum number of PSCells that can be prepared by each candidate SN is included in the SN Change Required message.

The LTM Configuration ID Mapping List IE may be included in the SN Change Required message and the SN Addition Request message.

The LTM Security Configuration List IE including a list of pair of {security key, sk-counter} is included in the SN Addition Request message and the SN Modification Request message to support subsequent inter-CU SCG LTM.

The SN Security Key IE included in the SN Addition Request message should be ignored if the procedure is triggered for the LTM.

WA: Different candidate PSCells in the same SN can have different Rel-19 set IDs. FFS on the solutions. Try to reuse the same solution for inter-CU LTM.

If the source SN has the SCG reference configuration, the source SN will provide the SCG reference configuration in the SN Change Required message and thus the MN will not request source SN to provide the SCG reference configuration.

Early data forwarding is supported in inter-CU LTM in DC.

Normal data forwarding can be initiated after the source SN decides to trigger the LTM cell switch for the UE. The exact timing of its initiation is left to implementation.

Enhance XN-U ADDRESS INDICATION message and define IE to cover Inter-CU MCG LTM case. FFS on Inter-SN LTM case.

MN uses SN modification request message to notify the Source SN that UE has successfully accessed to the target SN. FFS whether Handover Success is used from the target-SN to the MN to notify that UE has successfully accessed to the target SN.

The source SN generates the common LTM CSI resource configuration for inter-CU SCG LTM and then provides the common LTM CSI resource configuration to the MN via the SN Modification Request ACK message.

For the SSB based L1 measurement, the common CSI resource configuration may be included in the SN Change Required message and the SN Addition Request message.

The CSI Resource Configuration IE is the common CSI resource configuration, which refers to the ltm-CSI-ResourceConfig-r18 IE in the RRC spec.

The L1 Configuration IE is the L1 RS configuration per candidate PSCell, which refers to the ltm-SSB-Config-r18 IE in the RRC spec for the SSB based L1 measurement, or the ltm-NZP-CSI-RS-ResourceConfigToAddModList-r19 IE associated with the ltm-NZP-CSI-RS-ResourceSetToAddModList-r19 IE in the RRC spec for the CSI-RS based L1 measurement.

For the avoidance of simultaneous configurations of inter-CU MCG LTM and inter-CU SCG LTM, no RAN3 impact is foreseen.

FFS on whether LTM modification/cancel related procedures are needed.

The SP CSI-RS activation/(de)activation for the inter-CU LTM in DC is deprioritized in rel-19.

- Discussion

  - Inter-SCG LTM

Security issue

In RAN2, the following agreements have been reached:

- NW configures the corresponding sk-Counter in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG.

- A list of sk-counters is linked to a Rel-19 set ID configured by the SN.

In the S-CPAC like solution:

- MN shall send the { S-KSN,sk-Counter } list to each candidate SN via SN ADDITION REQUEST message, and SN shall use the first unused S-KSN for when UE access.

- Once UE send RRCReconfigurationcomplete to the MN, MN shall include the, sk-Counter in S-NODE RECONFIGURATION COMPLETE message.

Further, in RAN2, it agreed to support different Rel-19 ID even in the same SN. Similar with SA, there are two possible options:

- Option 1: MN sends { S-KSN,sk-Counter } list assigned to the candidate SN in the SN addition request message, then the candidate SN divide the { S-KSN,sk-Counter } list into multiple lists, and assigns {S-KSN,sk-Counter} sub-list to its own candidate cells and feedback via SN addition response message.

- Option 2: MN provides multiple { S-KSN,sk-Counter} lists to the candidate SN in the SN addition request message, MN and candidate SN may need to negotiate the number of list in advance or additional update procedure.

From our perspective, option 1 can be more efficient, which can assign security information at the first interaction between MN and candidate SN

Proposal 1: For the security issue, support MN sends { S-KSN,sk-Counter } list assigned to the candidate SN in the SN addition request message, then the candidate SN divide the { S-KSN,sk-Counter } list into multiple lists, and assigns {S-KSN, sk-Counter} list to its own candidate cells if needed. Candidate SN feedback the final { S-KSN, sk-Counter } list associated with candidate cells via SN addition response message.

ACCESS SUCCESS

Proposal 6-4: MN uses SN modification request message to notify the Source SN that UE has successfully accessed to the target SN. FFS whether Handover Success is used from the target-SN to the MN to notify that UE has successfully accessed to the target SN.

For the information after completion of SN initiated inter-CU SCG LTM, there are three options:

- Option 1: Reuse the SN MODIFICATION REQIRED message to inform the MN that UE successfully accessed the target PSCell.

- Option 2: Reuse the HANDOVER SUCCESS message to inform theMN the UE successfully accessed the target PSCell.

- Option 3: no need to inform, MN can be notification via RRCReconfigurtaion

As the MN will send the SN Reconfiguration Complete message to the T-SN, it is enough for MN to know UE has accessed. No need to introduce interaction between T-SN and MN.

Proposal 2: NO need to inform message from T-SN to MN about UE access.

LTM modification/cancel

Case 1: candidate SN can decide to update or release some of the prepared candidate PSCells.

Case 2: source SN decides to update or release some of the suggested candidate PSCell

- Case 2-1: the source SN decides to add or release a candidate PSCell prepared at the source SN

- Case 2-2: the source SN decides to update the suggested candidate PSCells based on the latest L3 measurement results, or to update the upper limited number of PSCells of the candidate SN.

For the LTM cancel case, try to make the current mechanism simple, we suggest only support basic function. For the above case, we think the candidate cell decide to release some of the prepared PScell is a basic procedure, only support the candidate triggered cancel in REL-19.

Proposal 3: For the partial LTM cancel, only support the candidate triggered cancel in REL-19.

Proposal 4: Do not support prepared candidate PScell modification in REL-19.

  - Inter MCG LTM with SN kept or release

In REL-19, we only support inter-MCG LTM with SN kept or release.

For inter-MCG LTM with SN kept, the target MN can only connected to the same SN as source.

For the inter-MCG LTM with SN release, the target node can without SN or connected with other SNs.

Based on this understanding, the taget MN can not connected with source SN and other potential target SN at the same time. We suggest remove SN ADDITION REQUEST message form target MN to other potential target SN in step 2.

Proposal 5: Remove SN ADDITION REQUEST message form target MN to other potential target SN in step 2.

What’s more, different with one shot handover, LTM can support subsequent cell switch. The prepared resource will not mend to release each time after HO.

Step 20a/20b: change the two steps to optional, only when no candidate cell left in the initial source MN, the source MN shall release source SN.

Step 20d/20e: remove the SN RELEASE procedure after HO.

Step 30/31: change the two steps to optional, only no candidate cell left in the initial source MN, the source MN shall release UE context.

Proposal 6: Remove unnecessary release procedure as LTM supports subsequent HO.

For data handing, possible cases list as below:

| Scenario | MCG LTM with SCG kept | MCG LTM with SCG release |  |
| --- | --- | --- | --- |
|  | Connected with source SN | No SN | Connected with other SN |
| Interaction between source SN to source MN | Data forwarding for SN terminated bearer. | Data forwarding for SN terminated bearer. Whether support early data forwarding? | Data forwarding for SN terminated bearer. Whether support early data forwarding? |
| Interaction between source MN and target MN | Early data forwarding Data forwarding | Early data forwarding Data forwarding | Early data forwarding Data forwarding |
| Interaction between target MN to target SN | No data forwarding or early data forwarding | No data forwarding or early data forwarding | No data forwarding or early data forwarding |

It can be seen, in all the case, data forwarding and early data forwarding are not needed. Similar, all the relative procedure can be removed.

Step 4, 20c: remove the description about multiple data forwarding addresses from target MN to source MN, data forwarding only will happen from source MN to target MN.

Step 22c in figure remove the description about target MN forward the SN Status to the target SN.

Proposal 7: Remove the description about data forwarding/early data forwarding form source SN to target SN.

An unclearly point is whether source SN can execute early data forwarding to source MN for SN terminated bearer. In MCG LTM with SCG release case maybe useful as source MN may further execute early data forwarding to the target MN.

Proposal 8: Discussion whether need support early data forwarding form source SN to source MN.

After step 28/29, the new source cell update security key for each candidate MN. If SN is configured, each candidate MN shall update the KSN via SN Modification procedure.

Proposal 9: Add SN modification procedure between candidate MN and SN to update KSN.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals:

Proposal 1: For the security issue, support MN sends { S-KSN,sk-Counter } list assigned to the candidate SN in the SN addition request message, then the candidate SN divide the { S-KSN,sk-Counter } list into multiple lists, and assigns {S-KSN, sk-Counter} list to its own candidate cells if needed. Candidate SN feedback the final { S-KSN, sk-Counter } list associated with candidate cells via SN addition response message.

Proposal 2: NO need to inform message from T-SN to MN about UE access.

Proposal 3: For the partial LTM cancel, only support the candidate triggered cancel in REL-19.

Proposal 4: Do not support prepared candidate PScell modification in REL-19.

Proposal 5: Remove SN ADDITION REQUEST message form target MN to other potential target SN in step 2.

Proposal 6: Remove unnecessary release procedure as LTM supports subsequent HO.

Proposal 7: Remove the description about data forwarding/early data forwarding form source SN to target SN.

Proposal 8: Discussion whether need support early data forwarding form source SN to source MN.

Proposal 9: Add SN modification procedure between candidate MN and SN to update KSN.

5. TP for BLCR

### 10.5.2	MR-DC with 5GC

SN initiated inter-SN SCG LTM

The SN initiated inter-SN SCG LTM procedure is used to configure inter-SN SCG LTM.

Figure 10.5.2-5 SN initiated inter-SN SCG LTM

Figure 10.5.2-5 shows an example signalling flow for the inter-SN SCG LTM initiated by the SN:

Editor’s Note: RAN2 assumes that the figure should be anchored together with RAN3. The final figure is up to the progresses made in RAN3.

1.	The source SN initiates the inter-SN SCG LTM procedure by sending the SN Change Required with a LTM Request Indication to the MN providing a list of candidate PSCell(s) for inter-SN SCG LTM preparation. The message may include an SCG reference configuration. Source SN may additionally send measurement results of candidate PSCells and the upper limit for the number of PSCells that can be prepared by each candidate SN to the MN. Source SN may send L1 RS configuration request and/or early sync information request for suggested candidate PSCell(s). The source SN may send the configuration ID mapping list for the suggested candidate PSCell(s) to the MN for subsequent inter-SN SCG LTM.

Editor’s Note: RAN2 assumes that how to indicate the list of candidate PSCells from source SN to MN is up to RAN3. RAN2 assumes that source SN may additionally the upper limit for the number of PSCells that can be prepared by each candidate SN to the MN.

2.	The MN requests each candidate SN to allocate resources for the UE by means of the SN Addition procedure. The MN may also provide a list of KSN and associated sk-Counter valuesfor each candidate SN, and forward the received measurement results to each candidate SN(s). The MN may transfer L1 RS configuration request and/or early sync information for suggested candidate PSCell(s) to SN. The MN also provides upper limit for the number of PSCells to each candidate SN. The MN may select one of the candidate SN(s) and request providing the SCG reference configuration as part of the SN Addition procedure. Once obtained, the MN provides the reference configuration to other candidate SN(s).

NOTE Y:	The subsequent SCG LTM configurations can be included within an MN or an SN RRC message. For intra-SN SCG LTM in MN RRC message case, the source SN and the target SN shown in Figure 10.5.2-5 are the same node, and step 2 can be skipped.

3.	The candidate SN(s) determines the LTM candidate cells based on the measurement results (if provided) and the upper limit for the number of PSCells that can be prepared by each candidate SN. The candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement or CSI acquisition, early UL/DL sync configuration, by sending SN Addition Request Acknowledge message to the MN.

4.	For SN terminated bearers using MCG resources, the MN provides Xn-U DL TNL address information in the Xn-U Address Indication message to the candidate SN(s).

5.	The MN sends the received L1 RS configuration, early UL/DL sync configuration of candidate cells to the source SN via SN Modification Request message.

6.	The source SN generates the common CSI resource configuration for L1 measurement or CSI acquisition on candidate PSCells. The source SN sends the generated common CSI resource configuration and the updated source SCG configuration to the MN via SN modification request acknowledge message. The source SN sends the configuration ID mapping list for the accepted candidate PSCell(s) to the MN for subsequent inter-SN SCG LTM.

7.	The MN transfers, during the LTM preparation phase, configuration ID mapping list, the common CSI resource configuration and the collected information of candidate cells to each candidate SN via SN Modification Request message.

8.	The candidate SN(s) responds with the updated candidate SCG configuration to the MN via SN Modification Request Acknowledge message. The candidate SN(s) may also provide the CSI report configuration.

9.	The MN generates the reference configuration for SN initiated inter-CU SCG LTM, which may include the MCG part of the reference configuration generated by the MN (if any), together with the SCG part of the reference configuration generated by the source or candidate SN. The MN sends an RRCReconfiguration message to the UE including the inter-SN SCG LTM configuration, i.e. a list of RRCReconfiguration* messages, in which each RRCReconfiguration* message contains the SCG configuration in the RRCReconfiguration** message received from the candidate SN and the source SN and possibly an MCG configuration. Besides, the RRCReconfiguration message can also include an updated MCG configuration, as well as the NR RRCReconfiguration*** message generated by the source SN, e.g., to configure the required measurements.

10.	The UE applies the RRCReconfiguration message received in step 9, stores the inter-SN SCG LTM configuration and replies to the MN with an RRCReconfigurationComplete message, including an SN RRCReconfigurationComplete*** message to the source SN.

11.	The MN informs the source SN with the SN RRCReconfigurationComplete*** message via SN Change Confirm message, to indicate that SCG LTM is prepared.

12.	 In case of SN terminated bearers, early data forwarding may take place. For the early data forwarding of SN terminated bearers, the MN forwards the PDCP SDU to the candidate SN(s). For the early transmission of MN terminated split/SCG bearers, the MN forwards the PDCP PDU to the candidate SN(s).

13.	The UE may perform DL synchronization with LTM candidate cell(s) before receiving the cell switch command, as specified in clause 9.2.3.5.2 in TS 38.300 [3].

14a.The UE may perform UL synchronization with LTM candidate cell(s) before receiving the cell switch command, as specified in clause 9.2.3.5.2 in TS 38.300 [3].

14b. The candidate SN(s) transfers the calculated TA value of candidate cell(s) to the source SN via the MN via the TA Information Transfer Message.

Editor’s Note: Details for TA information transfer procedure is up to RAN3 discussion.

15.	The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source SN, if the L1 measurement configuration in RRCReconfiguration is received in step 9. The UE can also perform L3 measurement reporting to the source SN, including beam level measurement results on cell(s) which are configured as LTM candidate cell(s) according to the received network configuration.

16.	The source SN decides to execute cell switch to a target PSCell.

17.	The source SN transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID and other related information for the target PSCell, as specified in clause 9.2.3.5.2 in TS 38.300 [3]. The UE applies the candidate configuration indicated by the target configuration ID.

18/19.	The source SN Cell Switch Notification Message by Cell Switch Notification procedure to the target SN via the MN, to indicate the triggering of LTM to the UE, including the target PScell ID and the TCI state ID(s)..

20.	The UE performs the random access procedure towards the target PSCell, if the UE does not have valid TA of the target PSCell.

21. The UE sends an MN RRCReconfigurationComplete* message to the MN, which includes an SN RRCReconfigurationComplete** message to the target SN.

22.	The MN informs the target SN that the UE has completed the reconfiguration procedure successfully via SN Reconfiguration Complete message, including the SN RRCReconfigurationComplete** message.

24-27.	The MN triggers the MN initiated SN Modification procedure to inform the last serving SN that UE has accessed to the target SN, and the last serving SN to switch to the prepared state, and if applicable, to allow provisioning of new data forwarding addresses based on the data forwarding proposals of the MN and the selected candidate SN. If applicable, the source SN to start normal data forwarding, timing is due to implementation.

28.	If applicable, a PDU Session path update procedure is triggered by the MN.

23~29. Editor’s Note: Details for related procedures and descriptions are up to RAN3 discussion.

NOTE X:	The steps 12-29 can be performed multiple times for subsequent SCG LTM cell switch execution using the SCG LTM candidate configuration(s) provided in step 9.

Editor’s Note: In the above descriptions and signalling flow for the overall procedure of SN-initiated inter-SN SCG LTM, the detail procedures and messages between MN, source or candidate SN(s) are up to RAN3 discussion.

10.7.2	MR-DC with 5GC

Un-related part omitted

Inter-MN MCG LTM with SN

Figure 10.7.2-2: Inter-MN MCG LTM with Secondary Node

Figure 10.7.2-2 shows an example signaling flow for Inter-MN MCG LTM with Secondary Node.

Editor’s Note: RAN2 assumes that the figure and procedure descriptions should be anchored together with RAN3. The final figure and procedure descriptions are up to the progresses made in RAN3.

NOTE 1:	For an Inter-MN MCG LTM without SN change, the source SN and the target SN shown in Figure 10.7.2-2 are the same node.

NOTE 2:	For an Inter-MN MCG LTM with SN addition, the source SN and steps involving the source SN in Figure 10.7.2-2 are ignored.

NOTE 3:	Figure 10.7.2-2 is not applied for an Inter-MN MCG LTM with SN changed to another SN, which is not supported.

1.	The source MN starts LTM preparation by initiating the Xn Handover Preparation procedure including MCG configuration and, if the UE is configured with an SCG, SCG configuration. The source MN includes the source SN UE XnAP ID, SN ID and the UE context in the source SN in the Handover Request message. The source MN may include the common CSI-RS resource configuration.

2.	If the candidate MN decides to keep the UE context in the SN, the candidate MN sends the SN Addition Request message to the SN including the SN UE XnAP ID as a reference to the UE context in the SN that was established by the source MN. If the candidate MN decides to change the SN allowing delta configuration, the candidate MN sends the SN Addition Request message to the candidate SN including the UE context in SCG part of the reference configuration. Otherwise, the candidate MN may send the SN Addition Request message to the candidate SN including neither the SN UE XnAP ID nor the UE context in the source SN that was established by the source MN. Within the SN Addition Request message, the candidate MN also includes the LTM related information, i.e., the source MN ID and the MN UE XnAP ID in the source MN, in order to indicate that the SN Addition Preparation procedure is triggered in relation to an LTM and to enable the SN to identify requests related to the same UE.

3.	The (candidate) SN replies with the SN Addition Request Acknowledge message. The (candidate) SN may include the indication of the full or delta RRC configuration.

3a.	For the SN terminated bearers using MCG resources, the candidate MN provides Xn-U DL TNL address information in the Xn-U Address Indication message.

Handover Request Acknowledge message the MN RRC reconfiguration message to be sent to the UE in order to perform LTM, and may also provide forwarding addresses to the source MN. The candidate MN indicates to the source MN that the UE context in the SN is kept if the candidate MN and the SN decided to keep the UE context in the SN in step 2 and step 3. The candidate MN(s) may also provide the CSI-RS report configuration.

4a.	The source MN sends the Xn-U Address Indication message to the (source) SN. This Xn-U Address Indication message notifies LTM to the (source) SN, which may decide to perform, if applicable, early data forwarding for SN-terminated bearers, together with the sending of an Early Status Transfer message to the source MN.

5.	The source MN sends an LTM Configuration Update message to the candidate MN(s) to update the LTM configurations of candidate cell(s). The source MN may include the common CSI-RS resource configuration.

6.	The candidate MN(s) sends the LTM Configuration Update Acknowledge message to the source MN. The candidate MN(s) may also provide the CSI-RS report configuration.

7.	The source MN sends an RRC reconfiguration message to the UE, including the LTM candidate configuration, i.e. a list of RRCReconfiguration* messages, in which each RRCReconfiguration* message contains an MCG configuration and possibly an SCG configuration in the RRCReconfiguration** message received from the candidate SN in step 3.

8.	The UE applies the RRC reconfiguration message received in step 7, stores the LTM candidate configuration and replies to the MN with an RRCReconfigurationComplete message.

9.	The UE may perform DL synchronization with LTM candidate cell(s) before receiving the cell switch command, as specified in clause 9.2.3.5.2 in TS 38.300 [3].

10a. The UE may perform UL synchronization with LTM candidate cell(s) before receiving the cell switch command, as specified in clause 9.2.3.5.2 in TS 38.300 [3].

10b. The candidate MN(s) sends the TA Information Transfer message to the source MN if early TA acquisition is performed to the candidate MN(s).

11.	The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source MN, if the L1 measurement configuration in RRCReconfiguration is received in step 7. The UE can also perform L3 measurement reporting to the source MN, including beam level measurement results on cell(s) which are configured as LTM candidate cell(s) according to the received network configuration.

12.	The source MN decides to execute cell switch to a target cell.

13.	The source MN transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID and other related information for the target cell, as specified in clause 9.2.3.5.2 in TS 38.300 [3]. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

14.	The source MN sends the Cell Switch Notification message to the target MN to indicate the initiation of Cell Switch command to the UE.

15.	The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321[6].

16. The UE sends an MN RRCReconfigurationComplete* message to the target MN, which may include an SN RRCReconfigurationComplete** message to the target SN.

17.	If configured with bearers requiring SCG radio resources, the UE synchronizes to the (target) SN.

NOTE x:	The order the UE performs Random Access towards the MN (step 15) and performs the Random Access procedure towards the (target) SN (step 17) is not defined.

18.	If the RRC connection reconfiguration procedure was successful, the target MN informs the (target) SN via SN Reconfiguration Complete message.

19.	The target MN sends the Handover Success message to the source MN to inform that the UE has successfully accessed the target cell.

20a/b.	The source MN may sends SN Release Request message to the (source) SN including a Cause indicating MCG mobility if no candidate cells exist in source MN. The source MN indicates to the (source) SN that the UE context in SN is kept, if it receives the indication from the target MN. The (source) SN acknowledges the release request.

The source MN sends XN-U Address Indication message to the (source) SN to transfer data forwarding information.

21a/b. Editor’s Note: FFS up to RAN3. The (source) SN sends the Secondary RAT Data Usage Report message to the source MN and includes the data volumes delivered to and received from the UE over the NR/E-UTRA radio as described in clause 10.11.2. The source MN sends the Secondary RAT Data Usage Report message to AMF to provide information on the used NR/E-UTRA resource.

22a/b. Editor’s Note: FFS up to RAN3. For bearers using RLC AM, the source MN sends the SN Status Transfer message to the target MN, including, if needed, SN Status received from the source SN.

23.	For bearers using RLC AM, the source MN sends the SN Status Transfer message to the target MN, including, if needed, SN Status received from the source SN.  	If applicable, data forwarding takes place from the source side SN to source MN, and to the target MN..

24	If applicable, data forwarding takes place from the source side (i.e. source MN or source SN). If the SN is kept, data forwarding may be omitted for the SN terminated bearers or QoS flows kept in the SN.

24-27.	The target MN initiates the Path Switch procedure. If the target MN includes multiple DL TEIDs for one PDU session in the Path Switch Request message, multiple UL TEID of the UPF for the PDU session should be included in the Path Switch Ack message in case there is TEID update in UPF.

28. The target MN (i.e. the new source MN) sends the LTM Configuration Update message to the other candidate MNs (may also include the original source MN) to inform that the UE has successfully accessed, and update the security information.

29. The candidate MN(s) responds the LTM Configuration Update Acknowledge message to the source MN.

30. The target MN may send the UE Context Release message to inform the source MN to release radio and C-plane related resources associated to the UE context if no LTM candidate cell(s) exist in the source MN. Any ongoing data forwarding may continue.

31.	Upon reception of the UE Context Release message from source MN, the (source) SN releases C-plane related resources associated to the UE context towards the source MN. Any ongoing data forwarding may continue. The SN shall not release the UE context associated with the target MN if the UE context kept indication was included in the SN Release Request message in step 20a.

aa3. Other candidate MN sends SN Modification Request message to other candidate SN for update the security information,

aa4. Other candidate SN sends SN Modification Response message to candidate MN,

Editor’s Note: In the above descriptions and signalling flow for the overall procedure of inter-MN MCG LTM with SN, the detail procedures and messages between MN, source or candidate SN(s) are up to RAN3 discussion.