3GPP TSG-RAN WG3 Meeting #129	R3-255608

Bangalore, India, 25 – 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1303 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Define AIoT‑FunctionIdentifier and AIoT‑CorrelationIdentifier IEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Jio Platforms Limited |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ambient_IoT_solutions-Core |  |  |  |  |  | Date: |  |  | 2025-08-04 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Ambient‑IoT work item introduces new NGAP procedures for inventory and command operations between the NG‑RAN and the Ambient‑IoT Core Network (AIOTF). Stage‑2 (TS 38.305/23.502) defines two identifiers used in these procedures: - AIoT Function Identifier – uniquely identifies the Ambient‑IoT Function (AIOTF) to which A‑IoT traffic shall be routed; and - AIoT Correlation Identifier – uniquely correlates all NGAP messages that belong to one Ambient‑IoT session (inventory or command). The Release‑19 baseline TS 38.413 contains FFS notes for these identifiers (see RAN3#128 work plan), pending definition of their encoding. Without a defined format, nodes cannot correlate sessions or route messages as required by Stage‑2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce two NGAP IEs to carry AIOTF identity and per‑session correlation using 128‑bit UUID‐sized encodings. No new procedures; references added where used by A‑IoT messages. NGAP remains NG‑RAN↔AMF; The identifiers are provided to facilitate correlation/routing across 5GS functions as specified by SA2; NGAP remains strictly between NG-RAN and AMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Close FFS on identifier encodings for A‑IoT messaging so nodes can correlate sessions and route via AMF toward AIOTF per Stage‑2. (A‑IoT BL TPs reference these IEs repeatedly. ) |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 9.3	Information Element Definitions

### 9.3.1	Radio Network Layer Related IEs

-----Skip to New IEs-----

#### 9.3.1.x AIoT Function ID

The AIoT-FunctionIdentifier IE uniquely identifies the Ambient IoT Function instance used for a procedure; its presence is defined per message.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality |
| --- | --- | --- | --- | --- | --- |
| AIoT Function ID | C (per message) |  | OCTET STRING (SIZE(16)) | 128-bit opaque identifier of the AIoT Function used for correlation/routing as defined by SA2 Stage-2. If the AIoT Function is realized as a 3GPP NF, this value may be set to the NF Instance ID (UUID) per TS 29.571. | reject |

#### 9.3.1.y AIoT Correlation ID

The AIoT-CorrelationIdentifier IE uniquely correlates all NGAP messages that belong to one A-IoT inventory/command session.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality |
| --- | --- | --- | --- | --- | --- |
| AIoT Correlation ID | C (per message) |  | OCTET STRING (SIZE(16)) | 128-bit opaque correlation value for an Ambient IoT session/context. The mapping of this value to 5GC SBI correlation mechanisms is outside the scope of this specification. | reject |

Informative note: NGAP peers are NG‑RAN and AMF. These identifiers are carried end‑to‑end within NGAP; any service‑based routing in the 5GC is defined by SA2 and is not specified in NGAP.

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

## 9.4	Message and Information Element Abstract Syntax (with ASN.1)

-- ProtocolIE-ID values to be assigned by the editors (e.g. id-AIoT-Function-ID, id-AIoT-Correlation-ID)

-- Update the global enumeration of ProtocolIE-ID as follows:
-- { ID id-AIoT-Function-ID     CRITICALITY reject    TYPE AIoT-FunctionIdentifier     PRESENCE conditional },
-- { ID id-AIoT-Correlation-ID  CRITICALITY reject    TYPE AIoT-CorrelationIdentifier  PRESENCE mandatory    },

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>