3GPP TSG-RAN WG3 Meeting #129	R3-255620

Bengaluru, India, 25 – 29 August 2025

Agenda item:	10.2

Source:	Samsung

Title:	(TP for SON BLCR for TS38.401) MRO for LTM

Document for:	Discussion & Decision

- Introduction

The contribution discussed the leftover issues on MRO for LTM.

- Discussion

- 2.1 Stage 2 (TS38.401)

For the target cell selection, coordination is done between gNB CU and source gNB DU: gNB CU to select the candidate target cells and gNB DU to decide the target cell for handover within the scope provided by gNB CU. Thus, the wrong cell selection can be divided the following cases:

- Wrong LTM prepared cell selection: gNB CU provides an improper candidate target cell list. So the source gNB DU cannot find a good choice within the provided scope.

- Wrong target cell selection for cell switch: gNB CU gives the suitable candidate target cell list, while the mistake happens in source gNB DU, which means source gNB DU should choose another cell in the candidate list to do the handover.

If case of Wrong LTM prepared cell selection, the gNB CU should perform optimization. In case of Wrong target cell selection for cell switch, the source gNB DU should perform optimization.

However the two cases are not included in the BLCR for TS38.401. The TP to 38.401 is provided in Annex A.

Proposal 1: Agree the TP on MRO for LTM to TS38.401 in Annex A.

- 2.2 LTM failure due to outdated TA

Progress in RAN3#127bis:

MRO will cover the scenario that RACH-less LTM fails including outdate TA and UE performs RACH based LTM. RAN3 will not consider the case of LTM cell switch failure due to outdated TA calculated by UE. For PDCCH order triggered early TA acquisition, RAN3 consider the case of RACH-less LTM fails due to outdate TA and UE performs RACH-based LTM failure recovery or RRC re-establishment to the same cell. Whether UE based solution or network based solution is used for LTM failure due to outdated TA? Whether LTM failure due to outdated TA is defined as a new failure case or it can covered by existing failure case?

Progress in RAN3#128:

RAN3 supports network-based solution for out dated TA. Reusing the same signaling flow as case1 and case2.

One issue of the network based solution is the signalling efficiency. RACH-less LTM failure due to outdate TA may happen in two scenarios:

Scenario 1: outdate TA is sent to the UE by the source gNB-DU via Cell Switch Command.

Scenario 2: outdate TA is calculated by UE.

In Scenario 2, the problem is not brought by the RAN. That’s why RAN3 agreed that “RAN3 will not consider the case of LTM cell switch failure due to outdated TA calculated by UE.”.

The target DU has no idea of whether source DU sent the TA in cell switch command or the UE calculated the TA leads to the failure. In Scenario 2, it is the UE which bring the problem. If the DU makes some optimization, it may bring negative impact on the network. Another issue is that this bring unnecessary signaling between target gNB-DU->CU->the source gNB-DU if the target DU always send the TA value of successful access after failure to the source DU via gNB CU.

In order to rule out scenario 2, the target DU needs to know whether the source DU has sent TA value in LTM cell switch command to the UE.

To solve this issue, the source gNB-DU sends such information (TA value sent in LTM cell switch command) to the gNB-CU and the gNB-CU forward the information to the target DU. If the information is received and the UE access to the target cell successful via RACH procedure, the target DU sends the TA value of successful access to source DU via CU. Otherwise, the target DU doesn’t need to trigger the procedure.

The source gNB-DU could send the indication of TA value sent in LTM cell switch command to the gNB-CU using DU-CU Cell Switch Notification message. And the gNB-CU sends the indication to the target gNB-DU via CU-DU Cell Switch Notification message. The message is needed anyway. There is no impact on the overall signalling flow. The only change is to add an Indication in DU-CU Cell Switch Notification and CU-DU Cell Switch Notification messages.

Proposal 2: Add “Indication of TA value sent in LTM cell switch command” to DU-CU Cell Switch Notification and CU-DU Cell Switch Notification messages

A TP for TS38.473 is provided in [1].

Proposal 3: Agree the TP on MRO for LTM to TS38.473 in [1].

- 2.3 Solve the FFSs in the BLCR for TS38.473

Some FFSs are left in the BLCR.

Name of TA Information:

| TA Information (name is FFS) | O |  |  |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >TA Value | M |  | INTEGER (0..4095) | Indicates the TA value used at successful Random Access, as defined in TS 38.213 [31]. | - |  |
| >gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | - |  |

For future extension, it is better to keep the name of the TA Information in general. In the semantic description of the TA value, the exact meaning can be described. The current description in the semantic description is clear enough. So the proposal is to remove the FFS on the name of TA information.

Proposal 4: Keep the name of TA information and remove the FFS.

Target Beam Information

| LTM Cell Switch Failure due to Wrong Beam Information | O |  |  |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >Target Beam Information | M |  | FFS | FFS if it corresponds to the target beam index or TCI state ID. | - |  |
| >gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | - |  |

For the target Beam information, TCI State ID is used in RAN2 specification (TS38.331, TS38.321). So TCI state ID should be used.

Proposal 5: Target Beam Information corresponds to TCI state ID. This is included in the provided TP for TS38.473 in [1].

Translation of the UE F1AP IDs:

Interaction with the Access and Mobility Indication procedure:

If the Beam Failure Recovery Information IE or the LTM Cell Switch Failure due to Wrong Beam Information IE or the TA Information IE (name is FFS) is included in the DU-CU ACCESS AND MOBILITY INDICATION message, the gNB-CU shall, if supported, forward this information to the source gNB-DU via the ACCESS AND MOBILITY INDICATION message including the Beam Failure Recovery Information IE, the LTM Cell Switch Failure due to Wrong Beam Information IE and the TA Information IE (name is FFS) respectively.

Editor’s note: FFS how to capture the translation of the UE F1AP IDs.

It is straight forward that the gNB-DU UE F1AP ID in the source DU should be included in the Access and Mobility Indication procedure. There is no confusion even if there is no description. If people want something in the specification, one sentence can be added as one example in the TP for TS38.473.

Proposal 6: It is straight forward that the gNB-DU UE F1AP ID in the source DU should be included in the Access and Mobility Indication procedure. It’s also fine to add a sentence on this as in the TP for TS38.473.

Structure of UE F1AP IDs:

It is ok to keep the structure as it is. It is also fine to restructure and use one APID and optional other information e.g. beam/TA info.

- 2.4 Additional information from target DU to CU and from CU to source DU for Case 1 & Case 2

The following information are in the table:

- Failure type (for case 1, case 2) in an implicit way or explicit way

- beam failure indicator in an implicit way or explicit way

For the failure type on case 1 or case 2, the CU or the source DU can know it is Case 1 if Beam Failure Recovery Information is received and it is Case 2 if LTM Cell Switch Failure due to Wrong Beam Information is received.

Observation 1:  The receiving node (CU or source DU) can know the failure type implicitly based on the received information.

If Beam Failure Recovery Information or LTM Cell Switch Failure due to Wrong Beam Information is received, the receiving node will know there is beam failure. No new information is needed.

Observation 2:  The receiving node (CU or source DU) can know the beam failure implicitly based on the received information.

- 2.5 UHI and ping-pong issue

RAN3#126 meeting agreed that:

CU can adjust the candidate Cell list for LTM based on UHI.

DU should be able to resolve LTM ping-pong based on information over F1.

During RAN3#127bis, the following agreement was taken:

For intra-CU case, CU detects the potential LTM ping-pong issue based on local information, DU decides and solves the LTM ping-pong issue.

During RAN3#128, the following agreement was achieved:

CU informs relevant information to the target DU via CU-DU Cell Switch Notification.

The following solutions need to be down selected:

1a: CU provide full UHI list with L1/L3 type to the DU (maximum item number is 16).

1b: CU provide full UHI list (except PScell) with L1/L3 type to the DU (maximum item number is 16).

2a: CU provide last consecutive L1 entries to the DU.

2b: CU provide FFS number of last consecutive L1 entries to the DU.

No stage2, work in stage3 with 1a.

If the above 1a information is included in F1AP message from CU to DU, the DU may consider the potential mobility issue detected including ping-pong.

FFS on the extra indication for ping-pong case.

DU is handling time critical related work. L3 related handover has no relation with the DU. It is not needed to send so many information to the DU. For example, if there are 10 entries in UHI and large portion of the entries are L3 related HO, then why to send those L3 HO information to the DU and add an additional type?

For example,

Cell1 to Cell 8 and L3 HO.

Cell 9-Cell 10 are L1 HO type

There is ping-pong between Cell 9 and Cell 10

Then send Cell9-Cell10 to the DU is enough. Based on the dual time in Cell9 and Cell10, the DU can make proper optimization. Sending Cell1 to Cell 8 to DU just bring additional complexity to the DU.

Proposal 7: Rule out option 1a and option 1b on UHI and ping-pong issue.

Regarding option 2a or option 2b, if the CU observe there is ping-pong for the last consecutive L1 entries, the CU can send last consecutive L1 entries to the DU. If the CU observed there is no ping-pong for the a consecutive L1 entries, the CU doesn’t need to send information to the DU. Maybe there is no ping-pong for non-consecutive L1 HO.

Proposal 8: Agee option 2a as way forward for UHI and ping-pong issue. The TP is included in [1].

Conclusion

This contribution discussed the leftover issues on MRO for LTM. We have the following observations and Proposals. It is proposed to agree the proposals.

Proposal 1: Agree the TP on MRO for LTM to TS38.401 in Annex A.

Proposal 2: Add “Indication of TA value sent in LTM cell switch command” to DU-CU Cell Switch Notification and CU-DU Cell Switch Notification messages

Proposal 3: Agree the TP on MRO for LTM to TS38.473 in [1].

Proposal 4: Keep the name of TA information and remove the FFS.

Proposal 5: Target Beam Information corresponds to TCI state ID. This is included in the provided TP for TS38.473 in [1].

Proposal 6: It is straight forward that the gNB-DU UE F1AP ID in the source DU should be included in the Access and Mobility Indication procedure. It’s also fine to add a sentence on this as in the TP for TS38.473.

Observation 1:  The receiving node (CU or source DU) can know the failure type implicitly based on the received information.

Observation 2:  The receiving node (CU or source DU) can know the beam failure implicitly based on the received information.

Proposal 7: Rule out option 1a and option 1b on UHI and ping-pong issue.

Proposal 8: Agee option 2a as way forward for UHI and ping-pong issue. The TP is included in [1].

- Annex A: TP to 38.401 on MRO for LTM

- 7.X	MRO support for LTM

For Too Late LTM cell switch and LTM cell switch to Wrong Cell, there are two sub-cases:

- if the first re-establishment attempt cell/ the cell UE attempts to re-connect/the cell UE attempts LTM recovery is one of the candidate cell provided by the gNB CU, it is the Wrong target cell selection for cell switch at the source gNB DU;

- else, it is Wrong LTM cell preparation at the gNB CU.

The gNB-CU receives a RLF report associated to an LTM mobility event from the UE or via the Failure indication message over Xn. The gNB-CU performs initial analysis. In case of failure due to inappropriate cell switch triggering or Wrong target cell selection for cell switch the gNB-CU may forward the RLF report to the last serving gNB-DU in case of too late LTM cell switch, or to the source gNB-DU in case of too early LTM cell switch or LTM cell switch to wrong cell.

The target gNB-DU identifies that a Beam Failure Recovery (BFR) has happened in the UE shortly after a successful LTM cell switch by detecting a time gap between the successful LTM cell switch and the BFR in the same cell is smaller than the configured threshold (e.g. Tstore_UE_cntxt).The target gNB-DU performs initial analysis and may send the recovery beam information to the source gNB-DU via the gNB-CU.

The target gNB-DU identifies that the UE successfully performs a RACH-based access while reconnecting/re-establishing/recovering to the same target cell but to a beam different from the beam used during the failed LTM cell switch execution. The target gNB-DU may send the reconnected/re-established/recovery beam information to the source gNB-DU via the gNB-CU. The source gNB-DU performs root cause analysis to detect RACH-less LTM failure caused by wrong beam selection.

The target gNB-DU identifies that the UE successfully performed a RACH-based access while reconnecting/re-establishing/recovering to the same target cell and same target beam but with TA value different from the one which was included in CU-DU CELL SWITCH NOTIFICATION message received from gNB-CU at LTM Cell Switch execution. The target gNB-DU sends the TA value used at successful RACH-based access to the source gNB-DU via the gNB-CU.

References

R2-255621, (TP for SON BL CR for TS38.473) MRO for LTM, Samsung