3GPP TSG-RAN WG3 Meeting #129	R3-255625

Bengaluru, India, 25th ~29th Aug, 2025

Title:	Fetching reference configuration from candidate gNB in inter-CU LTM

Agenda Item:	13.2

Source:	Huawei, Google, Nokia, Jio Platforms, CATT, CMCC, NTT Docomo, Lenovo, China Telecom, Samsung

Document for:	Discussion

- Introduction

At RAN3 #127 meeting, the following progress was made related to reference configuration generation.

The source gNB can generate reference configuration and provide a reference configuration for LTM in a Handover Request and LTM configuration update message.

The candidate gNB indicates whether a LTM candidate configuration is a complete candidate configuration in the Handover Request Acknowledge and LTM configuration update acknowledge message.

FFS on whether the source gNB can request a candidate gNB to provide a reference configuration.

The open issue was discussed for two meetings, but are still not converged. The three approaches are listed here for reference.

- Approach 1: Similar as F1AP, the source gNB can request a candidate-gNB to provide a reference configuration.

- Approach 2: For inter-CU LTM, only the source gNB can provide the reference configuration.

- Approach 3: If the source CU does not provide the reference configuration, it is up to the candidate DU to provide the reference configuration to the source CU and the source DU can decide which one to adopt as a reference configuration.

The proponents of this contribution propose RAN3 to conclude this issue with approach 1 at the last meeting of Rel-19.

- Discussion

During the last few meetings, the majority companies think that it is beneficial to allow the source gNB to request a candidate gNB to provide a reference configuration. Because the RRC signalling overhead can be reduced in some cases. For example, one of the candidate gNB has the most number of candidate cells, or the candidate gNBs coming from a common vendor but different from the source gNB. The commonalities of the cell configurations for those candidate cells are maximized in this two cases. And also, if there is no candidate cell in the source gNB, it seems not reasonable for the source gNB to generate the reference configuration.

While, the opponents think that it will incur much more complexity and signaling overhead. And it may even don’t work in multi-vendor scenario.

In general, the above reasons for opposition are not technically correct. First of all, allowing the source gNB to fetch a reference configuration will not bring any additional signaling overhead and complexity. As the source gNB may utilize the handover request procedure to fetch a reference configuration and send it to the candidate gNBs in the LTM configuration update request message to get the delta configuration. And one should know that the LTM configuration procedure is mandatory for subsequent LTM. According to the latest LTM BLCR for TS 38.423 in [1], the Reference Configuration IE is already included into the LTM configuration update procedure.

Furthermore, allowing request of reference configuration from a candidate gNB does not mean that all candidate gNBs need to be queried. One query is sufficient usually in which case no extra sinanaling overhead is introduced.

For the interoperability issue in multi-vendor scenario, fetching reference configuration from a candidate gNB does not worsen the interoperability issue if it indeed exists. Even the source gNB itself generating the reference configuration still may have the interoperability issue if the candidate gNB is from a different vendor.

Observation 1: Allowing the source gNB to request a candidate gNB to provide a reference configuration neither incurs extra signaling overhead and complexity, nor worsens the specious interoperability issue.

On the other hand, as captured in TS 38.331, it can be learned that when the LTM cell switch is triggered, the UE first and applies either the ltm-CandidateConfig (if the ltm-ConfigComplete is included in the ltm-Candidate) or the ltm-CandidateConfig on top of the ltm-ReferenceConfiguration. This is actually similar to the actions of when full configuration is performed. Therefore, there may not be much difference if the reference configuration comes from the current serving cell or one of the candidate gNB(s) which is likely to become a (temporary) serving gNB in the subsequent mobility. Not allowing the reference configuration to be acquired from the candidate gNBs only creates the inconsistency of the Inter-CU MCG LTM scenario and the other scenarios (i.e., Intra-CU LTM or Inter-CU SCG LTM).

Fig. Illustration of how delta configuration works in mobility scenarios

Observation 2: There is no clear benefit of mandating the reference configuration to be from the current serving cell configuration when subsequent mobility is allowed.

In the end, approach 3 allowing the candidate gNBs to produce the reference configuration autonomously seems violating the single reference configuration RAN2 agreement, therefore should not be pursued.

- RAN3 to allow the source gNB to request a candidate gNB to provide a reference configuration in Rel-19.

3	Conclusion

This document proposes:

Observation 1: Allowing the source gNB to request a candidate gNB to provide a reference configuration neither incurs extra signaling overhead and complexity, nor worsens the specious interoperability issue.

Observation 2: As specified in RRC protocol, the reference configuration/delta configuration mechanism during LTM cell switch works similarly as the full configuration/delta configuration during legacy Xn handover. Which proves no reason to disallow the candidate gNB to provide a reference configuration in inter-CU LTM.

Proposal 1:RAN3 to allow the source gNB to request a canddiate gNB to provide a reference configuration in Rel-19.

The corresponding TPs are provided in the [1] and [2].

Reference

[1] R3-255626	(TP for LTM BLCR for TS38.423): Reference configuration in inter-CU LTM

[2] R3-255627	(TP for LTM BLCR for TS38.473): Reference configuration in inter-CU LTM