3GPP TSG-RAN WG3 #129                                                 R3-255641

Bengaluru, IN, 25th – 29th Aug, 2025

Agenda Item:	10.2

Source:	CATT

Title:	(TP for 38.473 and 38.401) MRO for R18 LTM

Document for:	other

Introduction

In last RAN3 meeting, the following agreements and FFS are captured:

BFR shortly after successful LTM cell switch to the wrong beam(Case 1):

Use DU-CU ACCESS AND MOBILITY INDICATION message for target DU send recovery beam information to the CU. Only one recovery beam information included.

Additional Information with recovery beam information from target DU to CU:

- CU UE F1AP ID

- Failure type (for case 1, case 2) in an implicit way or explicit way

- beam failure indicator in an implicit way or explicit way

Use ACCESS AND MOBILITY INDICATION message for CU forward recovery beam information to the source DU. Only one recovery beam information included.

Additional Information with recovery beam information from target CU to DU:

- Source DU UE F1AP ID

- Failure type (for case 1, case 2) in an implicit way or explicit way

- beam failure indicator in an implicit way or explicit way

No new failure type for beam failure recovery is needed for TS38.300.

The Timer configured in DU (e.g., Tstore_UE_cntxt) used for the DU to detect BFR shortly after successful LTM cell switch to the wrong beam is up to implementation.

RACHless LTM cell switch failure due to wrong beam (Case 2)

Network-based solution for the LTM cell switch failure due to wrong beam using the same signaling flow as for Case1.

In case that the source DU selects a wrong beam among candidate beam list, the source DU is responsible for MRO optimization.

In case that the target DU provides a wrong candidate beam list, the target DU is responsible for MRO optimization.

Target DU needs to send the reconnect/re-established/recovery beam information to CU and CU forwards it to source DU.

CU does not need to send the old beam information to source DU.

Reply LS to RAN2 on the network based solution for case1 and case2.

Work on stage3 semantic description for C-RNTI IE in ACCESS AND MOBILITY INDICATION message.

New BL CR for TS38.420 Samsung

Reusing the same signaling flow as case1 and case2.

TA issues

RAN3 supports network-based solution for out dated TA.

TA acquisition Type signalling is not needed based on the network based solution of out dated TA. Work on the LS to RAN2.

UHI and ping-pong issue

CU informs relevant information to the target DU via CU-DU Cell Switch Notification.

Down selection of following solutions:

1a: CU provide full UHI list with L1/L3 type to the DU (maximum item number is 16).

1b: CU provide full UHI list (except PScell) with L1/L3 type to the DU (maximum item number is 16).

2a: CU provide last consecutive L1 entries to the DU.

2b: CU provide FFS number of last consecutive L1 entries to the DU.

Based on above agreements, here we provide some analysis on MRO for R18 mobility mechanisms.

Discussion

  - Case 1: BFR shortly after successful LTM cell switch to the wrong beam

In last RAN3 meeting, the following agreements were captured:

Use DU-CU ACCESS AND MOBILITY INDICATION message for target DU send recovery beam information to the CU. Only one recovery beam information included.

Additional Information with recovery beam information from target DU to CU:

- CU UE F1AP ID

- Failure type (for case 1, case 2) in an implicit way or explicit way

- beam failure indicator in an implicit way or explicit way

In BFR, BeamFailureRecoveryConfig IE within BWP-UplinkDedicated IE may be configured. This IE is captured below:

#### BeamFailureRecoveryConfig

The IE BeamFailureRecoveryConfig is used to configure the UE with RACH resources and candidate beams for beam failure recovery in case of beam failure detection. See also TS 38.321 [3], clause 5.1.1.

BeamFailureRecoveryConfig information element

-- ASN1START

-- TAG-BEAMFAILURERECOVERYCONFIG-START

BeamFailureRecoveryConfig ::=       SEQUENCE {

rootSequenceIndex-BFR               INTEGER (0..137)                                                          OPTIONAL, -- Need M

rach-ConfigBFR                      RACH-ConfigGeneric                                                        OPTIONAL, -- Need M

rsrp-ThresholdSSB                   RSRP-Range                                                                OPTIONAL, -- Need M

candidateBeamRSList                 SEQUENCE (SIZE(1..maxNrofCandidateBeams)) OF PRACH-ResourceDedicatedBFR   OPTIONAL, -- Need M

ssb-perRACH-Occasion                ENUMERATED {oneEighth, oneFourth, oneHalf, one, two,

four, eight, sixteen}

……

}

PRACH-ResourceDedicatedBFR ::=      CHOICE {

ssb                                 BFR-SSB-Resource,

csi-RS                              BFR-CSIRS-Resource

}

BFR-SSB-Resource ::=                SEQUENCE {

ssb                                 SSB-Index,

ra-PreambleIndex                    INTEGER (0..63),

...

}

BFR-CSIRS-Resource ::=              SEQUENCE {

csi-RS                              NZP-CSI-RS-ResourceId,

ra-OccasionList                     SEQUENCE (SIZE(1..maxRA-OccasionsPerCSIRS)) OF INTEGER (0..maxRA-Occasions-1)   OPTIONAL,   -- Need R

ra-PreambleIndex                    INTEGER (0..63)                                                                 OPTIONAL,   -- Need R

...

}

One SSB-Index or one NZP-CSI-RS-ResourceId may be selected to perform BFR. For CSI-RS, since it could provide more precise direction information than SSB index, source DU could also use it to optimize TCI state ID in LTM Cell Switch Command. But for simplicity, it is not supported to transfer CSI-RS related BFR information from target DU to source DU in Rel-19.

Proposal 1: CSI-RS-ResourceId should not be included in DU-CU ACCESS AND MOBILITY INDICATION message to indicate the recovery beam information if CSI-RS is selected in beam failure recovery procedure.

For the following text for DU-CU ACCESS AND MOBILITY INDICATION message in TS38.473:

| Beam Failure Recovery Information | O |  |  |  |
| --- | --- | --- | --- | --- |
| >Recovery TCI State | M |  | OCTET STRING | Includes the TCI-StateId IE used at Beam Failure Recovery, as defined in TS 38.331 [8]. |

TCI state ID is not configured by BeamFailureRecoveryConfig IE, so, SSB index should be used instead of TCIstateID.

Proposal 2: For BFR case, the recovery beam should be SSB index rather than TCIstateID.

For SSB-Index used in BFR or RLM, it may refer to CD-SSB or NCD-SSB as the description in TS38.331.

nonCellDefiningSSB If configured, the UE operating in this BWP uses this SSB for the purposes for which it would otherwise have used the CD-SSB of the serving cell (e.g. obtaining sync, measurements, RLM, BFD, beam management). Furthermore, other parts of the BWP configuration that refer to an SSB (e.g. the "SSB" configured in the QCL-Info IE; the "ssb-Index" configured in the RadioLinkMonitoringRS; CFRA-SSB-Resource; PRACH-ResourceDedicatedBFR) refer implicitly to this NCD-SSB. The NCD-SSB has the same values for the properties (e.g., ssb-PositionsInBurst, PCI, ssb-PBCH-BlockPower) of the corresponding CD-SSB apart from the values of the properties configured in the NonCellDefiningSSB-r17 IE. In the MIB associated with this NCD-SSB, the systemFrameNumber field indicates the frame boundary and frame number of the NCD-SSB. The subCarrierSpacingCommon and dmrs-TypeA-Position field in the MIBs associated with CD-SSB and NCD-SSB in the same cell are configured with the same values, respectively.

For TCI state ID or UL TCI state ID in LTM Cell Switch Command, it is associated with LTM-TCI-Info IE as below:

LTM-TCI-Info information element

-- ASN1START

-- TAG-LTM-TCI-INFO-START

LTM-TCI-Info-r18 ::=             SEQUENCE {

ltm-DL-OrJointTCI-StateToAddModList-r18        SEQUENCE (SIZE (1..maxNrofCandidateTCI-State-r18)) OF CandidateTCI-State-r18

OPTIONAL,    -- Need N

ltm-DL-OrJointTCI-StateToReleaseList-r18       SEQUENCE (SIZE (1..maxNrofCandidateTCI-State-r18)) OF TCI-StateId

OPTIONAL,    -- Need N

ltm-UL-TCI-StateToAddModList-r18               SEQUENCE (SIZE (1..maxNrofCandidateUL-TCI-r18)) OF CandidateTCI-UL-State-r18

OPTIONAL,    -- Need N

ltm-UL-TCI-StateToReleaseList-r18              SEQUENCE (SIZE (1.. maxNrofCandidateUL-TCI-r18)) OF TCI-UL-StateId-r17

LTM-TCI-Info and ltm-SSB-Config are configured for LTM candidate as below:

LTM-Candidate information element

-- ASN1START

-- TAG-LTM-CANDIDATE-START

LTM-Candidate-r18 ::=     SEQUENCE {

ltm-CandidateId-r18                            LTM-CandidateId-r18,

ltm-CandidatePCI-r18                           PhysCellId                                            OPTIONAL,    -- Need M

ltm-SSB-Config-r18                             LTM-SSB-Config-r18                                    OPTIONAL,    -- Need M

ltm-CandidateConfig-r18                        OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Need M

ltm-ConfigComplete-r18                         ENUMERATED {true}                                     OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfig-r18                     OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfigSUL-r18                  OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-TCI-Info-r18                               LTM-TCI-Info-r18                                      OPTIONAL,    -- Need M

ltm-NoResetID-r18                              INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

ltm-UE-MeasuredTA-ID-r18                       INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

...

}

LTM-SSB-Config-r18 ::= SEQUENCE {

ssb-Frequency-r18                              ARFCN-ValueNR,

subcarrierSpacing-r18                          SubcarrierSpacing,

ssb-Periodicity-r18                            ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1} OPTIONAL,   -- Need R

ssb-PositionsInBurst-r18                       CHOICE {

shortBitmap                                    BIT STRING (SIZE (4)),

mediumBitmap                                   BIT STRING (SIZE (8)),

longBitmap                                     BIT STRING (SIZE (64))

}                                                                                                    OPTIONAL,   -- Need R

ss-PBCH-BlockPower-r18                         INTEGER (-60..50)                                     OPTIONAL,   -- Need R

...

}

Observation 1: SSB used in BFR could be configured by BeamFailureRecoveryConfig IE while TCI state is configured by LTM-Candidate IE.

For target DU provide SSB index used in BFR to source DU, there are two understandings.

Understanding 1: SSB index or SSB partition in different BWP (NCD-SSB or CD-SSB) is different. Therefore, the source node needs to know the both SSB index and SSB configuration.  The solution is that recovery DU provides both SSB index and SSB configuration used in BFR to source DU.

Understanding 2: SSB index or SSB partition in different BWP (NCD-SSB or CD-SSB) is similar. Source DU is able to identify SSB index related to which “space”, therefore, no SSB configuration is needed, only send SSB index to source DU.

Proposal 3: RAN3 to discuss whether providing SSB configuration (current SSB Information IE) used in BFR to source DU in DU-CU ACCESS AND MOBILITY INDICATION message.

  - Case 2: LTM cell switch failure due to wrong beam

In last RAN3 meeting, for case 1, following FFS is captured.

Network-based solution for the LTM cell switch failure due to wrong beam using the same signaling flow as for Case1.

In case that the source DU selects a wrong beam among candidate beam list, the source DU is responsible for MRO optimization.

In case that the target DU provides a wrong candidate beam list, the target DU is responsible for MRO optimization.

Target DU needs to send the reconnect/re-established/recovery beam information to CU and CU forwards it to source DU.

CU does not need to send the old beam information to source DU.

Reply LS to RAN2 on the network based solution for case1 and case2.

Work on stage3 semantic description for C-RNTI IE in ACCESS AND MOBILITY INDICATION message.

New BL CR for TS38.420 Samsung

In last RAN3 meeting, we agree that target DU send the reconnect/re-established/recovery beam information to CU in case 2.

After LTM cell switch failure, UE performs cell selection. If LTM target cell is selected, UE may perform re-established or recovery based on LTM configuration. In case of successful recovery, target DU could send beam information to source DU, and source DU could use the beam information based on the beam configuration within LTM-Candidate information.

In case of re-established, UE may be re-establish to a same or different BWP comparing with LTM-Candidate information. If the BWP is same, target DU could use the SSB index directly. If different, besides SSB index, SSB configuration may be also provided to source DU.

After recovery/ re-established failure, UE may perform reconnect to LTM target cell. In this case, network cannot know this UE, i.e. this UE would be known as a new connected UE. Target DU should not sent DU-CU ACCESS AND MOBILITY INDICATION message to source DU.

Proposal 4: In case of successful recovery, target DU sends recovery beam information to source DU. In case of successful re-established, target DU sends SSB index to source DU and further discuss on whether also sends related SSB configuration. In case of reconnect, recovery beam information should not be sent to source DU.

  - Ping-pong issue

In last RAN3 meeting, the following agreements were captured.

CU informs relevant information to the target DU via CU-DU Cell Switch Notification.

Down selection of following solutions:

1a: CU provide full UHI list with L1/L3 type to the DU (maximum item number is 16).

1b: CU provide full UHI list (except PScell) with L1/L3 type to the DU (maximum item number is 16).

2a: CU provide last consecutive L1 entries to the DU.

2b: CU provide FFS number of last consecutive L1 entries to the DU.

For 2a/2b, L3 handover is removed from UHI. For example, UHI: (Cell A) L1 handover to (Cell B) L3 handover to (Cell C) L1 handover to (Cell D). CU does not provide above UHI to DU.

In our understandings, above UHI should also be sent to DU because (Cell B) L3 handover to (Cell C) needs optimization to trigger L1 handover instead of L3 handover if LTM is configured. So, UHI including L3 handover is also needed for DU to make optimization.

Moreover, it is not needed to decrease overload in F1. So, solution 1a/1b is preferred.

As for the F1 message to send UHI from CU to DU, ACCESS AND MOBILITY INDICATION message could be reused.

Proposal 5: It is proposed to send full UHI with L1/L3 type in ACCESS AND MOBILITY INDICATION message from CU to DU, i.e. solution 1a/1b.

  - FFS on ACCESS AND MOBILITY INDICATION message

In last RAN3 meeting, for DU-CU ACCESS AND MOBILITY INDICATION message, on UE F1AP ID, the FFS is captured as below:

Interaction with the Access and Mobility Indication procedure: If the Beam Failure Recovery Information IE or the LTM Cell Switch Failure due to Wrong Beam Information IE or the TA Information IE (name is FFS) is included in the DU-CU ACCESS AND MOBILITY INDICATION message, the gNB-CU shall, if supported, forward this information to the source gNB-DU via the ACCESS AND MOBILITY INDICATION message including the Beam Failure Recovery Information IE, the LTM Cell Switch Failure due to Wrong Beam Information IE and the TA Information IE (name is FFS) respectively. Editor’s note: FFS how to capture the translation of the UE F1AP IDs.

CU receives DU-CU ACCESS AND MOBILITY INDICATION message. In Beam Failure Recovery Information IE or the LTM Cell Switch Failure due to Wrong Beam Information IE or the TA Information IE, gNB-CU UE F1AP ID is included. CU should translate gNB-CU UE F1AP ID to gNB-DU UE F1AP ID when forwarding this information to the source gNB-DU. Since there is nothing special for CU, it is not necessary to capture the text on the translation of the UE F1AP IDs.

Proposal 6: It is proposed to just remove the Editor’s note on the translation of the UE F1AP IDs because translate gNB-DU UE F1AP ID to gNB-CU UE F1AP ID is gNB-CU’s implementation, no need to captured in specification.

There is FFS on whether there may be a case that wrong beam issue and TA issue occurs at the same time. For example, due to wrong TA, LTM fails, but most of the time beam signal varies fast, when UE perform recovery, new beam may be selected. So, both different beam and different TA may be detected by target DU at the same time. We think it is a common case. Target DU can send both of them to source DU, and source DU make the final analysis.

For the text in TS38.473, it may be updated as below:

Interaction with the Access and Mobility Indication procedure: If the Beam Failure Recovery Information IE, the LTM Cell Switch Failure due to Wrong Beam Information IE and the TA Information IE is included in the DU-CU ACCESS AND MOBILITY INDICATION message, the gNB-CU shall, if supported, forward this information to the source gNB-DU via the ACCESS AND MOBILITY INDICATION message including the Beam Failure Recovery Information IE, the LTM Cell Switch Failure due to Wrong Beam Information IE and the TA Information IE respectively.

Proposal 7: Both wrong beam issue and TA issue may occur during one LTM.

For the UE F1AP ID in DU-CU ACCESS AND MOBILITY INDICATION message and ACCESS AND MOBILITY INDICATION message, there is FFS captured below:

Editor’s note: FFS if a common structure is needed for UE F1AP IDs.

In our understanding, common structure is preferred because gNB-CU UE F1AP ID in Beam Failure Recovery Information, LTM Cell Switch Failure due to Wrong Beam Information and TA Information (name is FFS) is always same.

Proposal 8: It is proposed to adopt common structure for UE F1AP IDs in DU-CU ACCESS AND MOBILITY INDICATION message and ACCESS AND MOBILITY INDICATION message.

  - Stage 2 description

According to P4 and P7, stage 2 should be updated as below:

The target gNB-DU identifies that the UE successfully performs a RACH-based access while re-establishing/recovering to the same target cell but to a beam different from the beam used during the failed LTM cell switch execution. The target gNB-DU may send the re-established/recovery beam information to the source gNB-DU via the gNB-CU. The source gNB-DU performs root cause analysis to detect RACH-less LTM failure caused by wrong beam selection. The target gNB-DU identifies that the UE successfully performed a RACH-based access while re-establishing/recovering to the same target cell and same target beam but with TA value different from the one which was included in CU-DU CELL SWITCH NOTIFICATION message received from gNB-CU at LTM Cell Switch execution. The target gNB-DU sends the TA value used at successful RACH-based access to the source gNB-DU via the gNB-CU. The target gNB-DU identifies that the UE successfully performed a RACH-based access while re-establishing/recovering to the same target cell, but with both beam and TA value different from those used during the failed LTM cell switch execution. The target gNB-DU sends the re-established/recovery beam information and TA value used at successful RACH-based access to the source gNB-DU via the gNB-CU.

Proposal 9: It is proposed for RAN3 to discuss above stage2 description.

  - Mobility Information

According to legacy MRO, for the case that RLF occurs shortly after successful handover, source NG-RAN may not save UE context for the following reason:

1. Successful handover is the common case and usually the number of successful handed over UE is large. NG-RAN may not save UE context for each successful handed over UE.

2. Source NG-RAN stores UE context to wait for handover Report message from target NG-RAN. If no handover Report message received, UE context may have to be stored for 48 hours. For the same reason as above bullet, most of the time source NG-RAN cannot receive handover Report message.

Considering the complexity of saving UE context in source NG-RAN, Mobility Information is introduced in XN interface message e.g., indicating UE type.

If the cause for the failure is identified as a "Too Early HO" or a "HO to Wrong Cell", the NG-RAN node controlling the last serving cell shall, include in the HANDOVER REPORT message the C-RNTI used in the source cell of the last completed handover before the failure. If the NG RAN node controlling that source cell provided the Mobility Information, it is also included in the HANDOVER REPORT message. If used, the Mobility Information is prepared at the source NG RAN node of a handover and may refer to or identify any handover-related data at this NG RAN node.

Observation 2: For the case that RLF occurs shortly after successful handover, Mobility Information is introduced in XN interface message to avoid UE context saved in source NG-RAN for every successful handover.

For legacy handover, it is CU to perform MRO based on Mobility Information. While for LTM, it is DU to perform MRO. So, Mobility Information should be provided to DU to avoid UE context saved by DU for each successful handover.

For intra-CU inter-DU LTM, source DU could provide Mobility Information to CU via UE CONTEXT MODIFICATION RESPONSE, DU-CU CELL SWITCH NOTIFICATION or UE CONTEXT RELEASE COMPLETE message, etc. Then CU stores Mobility Information and provides Mobility Information back to DU via ACCESS AND MOBILITY INDICATION message.

If the cause for RLF which occurs shortly after successful handover is identified as a "Too Early LTM" or a "LTM to Wrong Cell", the gNB-CU shall include the C-RNTI used in the source cell of the last completed handover before the failure in the ACCESS AND MOBILITY INDICATION message. If the source gNB-DU controlling that source cell provided the Mobility Information, it is also included in the ACCESS AND MOBILITY INDICATION message. If used, the Mobility Information is prepared at the source gNB-DU of a LTM and may refer to or identify any LTM-related data at this source gNB-DU.

Proposal 10: It is proposed to introduce Mobility Information in F1 interface message. The TPs for stage 2 and stage 3 are in Annex.

Conclusions

Proposal 1: CSI-RS-ResourceId should not be included in DU-CU ACCESS AND MOBILITY INDICATION message to indicate the recovery beam information if CSI-RS is selected in beam failure recovery procedure.

Proposal 2: For BFR case, the recovery beam should be SSB index rather than TCIstateID.

Observation 1: SSB used in BFR could be configured by BeamFailureRecoveryConfig IE while TCI state is configured by LTM-Candidate IE.

Proposal 3: RAN3 to discuss whether providing SSB configuration (current SSB Information IE) used in BFR to source DU in DU-CU ACCESS AND MOBILITY INDICATION message.

Proposal 4: In case of successful recovery, target DU sends recovery beam information to source DU. In case of successful re-established, target DU sends SSB index to source DU and further discuss on whether also sends related SSB configuration. In case of reconnect, recovery beam information should not be sent to source DU.

Proposal 5: It is proposed to send full UHI with L1/L3 type in ACCESS AND MOBILITY INDICATION message from CU to DU, i.e. solution 1a/1b.

Proposal 6: It is proposed to just remove the Editor’s note on the translation of the UE F1AP IDs because translate gNB-DU UE F1AP ID to gNB-CU UE F1AP ID is gNB-CU’s implementation, no need to captured in specification.

Proposal 7: Both wrong beam issue and TA issue may occur during one LTM.

Proposal 8: It is proposed to adopt common structure for UE F1AP IDs in DU-CU ACCESS AND MOBILITY INDICATION message and ACCESS AND MOBILITY INDICATION message.

Proposal 9: It is proposed for RAN3 to discuss above stage2 description.

Observation 2: For the case that RLF occurs shortly after successful handover, Mobility Information is introduced in XN interface message to avoid UE context saved in source NG-RAN for every successful handover.

Proposal 10: It is proposed to introduce Mobility Information in F1 interface message. The TPs for stage 2 and stage 3 are in Annex.

Reference

[1] R3-251514“LS on SON for LTM”RAN WG3 Meeting #127-bis

TP for TS 38.473

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>>>>>

### 8.3.3	UE Context Release (gNB-CU initiated)

#### 8.3.3.1	General

The purpose of the UE Context Release procedure is to enable the gNB-CU to order the release of the UE-associated logical connection, candidate cells in conditional handover, conditional PSCell addition, or conditional PSCell change or LTM or subsequent CPAC. The procedure uses UE-associated signalling.

#### 8.3.3.2	Successful Operation

Figure 8.3.3.2-1: UE Context Release (gNB-CU initiated) procedure. Successful operation

The gNB-CU initiates the procedure by sending the UE CONTEXT RELEASE COMMAND message to the gNB-DU.

Upon reception of the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall release all related signalling and user data transport resources and reply with the UE CONTEXT RELEASE COMPLETE message. If the CG-SDT Kept Indicator IE is contained in the UE CONTEXT RELEASE COMMAND message and set to "true", the gNB-DU shall, if supported, consider that the UE is sent to RRC_INACTIVE state with CG-SDT configuration and store the configured CG-SDT resources, C-RNTI, CG-SDT-CS-RNTI, the CG-SDT related RLC configurations and F1-U connections associated with the SDT bearers while releasing the UE context.

If the old gNB-DU UE F1AP ID IE is included in the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall additionally release the UE context associated with the old gNB-DU UE F1AP ID.

If the UE CONTEXT RELEASE COMMAND message contains the RRC-Container IE, the gNB-DU shall send the RRC container to the UE via the SRB indicated by the SRB ID IE.

If the UE CONTEXT RELEASE COMMAND message includes the Execute Duplication IE, the gNB-DU shall perform CA based duplication or multi-path relay based duplication, if configured, for the SRB for the included RRC-Container IE.

If the Candidate Cells To Be Cancelled List IE is included in the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall consider that the gNB-CU is cancelling only the conditional handover, conditional PSCell addition, conditional PSCell change, or subsequent CPAC associated to the cells identified by the included NR CGIs and associated to the UE-associated signalingidentified by the gNB-CU UE F1AP ID IE and the gNB-DU UE F1AP ID IE.

If the Positioning Context Reservation Indication IE is included in the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall not release the positioning context including the SRS configuration for the UE.

If the LTM Cells To Be Released List IE is included in the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall, if supported, consider that the gNB-CU is cancelling only the LTM cells identified by the included NR CGIs and associated to the UE-associated signalingidentified by the gNB-CU UE F1AP ID IE and the gNB-DU UE F1AP ID IE.

If the Recommended SSBs for Paging List IE is included in the UE CONTEXT RELEASE COMPLETE message, the gNB-CU shall, if supported, store it and may use it as assistance information for subsequent paging.

If the DL LBT Failure Information Request IE is included in the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall, if supported, consider that the gNB-CU requests the reporting of DL LBT Failure information for the analysis of the MRO events of the UE specified in TS 38.300 [6],  and act as specified in TS 38.401 [4].

If the Mobility Information IE is provided in the UE CONTEXT RELEASE COMPLETE message, the gNB-CU shall, if supported, store this informationand use it as specified in TS 38.401 [4].

Interactions with UE Context Setup procedure:

The UE Context Release procedure may be performed before the UE Context Setup procedure to release an existing UE-associated logical F1-connection and related resources in the gNB-DU, e.g. when gNB-CU rejects UE access it shall trigger UE Context Release procedure with the cause value of UE rejection.

### 8.3.4	UE Context Modification (gNB-CU initiated)

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources or sidelink resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

Upon reception of the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall perform the modifications, and if successful reports the update in the UE CONTEXT MODIFICATION RESPONSE message.

<<<Unchanged part is omitted>>>

If the Ranging and Sidelink Positioning Service Information IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, update its service information for the UE accordingly. If the Ranging and Sidelink Positioning Authorized IE within the Ranging and Sidelink Positioning Service Information IE is set to "not authorized", the gNB-DU shall, if supported, initiate actions to ensure that the UE is no longer accessing the Ranging and Sidelink Positioning service.

If the Mobility Information IE is provided in the UE CONTEXT RELEASE COMPLETE message, the gNB-CU shall, if supported, store this informationand use it as specified in TS 38.401 [4].

Interaction with UE Inactivity Notification procedure

If the SDT Volume Threshold IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use the information during an SDT transaction to inform the gNB-CU via the UE INACTIVITY NOTIFICATION message as specified in TS 38.401 [4].

Interaction with UE Context Setup or UE Context Modification (gNB-CU initiated) procedures

If the UE CONTEXT MODIFICATION REQUEST message is sent for a UE context set up for S-CPAC and contains the Transmission Action Indicator IE set to "stop", the gNB-DU shall, if supported, reset the UE context for the included SpCell ID IE, prepare for subsequent CPAC. The gNB-DU shall include the SpCell ID IE as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message.

### 8.3.9	DU-CU Cell Switch Notification

#### 8.3.9.1	General

The purpose of the DU-CU Cell Switch Notification procedure is to enable thegNB-DU to inform the gNB-CU about the initiation of the cell switch command to the UE. This procedure is also used to transfer the selected TCI state from the gNB-DU to the gNB-CU. The procedure uses UE-associated signalling.

#### 8.3.9.2	Successful Operation

Figure 8.3.9.2-1: DU-CU Cell Switch Notification procedure. Successful operation.

The gNB-DU initiates the procedure by sending a DU-CU CELL SWITCH NOTIFICATION message.

Upon reception of the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall, if supported, consider that a cell switch command was sent to the UE where the target cell is indicated by the included Cell ID IE.

Interaction with CU-DU Cell Switch Notification procedure:

If the LTM Cell Switch Information IE is included in the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall, if supported, forward it to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message.

If the TA Information List IE is included in the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall, if supported, forward it to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message to transfer the TA value(s).

If the Mobility Information IE is provided in the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall, if supported, store this information and use it as specified in TS 38.401 [4].

- 8.11.2	DU-CU Access and Mobility Indication

- 8.11.2.1	General

This procedure is initiated by the gNB-DU to send the Access and Mobility related Information to the gNB-CU.

The procedure uses non-UE-associated signalling.

- 8.11.2.2	Successful Operation

Figure 8.11.2.2-1: DU-CU Access and Mobility Indication procedure. Successful operation

The DU-CU Access and Mobility Indication procedure is initiated by DU-CU ACCESS AND MOBILITY INDICATION message sent from the gNB-DU to the gNB-CU.

If the DU-CU ACCESS AND MOBILITY INDICATION message contains the DL LBT Failure Information List IE, the gNB-CU shall take it into account for optimisation of mobility parameters.

Interaction with the Access and Mobility Indication procedure:

If the Beam Failure Recovery Information IE, the LTM Cell Switch Failure due to Wrong Beam Information IEand the TA Information IE is included in the DU-CU ACCESS AND MOBILITY INDICATION message, the gNB-CU shall, if supported, forward this information to the source gNB-DU via the ACCESS AND MOBILITY INDICATION message including the Beam Failure Recovery Information IE, the LTM Cell Switch Failure due to Wrong Beam Information IE and the TA Information IE (name is FFS) respectively.

- 8.11.2.3	Abnormal Conditions

Not applicable.

[snip]

9.2	Message Functional Definition and Content

[snip]

- 9.2.2.6	UE CONTEXT RELEASE COMPLETE

This message is sent by the gNB-DU to confirm the release of the UE-associated logical F1 connectionor candidate cells in conditional handover, conditional PSCell addition,conditionalPSCell change or subsequent CPAC.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Recommended SSBs for Paging List | O |  | 9.3.1.297 |  | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) |  | YES | ignore |

- 9.2.2.8	UE CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-DU to confirm the modification of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the SgNB Resource Coordination Information IE as defined in subclause 9.2.117 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| DU To CU RRC Information | O |  | 9.3.1.26 |  | YES | reject |
| DRB Setup List |  | 0..1 |  | The List of DRBs which are successfully established. | YES | ignore |
| >DRB Setup Item IEs |  | 1 ..<maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>LCID | O |  | 9.3.1.35 | LCID for the primary pathor for the split secondary path for fallback to split bearer if PDCP duplication is applied. | - |  |
| >>DL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>DL UP TNL Information to Be Setup Item IEs |  | 1 ..<maxnoofDLUPTNLInformation> |  |  | - |  |
| >>>>DL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 ..<maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 | This IE is not used in this version of the specification. | YES | ignore |
| >>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.3.1.123 | Index to the currently fulfilled alternative QoS parameters set. | YES | ignore |
| >>TSC Traffic Characteristics Feedback | O |  | 9.3.1.302 |  | YES | ignore |
| >>ECN Marking or Congestion Information Reporting Status | O |  | 9.3.1.322 |  | YES | ignore |
| DRB Modified List |  | 0..1 |  | The List of DRBs which are successfully modified. | YES | ignore |
| >DRB Modified Item IEs |  | 1 ..<maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>LCID | O |  | 9.3.1.35 | LCID for the primary path or for the split secondary path for fallback to split bearer if PDCP duplication is applied. | - |  |
| >>DL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>DL UP TNL Information to Be Setup Item IEs |  | 1 ..<maxnoofDLUPTNLInformation> |  |  | - |  |
| >>>>DL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>RLC Status | O |  | 9.3.1.69 | Indicates the RLC has been re-established at the gNB-DU. | YES | ignore |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 ..<maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 | This IE is not used inthis version of the specification. | YES | ignore |
| >>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.3.1.123 | Index to the currently fulfilled alternative QoS parameters set. | YES | ignore |
| >>TSC Traffic Characteristics Feedback | O |  | 9.3.1.302 |  | YES | ignore |
| >>ECN Marking or Congestion Information Reporting Status | O |  | 9.3.1.322 |  | YES | ignore |
| SRB Failed to be Setup List |  | 0..1 |  | The List of SRBs which are failed to be established. | YES | ignore |
| >SRB Failed to be Setup Item IEs |  | 1 ..<maxnoofSRBs> |  |  | EACH | ignore |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| DRB Failed to be Setup List |  | 0..1 |  | The List of DRBs which are failed to be setup. | YES | ignore |
| >DRB Failed to be Setup Item IEs |  | 1 ..<maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| SCell Failed To Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell Failed to Setup Item |  | 1 ..<maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI9.3.1.12 | SCell Identifier in gNB | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| DRB Failed to be Modified List |  | 0..1 |  | The List of DRBs which are failed to be modified. | YES | ignore |
| >DRB Failed to be Modified Item IEs |  | 1 ..<maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Inactivity Monitoring Response | O |  | ENUMERATED (Not-supported, ...) |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the gNB-DU | YES | ignore |
| Associated SCell List | O |  | 9.3.1.77 |  | YES | ignore |
| SRB Setup List |  | 0..1 |  |  | YES | ignore |
| >SRB Setup Item |  | 1 ..<maxnoofSRBs> |  |  | EACH | ignore |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>LCID | M |  | 9.3.1.35 | LCID for the primary path if PDCP duplication is applied | - |  |
| SRB Modified List |  | 0..1 |  |  | YES | ignore |
| >SRB Modified Item |  | 1 ..<maxnoofSRBs> |  |  | EACH | ignore |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>LCID | M |  | 9.3.1.35 | LCID for the primary path if PDCP duplication is applied | - |  |
| Full Configuration | O |  | ENUMERATED (full, ...) |  | YES | reject |
| BH RLC Channel Setup List |  | 0..1 |  | The list of BH RLC channelswhich are successfully established. | YES | ignore |
| >BH RLC Channel Setup Item |  | 1 ..<maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | BH RLC Channel ID 9.3.1.113 |  | - |  |
| BH RLC Channel Modified List |  | 0..1 |  | The list of BH RLC channelswhich are successfully modified. | YES | ignore |
| >BH RLC Channel Modified Item |  | 1 ..<maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | BH RLC Channel ID 9.3.1.113 |  | - |  |
| BH RLC Channel Failed to be Setup List |  | 0..1 |  | The list of BH RLC channels whose setup has failed. | YES | ignore |
| >BH RLC Channel Failed to be Setup Item |  | 1 ..<maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | BH RLC Channel ID 9.3.1.113 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| BH RLC Channel Failed to be Modified List |  | 0..1 |  | The list of BH RLC channels whose modification has failed. | YES | ignore |
| >BH RLC Channel Failed to be Modified Item |  | 1 ..<maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | BH RLC Channel ID 9.3.1.113 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| SL DRB Setup List |  | 0..1 |  | The List of SL DRBs which are successfully established. | YES | ignore |
| >SL DRB Setup Item IEs |  | 1 ..<maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| SL DRB Modified List |  | 0..1 |  | The List of SL DRBs which are successfully modified. | YES | ignore |
| >SL DRB Modified Item IEs |  | 1 ..<maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| SL DRB Failed To Setup List |  | 0..1 |  | The List of SL DRBs which are failed to be setup. | YES | ignore |
| >SL DRB Failed To Setup Item |  | 1 ..<maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| SL DRB Failed To be Modified List |  | 0..1 |  | The List of SL DRBs which are failed to be modified. | YES | ignore |
| >SL DRB Failed To be Modified Item |  | 1 ..<maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>cause | O |  | 9.3.1.2 |  | - |  |
| Requested Target Cell ID | O |  | NR CGI9.3.1.12 | Special Cell or PSCell ID in the CPAC MCG InformationIEindicated in the UE CONTEXT MODIFICATION REQUEST message. | YES | reject |
| SCG Activation Status | O |  | 9.3.1.234 |  | YES | ignore |
| Uu RLC Channel Setup List |  | 0..1 |  |  | YES | ignore |
| >Uu RLC Channel Setup Item IEs |  | 1 ..<maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  |  |  |
| Uu RLC Channel Failed to be Setup List |  | 0..1 |  |  | YES | ignore |
| >Uu RLC Channel Failed to be Setup Item IEs |  | 1 ..<maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Uu RLC Channel Modified List |  | 0..1 |  |  | YES | ignore |
| >Uu RLC Channel Modified Item IEs |  | 1 ..<maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| Uu RLC Channel Failed to be Modified List |  | 0..1 |  |  | YES | ignore |
| >Uu RLC Channel Failed to be Modified Item IEs |  | 1 ..<maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| PC5 RLC Channel Setup List |  | 0..1 |  |  | YES | ignore |
| >PC5 RLC Channel Setup Item IEs |  | 1 ..<maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| PC5 RLC Channel Failed to be Setup List |  | 0..1 |  |  | YES | ignore |
| >PC5 RLC Channel Failed to be Setup Item IEs |  | 1 ..<maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| PC5 RLC Channel Modified List |  | 0..1 |  |  | YES | ignore |
| >PC5 RLC Channel Modified Item IEs |  | 1 ..<maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| PC5 RLC Channel Failed to be Modified List |  | 0..1 |  |  | YES | ignore |
| >PC5 RLC Channel Failed to be Modified Item IEs |  | 1 ..<maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| SDT Bearer Configuration Info | O |  | 9.3.1.277 |  | YES | ignore |
| UE Multicast MRB Setup List |  | 0..1 |  |  | YES | reject |
| >UE Multicast MRB Setup Item IEs |  | 1 ..<maxnoofMRBsforUE> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 | MRB ID for the UE. | - |  |
| >>Multicast F1-U Context Reference CU | M |  | 9.3.2.13 |  | - |  |
| ServingCellMO-encoded-in-CGC List |  | 0..1 |  |  | YES | ignore |
| >ServingCellMO-encoded-in-CGC Item IEs |  | 1 ..<maxNrofBWPs> |  | The servingCellMO which has been encoded in CellGroupConfig IE. | - |  |
| >>servingCellMO | M |  | INTEGER (1..64, ...) |  | - |  |
| >>BWP ID | M |  | INTEGER (0..4) |  | YES | ignore |
| Dedicated SI Delivery Indication | O |  | ENUMERATED(true,...) |  | YES | ignore |
| Configured BWP List |  | 0..1 |  | This IE is present when the gNB-DU configures at least one BWP with NCD-SSB or without SSB. | YES | ignore |
| >Configured BWP Item IEs |  | 1 ..<maxNrofBWPs> |  |  | EACH | ignore |
| >>BWP-Id | M |  | INTEGER (0..4) | The IE is used to refer to one BWP. | - |  |
| >>BWP Location And Bandwidth | M |  | INTEGER (0..37949) | The IE type range is the same as the locationAndBandwidth IE in BWP IE as specified in TS 38.331 [8]. |  |  |
| Early Sync Information |  | 0..1 |  |  | YES | ignore |
| >TCI States Configurations List | M |  | OCTET STRING | Includes theLTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| >Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |
| LTM Configuration |  | 0..1 |  |  | YES | ignore |
| >SSB Information | M |  | 9.3.1.202 | Includes the SSB Information for the requested target cell | - |  |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |
| >LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. | - |  |
| >LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. | - |  |
| S-CPAC Configuration |  | 0..1 |  |  | YES | ignore |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfigIE, as defined in TS 38.331 [8]. | - |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |
| Mobility Information | O |  | BIT STRING (SIZE (32)) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofDLUPTNLInformation | Maximum no. of DL UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying or L2 N3C relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channels allowed for L2 U2N or L2 U2U relaying per Remote UE or Relay UE, the maximum value is 512. |
| maxNrofBWPs | Maximum number of BWPs per serving cell, the maximum value is 8. |
| maxnoofMRBsforUE | Maximum no. of multicast MRB allowed towards one UE, the maximum value is 64. |

- 9.2.2.15	DU-CU CELL SWITCH NOTIFICATION

This message is sent by the gNB-DU to inform the gNB-CU about the initiation of the cell switch  command to the UE.

Direction:gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includesthe TCI-StateIdIE,as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateIdIE,as defined in TS 38.331 [8]. | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 ..<maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

[snip]

- 9.2.10	Self Optimisation Support Messages

- 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >>C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU. This IE is included in case the gNB-DU responsible for the LTM failure is not the gNB-DU serving the UE at the time of LTM failure. | YES | ignore |
| >>RLF Report Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |
| >>MobilityInformation | O |  | BIT STRING (SIZE (32)) |  | YES | ignore |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| Successful PSCell Change Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report information Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in TS 38.331 [8]. | - |  |
| Beam Failure Recovery Information | O |  |  |  | YES | ignore |
| >SSB Time/Frequency Configuration | M |  | 9.3.1.203 |  |  |  |
| >Recovery SSB index | M |  | INTEGER (0..63) | recovery SSB index | - |  |
| LTM Cell Switch Failure due to Wrong Beam Information | O |  |  |  | YES | ignore |
| >SSB Time/Frequency Configuration | M |  | 9.3.1.203 |  |  |  |
| >Target SSB index | M |  | INTEGER (0..63) | target beam index | - |  |
| TA Information | O |  |  |  | YES | ignore |
| >TA Value | M |  | INTEGER (0..4095) | Indicates the TA value used at successful Random Access, as defined in TS 38.213 [31]. | - |  |
| gNB-DU UE F1AP ID | C-ifBeam Failure Recovery Information or LTM Cell Switch Failure due to Wrong Beam Information or TA Information |  | 9.3.1.5 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports. Value is 64. |

- 9.2.10.2	DU-CU ACCESS AND MOBILITY INDICATION

This message is sent by the gNB-DU to provide access and mobility information to the gNB-CU.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| DL LBT Failure Information List |  | 0..1 |  |  | YES | ignore |
| > DL LBT Failure Information Item |  | 1 .. <maxnoofLBTFailureInformation> |  |  | - |  |
| >>DL LBT Failure Information | M |  | 9.3.1.327 |  | - |  |
| Beam Failure Recovery Information | O |  |  |  | YES | ignore |
| >SSB Time/Frequency Configuration | M |  | 9.3.1.203 |  |  |  |
| >Recovery SSB index | M |  | INTEGER (0..63) | recovery SSB index | - |  |
| LTM Cell Switch Failure due to Wrong Beam Information | O |  |  |  | YES | ignore |
| >SSB Time/Frequency Configuration | M |  | 9.3.1.203 |  |  |  |
| >Target SSB index | M |  | INTEGER (0..63) | target beam index | - |  |
| TA Information | O |  |  |  | YES | ignore |
| >TA Value | M |  | INTEGER (0..4095) | Indicates the TA value used at successful Random Access, as defined in TS 38.213 [31]. | - |  |
| gNB-CU UE F1AP ID | C-ifBeam Failure Recovery Information or LTM Cell Switch Failure due to Wrong Beam Information or TA Information |  | 9.3.1.4 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLBTFailureInformation | Maximum no. of UEs for which LBT Failure Information is provided, the maximum value is 64. |

[snip]

9.4	Message and Information Element Abstract Syntax (with ASN.1)

[snip]

- 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::= ¨

[snip]

F1U-PathFailure,

LTMResetInformation,

CuBeamFailureRecoveryInformation,

CuLTMCellSwitchFailureduetoWrongBeamInformation,

CuTAInformation,

DuBeamFailureRecoveryInformation,

DuLTMCellSwitchFailureduetoWrongBeamInformation,

DuTAInformation

[snip]

id-LTMResetInformation,

id-PreconfiguredSRSInformation,

id-CuBeamFailureRecoveryInformation,

id-CuLTMCellSwitchFailureduetoWrongBeamInformation,

id-CuTAInformation,

id-DuBeamFailureRecoveryInformation,

id-DuLTMCellSwitchFailureduetoWrongBeamInformation,

id-DuTAInformation,

[snip]

-- **************************************************************

--

--  Access And Mobility Indication ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- Access And Mobility Indication

--

-- **************************************************************

AccessAndMobilityIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { AccessAndMobilityIndicationIEs} },

...

}

[snip]

AccessAndMobilityIndicationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory }|

{ ID id-RAReportList				CRITICALITY ignore	TYPE RAReportList			PRESENCE optional }|

{ ID id-RLFReportInformationList				CRITICALITY ignore	TYPE RLFReportInformationList				PRESENCE optional }|

{ ID id-SuccessfulHOReportInformationList		CRITICALITY ignore	TYPE SuccessfulHOReportInformationList	PRESENCE optional }|

{ ID id-SuccessfulPSCellChangeReportInformationList	CRITICALITY ignore	TYPE SuccessfulPSCellChangeReportInformationList		PRESENCE optional }|

{ ID id-CuBeamFailureRecoveryInformation			CRITICALITY ignore 	TYPE CuBeamFailureRecoveryInformation  		PRESENCE optional }|

{ ID id-CuLTMCellSwitchFailureduetoWrongBeamInformation			CRITICALITY ignore 	TYPE CuLTMCellSwitchFailureduetoWrongBeamInformation		PRESENCE optional }|

{ ID id-CuTAInformation			CRITICALITY ignore 	TYPE CuTAInformation		PRESENCE optional }|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY ignore 	TYPE GNB-DU-UE-F1AP-ID		PRESENCE optional },

...

}

[snip]

-- **************************************************************

--

--  DU-CU Access And Mobility Indication ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- DU-CU Access And Mobility Indication

--

-- **************************************************************

DUCUAccessAndMobilityIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { DUCUAccessAndMobilityIndicationIEs} },

...

}

DUCUAccessAndMobilityIndicationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID							PRESENCE mandatory }|

{ ID id-DLLBTFailureInformationList				CRITICALITY ignore	TYPE DLLBTFailureInformationList		PRESENCE optional}|

{ ID id-DuBeamFailureRecoveryInformation		CRITICALITY ignore	TYPE DuBeamFailureRecoveryInformation	PRESENCE optional }|

{ ID id-DuLTMCellSwitchFailureduetoWrongBeamInformation		CRITICALITY ignore	TYPE DuLTMCellSwitchFailureduetoWrongBeamInformation	PRESENCE optional }|

{ ID id-DuTAInformation		CRITICALITY ignore	TYPE DuTAInformation	PRESENCE optional }|

{ ID id-gNB-CU-UE-F1AP-ID		CRITICALITY ignore	TYPE GNB-CU-UE-F1AP-ID	PRESENCE optional },

...

}

- 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

[snip]

id-TagIDPointer,

id-LocalOrigin,

id-SRSPosPeriodicConfigHyperSFNIndex,

id-rLFReportFailureType

[snip]

CuBeamFailureRecoveryInformation	::= SEQUENCE {

SSBTime/FrequencyConfiguration     SSBInformationItem,

sSBIndex						INTEGER(0..63),

iE-Extensions		ProtocolExtensionContainer { { CuBeamFailureRecoveryInformation-ExtIEs} } OPTIONAL,

...

}

CuBeamFailureRecoveryInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

CuLTMCellSwitchFailureduetoWrongBeamInformation	::= SEQUENCE {

SSBTime/FrequencyConfiguration     SSBInformationItem,

sSBIndex						INTEGER(0..63),

iE-Extensions		ProtocolExtensionContainer { { CuLTMCellSwitchFailureduetoWrongBeamInformation -ExtIEs} } OPTIONAL,

...

}

CuLTMCellSwitchFailureduetoWrongBeamInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

CuTAInformation	::= SEQUENCE {

TAValue						INTEGER(0..4095),

iE-Extensions		ProtocolExtensionContainer { { CuTAInformation-ExtIEs} } OPTIONAL,

...

}

CuTAInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

[snip]

DuBeamFailureRecoveryInformation	::= SEQUENCE {

SSBTime/FrequencyConfiguration     SSBInformationItem,

sSBIndex						INTEGER(0..63),

iE-Extensions		ProtocolExtensionContainer { { DuBeamFailureRecoveryInformation-ExtIEs} } OPTIONAL,

...

}

DuBeamFailureRecoveryInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

DuLTMCellSwitchFailureduetoWrongBeamInformation	::= SEQUENCE {

SSBTime/FrequencyConfiguration     SSBInformationItem,

sSBIndex						INTEGER(0..63),

iE-Extensions		ProtocolExtensionContainer { { DuLTMCellSwitchFailureduetoWrongBeamInformation -ExtIEs} } OPTIONAL,

...

}

DuLTMCellSwitchFailureduetoWrongBeamInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

DuTAInformation	::= SEQUENCE {

TAValue						INTEGER(0..4095),

iE-Extensions		ProtocolExtensionContainer { { DuTAInformation-ExtIEs} } OPTIONAL,

...

}

DuTAInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

[snip]

RLFReportInformationList	::= SEQUENCE (SIZE(1.. maxnoofRLFReports)) OF RLFReportInformationItem

RLFReportInformationItem	::= SEQUENCE {

nRUERLFReportContainer		NRUERLFReportContainer,

uEAssitantIdentifier		GNB-DU-UE-F1AP-ID		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { RLFReportInformationItem-ExtIEs} }	OPTIONAL,

...

}

RLFReportInformationItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ID id-C-RNTI				CRITICALITY ignore EXTENSION C-RNTI					PRESENCE optional },

{ID id-rLFReportFailureType	CRITICALITY ignore EXTENSION RLFReportFailureType		PRESENCE optional },

...

}

RLFReportFailureType ::= ENUMERATED{ toolateLTM, tooearlyLTM, lTMtowrongcell,...}

[snip]

- 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules

[snip]

id-LocalOrigin										ProtocolIE-ID ::= 854

id-LTMResetInformation								ProtocolIE-ID ::= 855

id-SRSPosPeriodicConfigHyperSFNIndex				ProtocolIE-ID ::= 856

id-PreconfiguredSRSInformation						ProtocolIE-ID ::= 857

id-rLFReportFailureType								ProtocolIE-ID ::= 999 --to be assigned by MCC

id-CuBeamFailureRecoveryInformation					ProtocolIE-ID ::= 998 --to be assigned by MCC

id-CuLTMCellSwitchFailureduetoWrongBeamInformation  ProtocolIE-ID ::= 998 --to be assigned by MCC

id-CuTAInformation                                         ProtocolIE-ID ::= 998 --to be assigned by MCC

id-DuBeamFailureRecoveryInformation                     ProtocolIE-ID ::= 998 --to be assigned by MCC

id-DuLTMCellSwitchFailureduetoWrongBeamInformation  ProtocolIE-ID ::= 998 --to be assigned by MCC

id-DuTAInformation                                         ProtocolIE-ID ::= 998 --to be assigned by MCC

TP for TS 38.401

7.X	MRO support for LTM

The gNB-CU receives a RLF report associated to an LTM mobility event from the UE or via the Failure indication message over Xn. The gNB-CU performs initial analysis and in case of failure due to inappropriate cell switch triggering may forward the RLF report to the last serving gNB-DU in case of too late LTM, or to the source gNB-DU in case of too early LTM or LTM to wrong cell.

If the cause for RLF which occurs shortly after successful handover is identified as a "Too Early LTM" or a "LTM to Wrong Cell", the gNB-CU shall include the C-RNTI used in the source cell of the last completed handover before the failure in the ACCESS AND MOBILITY INDICATION message. If the source gNB-DU controlling that source cell provided the Mobility Information, it is also included in the ACCESS AND MOBILITY INDICATION message. If used, the Mobility Information is prepared at the source gNB-DU of a LTM and may refer to or identify any LTM-related data at this source gNB-DU.

The target gNB-DU identifies that a Beam Failure Recovery (BFR) has happened in the UE shortly after a successful LTM cell switch by detecting a time gap between the successful LTM cell switch and the BFR in the same cell is smaller than the configured threshold (e.g. Tstore_UE_cntxt).The target gNB-DU performs initial analysis and may send the recovery beam information to the source gNB-DU via the gNB-CU.

The target gNB-DU identifies that the UE successfully performs a RACH-based access while re-establishing/recovering to the same target cell but to a beam different from the beam used during the failed LTM cell switch execution. The target gNB-DU may send the re-established/recovery beam information to the source gNB-DU via the gNB-CU. The source gNB-DU performs root cause analysis to detect RACH-less LTM failure caused by wrong beam selection.

The target gNB-DU identifies that the UE successfully performed a RACH-based access while re-establishing/recovering to the same target cell and same target beam but with TA value different from the one which was included in CU-DU CELL SWITCH NOTIFICATION message received from gNB-CU at LTM Cell Switch execution. The target gNB-DU sends the TA value used at successful RACH-based access to the source gNB-DU via the gNB-CU.

The target gNB-DU identifies that the UE successfully performed a RACH-based access while re-establishing/recovering to the same target cell, but with both beam and TA value different from those used during the failed LTM cell switch execution. The target gNB-DU sends the re-established/recovery beam information and TA value used at successful RACH-based access to the source gNB-DU via the gNB-CU.