3GPP TSG-RAN WG3 #129                                                 R3-255645

Bengaluru, India, 25 - 29 August, 2025

Source:	CATT

Title:	Discussion on Evolution of Duplex Operation

Agenda Item:	19.2

Document for:	Discussion and decision

- Introduction

This document discusses leftover issue, RACH configuration exchange, one and–	One or more SRS resource configurations and DC case of Evolution of Duplex Operation.

- Discussion

  - RACH configuration exchange

BWP-UplinkCommon ::=                SEQUENCE {

sbfd-RACH-Config-r19                         CHOICE {

sbfd-RACH-SingleConfig-r19                   NULL,

sbfd-RACH-DualConfig-r19                     SBFD-RACH-DualConfig-r19

SBFD-RACH-DualConfig-r19 ::=                         SEQUENCE {

sbfd-AdditionalRACH-Config-r19                       RACH-ConfigCommon                           OPTIONAL,  -- Need R

sbfd-RACH-DualConfig-ValidRO-AcrossSymbolTypes-r19    ENUMERATED {enabled}                        OPTIONAL  -- Need R

38.300

Two RACH configuration options are specified for SBFD RA in TS 38.331 [12]. A cell can configure only one RACH configuration option. This can be either: 1) A single RACH configuration that supports both non-SBFD RA operation and SBFD RA operation, or 2) A dual RACH configuration where a RACH configuration is used for non-SBFD RA operation and an additional RACH configuration is designated for SBFD RA operation, as outlined in TS 38.331 [12].

According to RAN 1/2 agreement, two options for RACH configuration are supported. Option 1 is reuse/enable leagcy tdd-UL-DL-ConfigurationCommon IE for SBFD RA operation, option 2 requires an additional RACH configuration for SBFD RA operation. For option 2, RAN3 need to exchange SBFD RACH configuration between NG-RAN (Xn) and CU/DU (F1) i.e., SBFD-RACH-DualConfig-r19.

Proposal 1: For option 2, DU provides SBFD RACH configuration to CU in Served Cell Information IE, CU further sends to neighbour CU in Served Cell Information NR. Neighbour CU provides SBFD RACH configuration to DU via Neighbour Cell Information List.

  - Simultaneous configuration of SBFD and DC

LS [1] received from RAN2 as below

RAN2 has discussed whether SBFD and dual connectivity (DC) can be configured simultaneously for an SBFD-aware UE. However, no consensus has been reached. RAN2 would like to seek inputs from RAN1 by asking the following question:

Question: Whether SBFD and DC can be configured simultaneously, and whether there is any issue with such configuration for an SBFD-aware UE?

The answer of above question from RAN1 as below:

From RAN1 perspective, considering the specification impact, SBFD and NR-DC can be configured simultaneously only for inter-band NR-DC if a UE is capable of simultaneous transmission and reception indicated by UE capability simultaneousRxTxInterBandCA, where SBFD operation is applicable on only one NR TDD carrier.

Inform RAN2 that specification impact to support this feature is not restricted to RAN1 but might result in specification impact on RAN4 as well. If supported, at least the following specification change to TS 38.213 should be adopted and the RAN4 specification for the configured transmitted power level for NR-DC in TS38.101-1 Clause 6.2B.4.1 may also need to be updated accordingly.

It seems that RAN1 can support inter-band DC with some specification enhancement from technical point view. As RAN3 in CC, we can make some initial analysis about RAN3 specification impact and take RAN1/RAN2 conclusion into account.

From RAN3 point view, SBFD aware UE require to reduce interference between MN and SN. They should exchange SBFD time and location information and SSB/NZP CSI-RS resource/strongest DL beam/CLI mitigation request.

- SBFD time/location information and NZP CSI-RS resource configuration already have been included in served cell information which can be naturally supported for DC.

- Victim gNB (MN or SN) sends strongest DL beam and CLI Mitigation Indication to aggressor gNB (SN or MN) by reusing new introduced message i.e., CLI indication in Xn/F1. But the direction may include between M-NG-RAN node and S-NG-RAN node.

Proposal 2: SBFD DC can be naturally support with some small specification enhancement in RAN3. The final decision should be decided in other group.

  - Strongest NZP CSI-RS Resource Indication

Update the description of NZP CSI-RS Resource Indication IE in XnAP CR and CRI IE in F1AP CR to clarify the value is the relative index of the CSI-RS resources within a set of resources, the details to be checked.

maxNrofNZP-CSI-RS-Resources             INTEGER ::= 192     -- Maximum number of Non-Zero-Power (NZP) CSI-RS resources

maxNrofNZP-CSI-RS-ResourcesPerSet       INTEGER ::= 64      -- Maximum number of NZP CSI-RS resources per resource set

maxNrofNZP-CSI-RS-ResourceSets          INTEGER ::= 64      -- Maximum number of NZP CSI-RS resource sets per cell

Follow the current one set of NZP CSI-RS resource design, one NZP CSI-RS resource set includes 64 NZP CSI-RS resources but the maximum number of NZP CSI-RS resource is 192. In current BL CR, the CRI or NZP CSI-RS resource indication is up to 64 which may require clarification that it corresponds to the NZP CSI-RS resources configuration i.e., highlight IE as below.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NZP-CSI-RS-ResourceSet | M |  | OCTET STRING | Includes the NZP-CSI-RS-ResourceSet IE, as defined in TS 38.331 [10]. |
| NZP-CSI-RS-Resource List |  | 1 |  |  |
| >NZP-CSI-RS-Resource Item |  | 1..<maxnoofNZP-CSI-RS-ResourcesPerSet> |  |  |
| >>NZP-CSI-RS-Resource | M |  | OCTET STRING | Includes the NZP-CSI-RS-Resource IE, as defined in TS 38.331 [10]. |

Another more straight forward way is indicate the strongest CRI via nzp-CSI-RS-ResourceId, it would be clearer. Also, there is a lot of other information in NZP-CSI-RS-Resource e.g., resourceMapping and powerControlOffset which is not useful for indicating strongest CRI. If we indicate the strongest CRI is nzp-CSI-RS-ResourceId, the rang should be 0..191.

Proposal 3: Follow the current BL CR design that only one NZI CSI-RS resource set. The strongest NZP CSI-RS resource indication/CRI can up to 64 which is align with NZP CSI-RS Resources Configuration. Another way is use nzp-CSI-RS-ResourceId to indicate the strongest NZP CSI-RS resource indication/CRI. The range is 0..191.

  - SBFD time and frequency configuration

According to RAN1 LS, the SBFD time domain information including the following parameters:

- SBFD-StartingSlotIndex

- SBFD-StartingSymbolIndex

- SBFD-EndingSlotIndex

- SBFD-EndingSymbolIndex

SBFD frequency domain information including the following parameters:

- ulSubbandlocationAndBandwidth

- firstdlSubbandlocationAndBandwidth

- seconddlSubbandlocationAndBandwidth

Proposal 4: Update SBFD configuration in BL CR including RAN1 agreed parameters.

  - One or more SRS resource configurations

The gNB1 with aggressor UE will send SRS resource configurations to gNB2 which includes victim UE. The principle should be similar as NZP CSI-RS resource exchange. The question is whether there are more than one RS-ResourceSets or only one RS-ResourceSet which is similar as NZP CSI-RS resourceSet. Since there is no limitation provide by other groups, we propose to send a list of SRS-ResourceSet and a list of SRS-Resource. In details, gNB1 with aggressor UE sends SRS Resources Configuration includes a list of RS-ResourceSet and a list of SRS-Resource in served cell information. gNB2 which includes victim UE will feedback the SRS-ResourceSetId and SRS-ResourceId to gNB1 with aggressor UE. Note that the SRS-ResourceId can be same in different SRS-ResourceSet.

Proposal 5: gNB1 with aggressor UE send a list of RS-ResourceSet and a list of SRS-Resource in served cell information to gNB2 which includes victim UE. gNB2 sends back SRS-ResourceSetId and SRS-ResourceId to gNB1via CLI indication. The above signalling design can be also applied to F1 between CU and DU.

- Conclusion

Proposal 1: For option 2, DU provides SBFD RACH configuration to CU in Served Cell Information IE, CU further sends to neighbour CU in Served Cell Information NR. Neighbour CU provides SBFD RACH configuration to DU via Neighbour Cell Information List.

Proposal 2: SBFD DC can be naturally support with some small specification enhancement in RAN3. The final decision should be decided in other group.

Proposal 3: Follow the current BL CR design that only one NZI CSI-RS resource set. The strongest NZP CSI-RS resource indication/CRI can up to 64 which is align with NZP CSI-RS Resources Configuration. Another way is use nzp-CSI-RS-ResourceId to indicate the strongest NZP CSI-RS resource indication/CRI. The range is 0..191.

Proposal 4: Update SBFD configuration in BL CR including RAN1 agreed parameters.

Proposal 5: gNB1 with aggressor UE send a list of RS-ResourceSet and a list of SRS-Resource in served cell information to gNB2 which includes victim UE. gNB2 sends back SRS-ResourceSetId and SRS-ResourceId to gNB1via CLI indication. The above signalling design can be also applied to F1 between CU and DU.

- Reference

[1] R3-253028, LS on simultaneous configuration of SBFD and DC, RAN2.