3GPP TSG RAN WG3 Meeting #129                                                                              R3-255647

Bengaluru, India, 25 - 29 August, 2025

Agenda Item:	21.3

Source:	CATT

Title: 	Discussion on NR XR enhancement for others

Document for:	Discussion and Approval

- Introduction

This paper discusses NR XR enhancement for others including following issue.

For Specify uplink congestion signalling:

CU sends the uplink rate control indication per QoS flow over F1 to DU.

Other additional assistance information from CU to DU?

- Recommended UL bit rate infor per QoS flow

- Measured bit rate per QoS flow

- No additional information

For avoid RLC unnecessary retransmission

Reuse the existing PDCP discard indication in F1-U to indicate the gNB-DU to stop the transmission/retransmission of a RLC SDU or the segment of a RLC SDU. FFS on how to capture this agreement.

For uplink, FFS on whether/how gNB-DU can configure the autonomous retransmission and/or enhanced polling thresholds for the DRB.

For downlink, FFS on whether/how enhance Signalling between CU and DU to support timely retransmission.

- Discussion

  - UL bit rate control

Other additional assistance information from CU to DU?

- Measured bit rate per QoS flow

It is consider that DU cannot identify QFI hence companies propose to CU detects measured per QoS flow UL bit rate and sends to DU via F1. However, the Uu channel condition cannot be detected by CU in time. The DDDS has latency which cannot reflect the real time buffer situation in DU. Therefore, CU may not be able to provide suitable per QoS flow UL bit rate but MAC CE method is aim to achieve real time and fast UL bit rate control.

Observation 1: CU may be not able to provide real time per QoS UL bit rate because CU does not know Uu channel condition in time. But the MAC CE method is aim to achieve real time and fast UL bit rate control.

-	Recommended UL bit rate infor per QoS flow

This option is similar as above. The difference is that DU is able to take its own status into account and send suitable bit rate to UE. For example, CU provides recommended UL bit rate QoS 1 10mbps, QoS 2 20 mbps and QoS 3 30mbps. QoS 1 and QoS 2 mapped to DRB1, QoS 3 mapped to DRB2. If DU can only identify DRB level status, it just proposes the same adjust for QoS 1 and QoS 2 e.g., QoS 1 10-5mbps, QoS 2 20-5 mbps and QoS 3 30-10 mbps. The adjustment in DU is still per DRB level, benefit is limited.

Observation 2: If DU cannot identify QFI, it still cannot adjust recommended UL bit rate base on its own per QoS flow status. The benefit is limited.

- No additional information

DU is able to detect per QoS flow UL bit rate by proration split per DRB UL bit rate into per QoS flow according to QoS to DRB mapping. For example, DU knows per DRB bit rate is 40 mbps and there are three QoS flows mapped to one DRB with 2:1:1 QoS requirement. DU will indicate UL bit rate of QoS 1-3 with 20 mbps, 10 mbps and10 mbps. Another method is that DU is based on the implementation to identify QFI.

Observation 3: Without any additional information, DU is able to detect per QoS flow UL bit rate by implementation.

Proposal 1: No addition information is needed from CU to DU for UL bit rate control.

  - RLC retransmission

Unnecessary retransmissions avoidance

When the TX RLC entity receives a discard indication of the SDU from PDCP, the TX RLC entity considers the SDU as an outdated SDU. The TX RLC entity does not perform any transmission and retransmission of such SDU/SDU segment.

Based on RAN2 agreement, PDCP will send a discard indication to Tx RLC to stop any transmission and retransmission when timer expired in PDCP. For F1, PDCP is located at CU while RLC is located at DU.

Reuse the existing PDCP discard indication in F1-U to indicate the gNB-DU to stop the transmission/retransmission of a RLC SDU or the segment of a RLC SDU. FFS on how to capture this agreement.

In legacy, if RLC layer is sending RLC SDU to lower layer but it receives DL user data for PDCP discard at same time, how to handle this case is based on DU’s implementation. For now, we do not see the reason to update specifications. The current PDCP discard mechanism can be reused.

Proposal 2: Existing PDCP discard indication can be reused.

Autonomous retransmission and polling

For uplink, FFS on whether/how gNB-DU can configure the autonomous retransmission and/or enhanced polling thresholds for the DRB.

PDCP-Config information element

remainingTimeBasedReTxThreshold-r19              RLC-AM-RemainingTimeThreshold-r19          OPTIONAL,   -- Cond RLC-AM

remainingTimeBasedPollingThreshold-r19           RLC-AM-RemainingTimeThreshold-r19          OPTIONAL    -- Cond RLC-AM

unchanged part is omitted

RLC-AM-RemainingTimeThreshold-r19 ::= INTEGER (1..64)

The autonomous retransmission and enhanced polling thresholds are configured by PDCP-config via RRC message and autonomous retransmission and polling is performed by RLC layer of UE. PDCP layer will indicate lower layer that the condition for remaining-time-based RLC retransmission is met for the corresponding PDCP Data PDU. No RAN3 impact is identified for UL autonomous retransmission and polling

Proposal 3: Based on RAN2 updated specifications, the RLC-AM-RemainingTimeThreshold is generated by PDCP. PDCP layer will indicate lower layer when condition is met. No RAN3 impact.

For downlink, FFS on whether/how enhance Signalling between CU and DU to support timely retransmission.

However, for DL in case of CU/DU split, it is DU to perform autonomous retransmission and polling in RLC layer but threshold configuration is generated by PDCP which is located at CU. There are three options can be considered.

- Option 1: CU generates RLC-AM-RemainingTimeThreshold and sends to DU. It requires DU to calculate remaining time in RLC layer, which is a new behaviors. When remaining time in RLC layer less than RLC-AM-RemainingTimeThreshold, DU triggers autonomous retransmission or polling.

- Option 2: DU generates RLC-AM-RemainingTimeThreshold. It conflicts with RAN2 agreement that PDCP layer generate RLC-AM-RemainingTimeThreshold. When remaining time in RLC layer less than RLC-AM-RemainingTimeThreshold, DU triggers autonomous retransmission or polling.

- Option 3: CU generates RLC-AM-RemainingTimeThreshold and calculates remaining time. When remaining time less than RLC-AM-RemainingTimeThreshold, CU indicates DU to trigger autonomous retransmission RLC SDU or polling.

It considers that both option 1 and option 2 have new DU/RLC layer behaviours, we prefer option 3. It requires CU send a retransmission or polling indication to DU. F1-U specification need to be enhanced. The changes would be similar as DL discard mechanism.

Proposal 4: CU sends “retransmission in RLC or polling indication” to DU by DL user data for DL timely retransmission in DU.

For detail specification impact, there are three options

- Option 1: Add retransmission present flag and retransmission block present flag. Also, add retransmission PDCP PDU SN and retransmission block related information. The problem is that it will occupy all present bit and introduce a long DL user data frame.

- Option 2: Add retransmission/polling present flag and retransmission/polling PDCP PDU SN and block related information. It conflict with RAN2 logical that PDCP layer decide whether retransmission or polling is performed.

- Option 3: Add retransmission/polling present flag and retransmission/polling PDCP PDU SN and block related information. Also, CU indicate to the type of configuration i.e., retransmission/polling to DU in F1AP. DU will perform retransmission or polling base on configuration from CU.

We slightly prefer option 3 even if it introduces both F1-C and F1-U impact but this option is clearer.

Proposal 5: CU sends the type of remaining-time-based RLC configuration i.e., retransmission or polling to DU via F1AP.

3 Conclusions

In this contribution the following observations and proposal were derived:

Observation 1: CU may be not able to provide real time per QoS UL bit rate because CU does not know Uu channel condition in time. But the MAC CE method is aim to achieve real time and fast UL bit rate control.

Observation 2: If DU cannot identify QFI, it still cannot adjust recommended UL bit rate base on its own per QoS flow status. The benefit is limited.

Observation 3: Without any additional information, DU is able to detect per QoS flow UL bit rate by implementation.

Proposal 1: No addition information is needed from CU to DU for UL bit rate control.

Proposal 2: Existing PDCP discard indication can be reused.

Proposal 3: Based on RAN2 updated specifications, the RLC-AM-RemainingTimeThreshold is generated by PDCP. PDCP layer will indicate lower layer when condition is met. No RAN3 impact.

Proposal 4: CU sends “retransmission in RLC or polling indication” to DU by DL user data for DL timely retransmission in DU.

Proposal 5: CU sends the type of remaining-time-based RLC configuration i.e., retransmission or polling to DU via F1AP.

4 References