3GPP TSG-RAN WG3 Meeting #129	R3-255651

Bengaluru, India, 25th - 29th Aug, 2025

Title: 	Handling of UE Radio Capability for Paging

Source: 	Huawei

Agenda item:	8.1

Document Type:	Discussion

# 1. Introduction

The last SA2-169 meeting sent the reply LS on the UE radio capability for paging, with the following contents in [1].

1. Overall Description: SA2 thanks RAN3 for the questions related to UE radio capability for paging information. SA2 has provides the following feedback: RAN3 would like to ask SA2 that when the NG-RAN node receives the UE Radio Capability for Paging IE in the INITIAL UE CONTEXT SETUP REQUEST message, and if the UE Radio Capability for Paging Information does not contain paging information on all NG-RAN RATs that the UE supports, then should the NG-RAN node send to the AMF the paging information on the missing NG-RAN RATs or all NG-RAN RATs that the UE supports in the UE RADIO CAPABILITY INFO INDICATION message? SA2 Answer 1: SA2 considers that “UE Radio Capability for Paging” is an Information Element that will be stored in AMF and transferred between NG-RAN and AMF. The AMF never interprets the information in it, nor processes it. Hence SA2 believe that the RAN needs to send the information as a single IE. AMF replaces the old IE, if there is one stored, with the new received IE. NG-RAN is responsible to check and upload the proper information (e.g. gNB may include NR and additionally E-UTRA information but does not include NB-IoT) to AMF. NG-RAN shall not just provide “missing NG-RAN RAT”. RAN3 would also like to ask if the “that PLMN” in TS 23.501 clause 5.4.4.3 is interpreted as the serving PLMN? SA2 Answer 2: SA2 confirms that “that PLMN” can be interpreted as serving PLMN. 2. Actions: To RAN3 ACTION: SA2 kindly asks RAN3 to take above information into consideration and updates corresponding specs applicable.

This contribution provides our further considerations according the SA2 response and the CRs discussed at the previous meeting.

# 2. Discussion

This reply LS was triggered by the RAN3 LS [2] when the NG-RAN receives the UE Radio Capability for Paging IE in the INITIAL UE CONTEXT SETUP REQUEST message. The corresponding CRs were already discussed at the previous 127bis meeting.

| R3-252188 | Correction on UE radio capability for paging information (CMCC, Ericsson, Huawei, CATT, Qualcomm Inc., Vodafone, ZTE, Nokia) | CR1275r, TS 38.413 v16.16.0, Rel-16, Cat. F ZTE: The current spec describes that AMF shall replace the received information from RAN HW, Nok: Technically endorsed the CRs and check with SA2 Missing NG-RAN RAT(s) information or total NG-RAN RAT(s) information sent to AMF from NG-RAN? |
| --- | --- | --- |
| R3-252189 | Correction on UE radio capability for paging information (CMCC, Ericsson, Huawei, CATT, Qualcomm Inc., Vodafone, ZTE, Nokia) | CR1276r, TS 38.413 v17.11.0, Rel-17, Cat. A |
| R3-252190 | Correction on UE radio capability for paging information (CMCC, Ericsson, Huawei, CATT, Qualcomm Inc., Vodafone, ZTE, Nokia) | CR1277r, TS 38.413 v18.5.0, Rel-18, Cat. A |

According to the SA2 response, it clearly stated that when the NG-RAN checks the UE Radio Capability for Paging Information does not contain paging information on all NG-RAN RATs that the UE supports, it should upload all “UE Radio Capability for Paging” as a single Information Element, instead of just paging information on the missing NG-RAN RATs. Then the CRs above could make corresponding updates.

- Observation 1:  SA2 clearly stated that when the NG-RAN checks the UE Radio Capability for Paging Information does not contain paging information on all NG-RAN RATs that the UE supports, it should upload all “UE Radio Capability for Paging” as a single Information Element, instead of just paging information on the missing NG-RAN RATs.

In addition, it can be observed that in the INITIAL CONTEXT SETUP REQUEST message, the UE Radio Capability for Paging (URCP) can be included on the top level, or contained in the Core Network Assistance Information for RRC INACTIVE IE as excerpted below.

#### 9.3.1.15 Core Network Assistance Information for RRC INACTIVE This IE provides assistance information for RRC_INACTIVE configuration. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. UE Identity Index Value; M; 9.3.3.23; -. UE Specific DRX; O; Paging DRX 9.3.1.90; -. <Skip the irrelevance>. Extended UE Identity Index Value; O; 9.3.3.52; YES; ignore. UE Radio Capability for Paging; O; 9.3.1.68; YES; ignore.

For the latter case, this URCP was introduced in R3-212733 at RAN3-113 meeting as given as follows. It can be observed that the motivation is to avoid the URCP from being reconstructed at every instance during RAN paging if the URCP is not available at the anchor NG-RAN node (e.g., after the handover procedure).

| R3-211622 | Supporting use of UE Radio Capability for Paging in RRC_INACTIVE (Qualcomm Incorporated, ZTE, Ericsson, Nokia, Nokia Shanghai Bell, Deutsche Telekom, LG Electronics) | CR0547r1, TS 38.413 v16.5.0, Rel-16, Cat. F HW: which IEs cannot be created? QC: WUS support HW: does not apply to INACTIVE QC: RAN2 is looking into situations where this is needed HW: RAN2 is looking at this in Rel-17 QC: the argument can be made for Rel-16 – we should minimize the number of future problems E///: Anchor node should not have many problems, but this repeats a previous step VF: it would be nice to check which RAT this applies to E///: in NGAP we have signaling for NR/LTE etc. NEC: this might result in duplicate info in some messages Nok: TEI ID is needed only for Cat B for Rel-17, so does not apply here # 109_UEradioCapPagingINACTIVE - Rel-16 or future Rel-17? - any duplication of info? - check tdoc type etc. - add TEI identifier in title according to MCC guidance? (QC - moderator) rev in R3-212733 (+HW co-sign) Agreed 1623 rev in R3-212734 (+HW co-sign) Endorsed Summary of offline disc R3-212735 noted |
| --- | --- | --- |

This means the CN can provide the URCP on the top level, or in the Core Network Assistance Information for RRC INACTIVE IE, or even both.

In order for the NG-RAN to check the URCP as indicated in the SA2 reply LS, we suggest RAN3 to discuss also the URCP contained in the Core Network Assistance Information for RRC INACTIVE IE when introducing the newly added interactions with the UE Radio Capability Info Indication procedure, e.g., in case the top level URCP is not present. And the detailed wording can be discussed online, and make revisions towards the submitted CRs if any agreement.

RAN3 to discuss whether the newly added interactions with the UE Radio Capability Info Indication procedure should also consider the URCP contained in the Core Network Assistance Information for RRC INACTIVE IE, e.g., in case the top level URCP is not present.

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals.

- Observation 1:  SA2 clearly stated that when the NG-RAN checks the UE Radio Capability for Paging Information does not contain paging information on all NG-RAN RATs that the UE supports, it should upload all “UE Radio Capability for Paging” as a single Information Element, instead of just paging information on the missing NG-RAN RATs.

- RAN3 to discuss whether the newly added interactions with the UE Radio Capability Info Indication procedure should also consider the URCP contained in the Core Network Assistance Information for RRC INACTIVE IE, e.g., in case the top level URCP is not present.

If agreed, the detailed wording can be discussed online, and make revisions towards the submitted CRs.

# 4. Reference

- S2-2506082 Reply LS on Handling of UE Radio Capability for Paging, SA2

- R3-252022 LS on Handling of UE Radio Capability for Paging, RAN3

- R3-212735 Summary of Offline Discussion on use of UE Radio Capability for Paging in RRC_INACTIVE, Qualcomm Incorporated (moderator)