3GPP TSG-RAN WG3 Meeting #129												 R3-255679

Bengaluru, India, 25th ~29th Aug, 2025

Agenda Item:	19.2

Source:	Huawei, China Telecom, China Unicom

Title:	Further discussions on remaining open issues for SBFD

Document for: Discussion

1	Introduction

In the BL CR [1] to TS 38.423 for SBFD, there are some Editor’s note/FFS, this contribution is provided to address these Editor’s note/FFS.

2	Discussion

Issue 1: The Editor’s Note about procedure text

In section 8.4.y CLI Indication procedure, there is one Editor’s Note：

#### 8.4.y.2 Successful Operation Figure 8.4.y.2-1: CLI Indication, successful operation NG-RAN node1 initiates the procedure by sending the CLI INDICATION message to NG-RAN node2. If the CLI INDICATION message is received, the NG-RAN node2 may act as specified in TS38.300 [9]. Editor’s Note: The procedure text might be updated pending on further discussion.

For this Editor’s Note, we think the current procedure text is fine to specify the behaviour of receiving NG-RAN node, the Editor’s Note should be removed. And the message name in the figure should be updated accordingly.

Proposal 1: Remove the Editor’s Note in section 8.4.y CLI Indication procedure about the procedure text, and update the message name in Figure 8.4.6.2-1.

Issue 2: FFS about SBFD Configuration

As the field SBFD Configuration IE is still FFS in XnAP BL CR, and RAN2 has not yet completed the signaling design till last meeting. In RAN2#130 meeting, there was endorsed RRC running CR [2], in which the SBFD time and frequency configuration are given as the highlighted part:

TDD-UL-DL-Pattern ::= SEQUENCE { dl-UL-TransmissionPeriodicity ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10}, nrofDownlinkSlots INTEGER (0..maxNrofSlots), nrofDownlinkSymbols INTEGER (0..maxNrofSymbols-1), nrofUplinkSlots INTEGER (0..maxNrofSlots), nrofUplinkSymbols INTEGER (0..maxNrofSymbols-1), ..., [[ dl-UL-TransmissionPeriodicity-v1530 ENUMERATED {ms3, ms4} OPTIONAL -- Need R ]], [[ sbfd-StartingSlotIndex-r19 INTEGER (0..maxNrofSlots-1) OPTIONAL, -- Need R sbfd-StartingSymbolIndex-r19 INTEGER (0..maxNrofSymbols-1) OPTIONAL, -- Need R sbfd-EndingSlotIndex-r19 INTEGER (0..maxNrofSlots-1) OPTIONAL, -- Need R sbfd-EndingSymbolIndex-r19 INTEGER (0..maxNrofSymbols-1) OPTIONAL -- Need R ]] }

SCS-SpecificCarrier ::= SEQUENCE { offsetToCarrier INTEGER (0..2199), subcarrierSpacing SubcarrierSpacing, carrierBandwidth INTEGER (1..maxNrofPhysicalResourceBlocks), ..., [[ txDirectCurrentLocation INTEGER (0..4095) OPTIONAL -- Need S ]] [[ ul-subbandlocationAndBandwidth INTEGER (0..37949) OPTIONAL, -- Need R firstDLsubbandlocationAndBandwidth INTEGER (0..37949) OPTIONAL, -- Need R secondDLsubbandlocationAndBandwidth INTEGER (0..37949) OPTIONAL -- Need R ]] }

In RAN3#127 meeting, there was agreement about how to indicate the SBFD time and frequency configuration:

The SBFD time and frequency configuration is encoded as container defined by RAN2.

Currently, the TDD UL-DL Configuration Common NR IE and the Carrier List IE are included in Served Cell Information NR IE to indicate the legacy time and frequency configuration of the cell.

| >>>TDD UL-DL Configuration Common NR | O |  | OCTET STRING | Includes the tdd-UL-DL-ConfigurationCommon contained in the SIB1 message as defined in TS 38.331 [10] | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >>>Carrier List | O |  | NR Carrier List 9.2.2.63 | If included, the Transmission Bandwidth IE shall be ignored. | YES | ignore |

For SBFD time configuration, the TDD UL-DL Configuration Common NR IE is directly referred to the field tdd-UL-DL-ConfigurationCommon in TS 38.331 which will includes the SBFD time configuration (sbfd-StartingSlotIndex-r19, sbfd-StartingSymbolIndex-r19, sbfd-EndingSlotIndex-r19, sbfd-EndingSymbolIndex-r19). Thus, we can infer that the current XnAP specification already supports the exchange of SBFD time configuration naturally.

Observation 1: With the latest endorsed running CR in RAN2, the current XnAP specification already supports the exchange of SBFD time configuration.

For SBFD frequency configuration, we think it is better to include the entire scs-SpecificCarrierList field which is encoded to SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier to overwrite the Carrier List IE, considering that the current Carrier List IE in XnAP explicitly defines the explicit values of SCS, offset to Carrier and Carrier bandwidth. So, the newly introduced SBFD Configuration IE could be encoded as OCTET STRING form to include scs-SpecificCarrierList field in TS 38.331, and if included, the Transmission Bandwidth IE and the Carrier List IE shall be ignored.

As we could see that, with the latest agreement in RAN2, SBFD timing info is already included, but SBFD frequency info needs to be introduced in RAN3, since SBFD is a new feature, it would be better to reflect SBFD frequency and timing info in a standalone IE, thought SBFD timing info might be duplicated.

Proposal 2: The SBFD Configuration IE is encoded as two sub-IEs, both sub-IEs could be encoded as OCTET STRING, one referring to  scs-SpecificCarrierList IE defined in TS 38.331 for frequency info, and the other one referring to tdd-UL-DL-ConfigurationCommon for timing info.

Issue 3: The ambiguity about CRI

In the current version of XnAP and F1AP BL CRs, the NZP CSI-RS Resource Indication(CRI) is encoded to integer with value from 1 to 64, which is descripted as Strongest DL NZP CSI-RS beam information, as following:

XnAP:

| >>NZP CSI-RS Resource Indication | O |  | INTEGER (1..64) | Strongest DL NZP CSI-RS beam information | – |  |
| --- | --- | --- | --- | --- | --- | --- |

F1AP:

| >>CRI | O |  | INTEGER (1..64) | Strongest DL NZP CSI-RS beam information |
| --- | --- | --- | --- | --- |

And for the NZP CSI-RS Resources Configuration, which is encoded to OCTET STRING including the NZP-CSI-RS-Resource IE in RRC specification, in which the value of NZP-CSI-RS-ResourceId is an integer from 0 to 191:

#### – NZP-CSI-RS-ResourceId The IE NZP-CSI-RS-ResourceId is used to identify one NZP-CSI-RS-Resource. NZP-CSI-RS-ResourceId information element -- ASN1START -- TAG-NZP-CSI-RS-RESOURCEID-START NZP-CSI-RS-ResourceId ::= INTEGER (0..maxNrofNZP-CSI-RS-Resources-1) -- TAG-NZP-CSI-RS-RESOURCEID-STOP -- ASN1STOP

As can be seen, it is unclear what the value of CRI means, there would be ambiguity whether the CRI value is NZP-CSI-RS-ResourceIndex-r18 (integer from 0 to 63) or nzp-CSI-RS-ResourceId (integer from 0 to 191).

Observation 3: There is ambiguity for what value of CRI IE represent in current XnAP and F1AP BL CRs.

According to the LS from RAN1 [5], the CSI-RS Resource Indicator should be the relative index of the CSI-RS resources within a set of resources:

Strongest DL beam information in the context of NZP-CSI-RS is defined as a CSI-RS Resource Indicator (CRI) value. CRI is a relative index in the range [1 .. N], where N is the number of CSI-RS resources within a set of resources for which the configuration is provided to a gNB.

Proposal 3: Update the description of NZP CSI-RS Resource Indication IE in XnAP CR and CRI IE in F1AP CR to clarify the value is the relative index of the CSI-RS resources within a set of resources.

# 3	Conclusion

Based on the above discussion we have the following proposals:

Issue 1: The Editor’s Note about procedure text

Proposal 1: Remove the Editor’s Note in section 8.4.y CLI Indication procedure about the procedure text, and update the message name in figure to CLI INDICATION.

Issue 2: FFS about SBFD Configuration

Observation 1: With the latest endorsed running CR in RAN2, the current XnAP specification already supports the exchange of SBFD time configuration naturally.

Proposal 2: The SBFD Configuration IE is encoded as two sub-IEs, both sub-IEs could be encoded as OCTET STRING, one referring to scs-SpecificCarrierList IE for frequency info, and the other one referring to tdd-UL-DL-ConfigurationCommon for timing info, as defined in TS 38.331..

Issue 3: The ambiguity about CRI

Observation 3: There is ambiguity for what value of CRI IE represent in current XnAP and F1AP BL CRs.

Proposal 3: Update the description of NZP CSI-RS Resource Indication IE in XnAP CR and CRI IE in F1AP CR to clarify the value is the relative index of the CSI-RS resources within a set of resources.

And the TP to BL CR of TS 38.423 is given in Annex.

4	Reference

- R3-253992, (BL CR to 38.423) introduction of Evolution of NR duplex operation, Huawei, China Telecom, China Unicom, China Mobile, Ericsson, Qualcomm, Samsung, ZTE Corporation, LG Electronics, Nokia, Jio Platforms (JPL), CATT

- R2-2503862, RRC running CR for Evolution of NR duplex operation (SBFD), Huawei, HiSilicon