3GPP TSG-RAN WG3 Meeting #129	R3-255680

Bengaluru, India, 25th ~29th Aug, 2025

Title:	Discussion on introduction of SRS resource configurations for SBFD

Agenda Item:	19.2

Source:	Huawei, China Telecom, China Unicom

Document for:	Discussion and Approval

1	Introduction

During RAN#108, the WID was updated [1] with the addition of the following bullet:

In this contribution, we will discuss how to exchange one or more SRS resource configurations.

2	Discussion

The intention of the added bullet is to exchange the SRS resource configurations used by UEs configured with SBFD over Xn, so that neighbour base station could evaluate and mitigate the inter-UE CLI interference. According to the online discussions and offline discussions happened during and after RAN#108, it seems that there are different understandings about the “SRS resource configurations”, though the basic and common understanding points to the UL sounding resource configured to UE.

Basically, there are two options here, regarding the reference of “SRS resource configurations”:

Option 1: SRS resource configurations refer to “SRS-resource” within “SRS-RSRP-MeasResource”

#### – SRS-RSRP-MeasResource The IE SRS-RSRP-MeasResource specifies SRS-RSRP measurement resource based on SRS-Resource in SRS-Config, which the UE may be configured to measure L1-SRS-RSRP. SRS-RSRP-MeasResource information element -- ASN1START -- TAG-SRS-RSRP-MEASRESOURCE-START SRS-RSRP-MeasResourceSet-r19 ::= SEQUENCE { srs-RSRP-MeasResourceId-r19 SRS-RSRP-MeasResourceId-r19 OPTIONAL, -- Need R srs-Resource-r19 SRS-Resource OPTIONAL, -- Need R qcl-InfoPeriodicSRS-RSRP-MeasResource TCI-StateId OPTIONAL, -- Need R ... } -- TAG-SRS-RSRP-MEASRESOURCE-STOP -- ASN1STOP

Option 2: SRS resource configurations refer to “SRS-resource” within “SRS-Config”

#### – SRS-Config The IE SRS-Config is used to configure sounding reference signal transmissions. The configuration defines a list of SRS-Resources, a list of SRS-PosResources, a list of SRS-PosResourceSets and a list of SRS-ResourceSets. Each resource set defines a set of SRS-Resources or SRS-PosResources. The network triggers the transmission of the set of SRS-Resources or SRS-PosResources using a configured aperiodicSRS-ResourceTrigger (L1 DCI). The network does not configure SRS specific power control parameters alpha (without suffix) or pathlossReferenceRS if unifiedTCI-StateType is configured for the serving cell. SRS-Config information element -- ASN1START -- TAG-SRS-CONFIG-START SRS-Config ::= SEQUENCE { srs-ResourceSetToReleaseList SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSetId OPTIONAL, -- Need N srs-ResourceSetToAddModList SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSet OPTIONAL, -- Need N srs-ResourceToReleaseList SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-ResourceId OPTIONAL, -- Need N srs-ResourceToAddModList SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-Resource OPTIONAL, -- Need N tpc-Accumulation ENUMERATED {disabled} OPTIONAL, -- Need S ..., ////omit some text//// } SRS-ResourceSet ::= SEQUENCE { srs-ResourceSetId SRS-ResourceSetId, srs-ResourceIdList SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-ResourceId OPTIONAL, -- Cond Setup ////omit some text//// } SRS-Resource ::= SEQUENCE { srs-ResourceId SRS-ResourceId, nrofSRS-Ports ENUMERATED {port1, ports2, ports4}, ptrs-PortIndex ENUMERATED {n0, n1 } OPTIONAL, -- Need R ////omit some text//// }

As could be seen from above, and the contexts in RAN2, it is very clear that the defition of “SRS-resource” in option 1 and option 2 is different. For option 1, it is the measurement configuration, i.e. configured for the UE to measure L1-SRS-RSRP; while for option 2, it is the UL sounding resource for the UE to transmit SRS.

Based on the two different defintions, the intention of two options are different, in the architecture shown above, Option 1 intends to allow gNB A to tell gNB B to configure this sounding info for UEs under gNB B to measure; while option 2 just intends to allow gNB A to tells gNB B the UL sounding info used by UEs served by gNB A, for gNB B to take them into account.

Observation 1: The intention of two options are different:

- Option 1 intends to allow gNB A to tell gNB B to configure this sounding info for UEs under gNB B to measure;

- Option 2 intends to allow gNB A to tells gNB B the UL sounding info used by UEs served by gNB A.

From the observations above, we could see further difference. For option 1, gNB A tries to guide gNB B’s behaviour, and also tries to get the measurement results of gNB B back (understanding from offline discussion) to further evaluate and take potential actions, i.e the behaviour in gNB A and B are a bit dependent with each other, and interference mitigation behavoir in two gNBs might be expected; while for option 2, gNB A just tell gNB B the UL sounding info which is used by its serving UEs, such info is for gNB B to consider, i.e. the behaviour in gNB A and B is independent with each other, the interference mitigation behavoir in two gNBs are left to gNB implemention.

Observation 1bis: The gNB behaviour of two options are different, option 1 seems to introduce behaviour dependency between two gNBs, while option 2 tries to let the behaviour of two gNBs independent with each other and leave to gNB implementation upon reception of SRS resource.

However, as we could see that for either option 1 or option 2, we see commonalities, i.e. both options will contain SRS resource configuration which points to the IE “SRS-resource”, see below. In our understanding, to just exchange the minimum set of SRS resource should be a good compromise, i.e. introduce a new IE which includes list of SRS-Resource field, which directly refer to SRS-Resource IE as defined in TS 38.331.

For XnAP, the new IE could be introduced to Served Cell Information NR IE, as similar way as NZP-CSI-RS resource. the exactly content of the IE can be express as following, and the maximum number of SRS resource in one cell can be set to 320, this value may be sufficient to include all SRS resource used by the belonging UEs.

#### 9.2.2.x3	SRS Resources Configuration

This IE contains the SRS resources configuration in an NR cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SRS Resource List |  | 1 |  |  |
| >SRS Resource Item |  | 1..<maxnoofSRS-Resource> |  |  |
| >>SRS Resource | M |  | OCTET STRING | Includes the SRS-Resource IE, as defined in TS 38.331 [10]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofSRS-Resource | Maximum no. of SRS Resource. Value is 320. |

Proposal 1: Introduce new SRS Resource Configuration IE in Served Cell Information NR IE of XnAP, which includes a list of SRS Resource.

For F1AP impact, currently, the SRS-Resource configured to UE for UL transmission is determined by gNB-DU, and provided to gNB-CU in UE CONTEXT SETUP RESPONSE message or UE CONTEXT MODIFICATION RESPONSE message (see CellGroupConfig included in DU To CU RRC Information). So, we think the current F1AP specification has already supported to provide the SRS resource configuration of UE from gN-DU to gNB-CU.

Similarly, we think the SRS-RSRP-MeasResource configuration used for UE to measure SRS-RSRP is also determined by gNB-DU, and provided to gNB-CU in UE context management procedures (also in CellGroupConfig included in DU To CU RRC Information), i.e. the current F1AP specification has already supported to provide the SRS-RSRP measurement configuration of UE from gN-DU to gNB-CU.

However, to dedecide the SRS-RSRP measurement configuration, the gNB-DU needs to know the SRS-Resource of the neighbour gNB. the F1AP should be enhaced to support this, i.e., the gNB-CU shall provide the SRS-Resource configuration receieved from neighour gNB to the its gNB-DU in Neighbour-Node Cells List IE, and the design of new IE could be similar with XnAP, as following:

#### 9.2.2.x3	SRS Resources Configuration

This IE contains the SRS resources configuration in an NR cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SRS Resource List |  | 1 |  |  |
| >SRS Resource Item |  | 1..<maxnoofSRS-Resource> |  |  |
| >>SRS Resource | M |  | 9.3.1.193 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSRS-Resource | Maximum no. of SRS Resource. Value is 320. |

Proposal 2: Introduce new SRS Resource Configuration IE in Neighbour-Node Cells List IE of F1AP from gNB-CU to gNB-DU, which includes list of SRS Resource received from neighbour gNB.

We also noticed that there are some accompanying proposals, including:

- To put the SRS-Resource in the CLI Indication message, together with a “UE-UE CLI Indication”, the intention is  the node that receives the “UE-UE CLI Indication” will signal SRS resource configuration back, the claimed benefits are to help reducing the signalling of SRS resource which is sent only when a victim gNB sends out a “UE-UE CLI Indication”, comparing to the proposal of putting SRS-Resource in cell info where such info might be sent more frequently.

- Upon reception of SRS-Resource, the gNB could signal back the SRS measurement results (MeasResultCLI-r16), so that the sending node is able to learn the interference status better and take possible actions.

In our understanding, both proposals still intend to introduce some dependencies between gNBs, which seems to be unnecessary, we are open to discuss. From rapporteur’s perspective, we suggest to start from the minimut set, i.e. just to exchange SRS-Resource and include in cell info IE, over Xn and F1, and further discuss if any additions are needed during online discussion.

Corresponding TPs with minimum set to 38.423 and 38.473 could be seen in [2] [3].

3	Conclusion

This document proposes:

Observation 1: The intention of two options are different:

- Option 1 intends to allow gNB A to tell gNB B to configure this sounding info for UEs under gNB B to measure;

- Option 2 just intnents to allow gNB A to tells gNB B the UL sounding info used by UEs under gNB A.

Observation 1bis: The gNB behaviour of two options are different, option 1 seems to introduce dependency between two gNBs, while option 2 tries to let the behaviour of two gNBs independent with each other and leave to gNB implementation upon reception of SRS resource.

Proposal 1: Introduce new SRS Resource Configuration IE in Served Cell Information NR IE of XnAP, which includes a list of SRS Resource.

Proposal 2: Introduce new SRS Resource Configuration IE in Neighbour-Node Cells List IE of F1AP from gNB-CU to gNB-DU, which includes list of SRS Resource received from neighbour gNB.

4	Reference

[1]	RP-251874, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei

[2]	R3-255682, (TP to BL CR for 38.423) introduction of SRS resource configurations for Evolution of NR duplex operation Sub-band full duplex (SBFD), Huawei

[3]	R3-255683, (TP to BL CR for 38.473) introduction of  SRS resource configurations forEvolution of NR duplex operation Sub-band full duplex (SBFD), Huawei