3GPP TSG-RAN WG3 Meeting #129	R3-255690

Bengaluru, IN, 25 – 29 August, 2025

Agenda Item:	8.3

Source:	ZTE Corporation

Title:	Discussion on QoE MBS communication service types

Document for:	Discussions & Approval

1	Introduction

In previous meeting, RAN3 provided its view on SA4 defined @communicationServiceType IE for NR QoE to SA4. Before the start of this meeting, SA4 responsd its view with the modification for this IE shown in LS(R3-255028). In this contribution, we will further discuss the RAN3 impact based on SA4 modification and provide TPs to capture the potential agreements if necessary.

- 2	Background

RAN3 introduced the NR QoE for multicast and broadcast sevice at the end of Rel-18 with the MBS Communication Service Type IE shown below.:

==== MBS Communication Service Type IE defined in TS 38.413 ====

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MBS Communication Service Type | O |  | ENUMERATED (broadcast, multicast, ...) | This IE indicates the type of MBS communication service for which the QoE measurement collection should be performed. | YES | ignore |

==== MBS Communication Service Type IE defined in TS 38.413 ====

In received LS(R3-255028) from SA4, SA4 further modified the @communicationServiceType with additional codepoints after it received RAN3 LS(R3-250858).

==== @communicationServiceType IE defined in TS 26.247 ====

| Element or Attribute Name |  | Use | Description |
| --- | --- | --- | --- |
|  | @communicationServiceType | O | When present, this attribute indicates a list of communication service type(s) for which the collection and reporting of QoE metrics is requested, and shall contain one or more of the following values: - The value unicast refers to the common unicast communication type carrying media streaming services. - The value mbsMulticast refers to the MBS Multicast communication service per clause 21.1 of TS 38.300 [71]. - The value mbsBroadcast refers to the MBS Broadcast communication service per clause 21.1 of TS 38.300 [71]. - The value all refers to all communication service types. When absent, quality metrics collection is requested for all communication service types. |

==== @communicationServiceType IE defined in TS 26.247 ====

- 2	Discussion

Currently, 4 codepoints are included in this @communicationServiceType IE which can represent 4 types of communication serivce types: unicast, multicast, broadcast, and all. UE can only trigger the QoE measurement when UE is using specific communication service types.

Observation 1: The SA4 defined @communicationServiceType IE can represent 4 cases for 1 QoE configuration: unicast, multicast, broadcast, and all these 3 types of data transmission.

Meanwhile in RAN3 specifications, the IE RAN3 defined can represent 3 different types of communication service types: multicast, broadcast, and unicast(when this IE is absent). It is clear that there is a gap between SA4 definition and RAN3 definition for the communication service type for NR QoE. More specifically, there is no way for RAN3 to configure 1 QoE configuration which can be triggered by all defined communications service types.

Proposal 1: By using RAN3 defined mechanism, 1 QoE measurement can not be triggered by either unicast, multicast, or broadcast communication service type which does not fulfill SA4 requirement.

This issue may be solved by checking the following alternative:

- RAN3 changes nothing. This can be solved by implementation.

- Introduce new codepoint(e.g. all )for MBS Communication Service Type IE to align with SA4 definition.

- From our point of view, a new codepoint for communication service type is more gracefull than all based on implementation. We may introduce “all” codepoint for MBS Communication Service Type IE so that a QoE measurement can be triggered regardless of communciation service type. This is also sync up with SA4 IE name. In addition, further clarify when this IE is not present, it means the QoE measurement can only be triggered when UE is using unicast service.

- Proposal 2: Introduce new codepoint “all ” for MBS Communication Service Type IE.With this “all” for MBS Communication Service Type IE, the QoE measurement may be triggered when UE is using either unicast, multicast, or broadcast service for one QoE configuration.

- Proposal 3: Update semantic description for MBS Communication Service Type IE, without the presence of this IE, the QoE measurement can only be triggered by unicast service.

- CR for RAN3 specs has been provided to capture RAN3 potential progress and can be considered as baseline.

- In addition, RAN3 may send reply LS to SA4 with RAN3 understanding and updates for the communication service types. A draft reply LS has been prepared and can be considered as baseline.

- Proposal 4: Reply SA4 LS with RAN3 updates on QoE configuration.

- 3	Conclusion

Observation 1: The SA4 defined @communicationServiceType IE can represent 4 cases for 1 QoE configuration: unicast, multicast, broadcast, and all these 3 types of data transmission.

Proposal 1: By using RAN3 defined mechanism, 1 QoE measurement can not be triggered by either unicast, multicast, or broadcast communication service type which does not fulfill SA4 requirement.

- Proposal 2: Introduce new codepoint “all ” for MBS Communication Service Type IE.With this “all” for MBS Communication Service Type IE, the QoE measurement may be triggered when UE is using either unicast, multicast, or broadcast service for one QoE configuration.

- Proposal 3: Update semantic description for MBS Communication Service Type IE, without the presence of this IE, the QoE measurement can only be triggered by unicast service.

- Proposal 4: Reply SA4 LS with RAN3 updates on QoE configuration.