3GPP TSG-RAN WG3 Meeting #129	R3-255695

Bengaluru, IN, 25 – 29 August, 2025

Agenda Item:	8.3

Source:	ZTE Corporation

Title:	Discussion on UAV remaining issues

Document for:	Discussions & Approval

1	Introduction

RAN3 has discussed and made progress on how to report the aerial UE altitude information in previous meeting. The following remaining issues has left and will be further discussed this meeting:

- Aerial UE flight information reporting failed” indication in the Location Reporting Failure message. (FFS Option 1 for Failure Handling)

- Aerial UE flight information reporting failed” indication in the Location Report message and new cause value. (FFS Option 2 for Failure Handling)

- 2	Discussion

Based on the current mechanism for Location Reporting procedure(e.g. including Location Reporting Control, Location Reporting, and Location Reporting Failure Indication), only CN can cancel the location reporting normally. As specified in TS 38.413:

==== content in TS 38.413====

The AMF initiates the procedure by sending a LOCATION REPORTING CONTROL message to the NG-RAN node. On receipt of the LOCATION REPORTING CONTROL message the NG-RAN node shall perform the requested location reporting control action for the UE.

The Location Reporting Request Type IE indicates to the NG-RAN node whether:

-	to report directly;

-	to report upon change of serving cell;

-	to report UE presence in the area of interest;

-	to stop reporting at change of serving cell;

-	to stop reporting UE presence in the area of interest;

-	to cancel location reporting for the UE;

-	to report upon change of serving cell and to report UE presence in the area of interest.

==== content in TS 38.413====

Observation 1: Under normal conditions, NG-RAN shall not proactively cancel location reporting for a given UE.

In addition, 3GPP allows NG-RAN to send information to CN with the proper cause value when NG-RAN can not performing configured location reporting for a specific UE. Detail can be checked in TS 38.413:

==== content in TS 38.413====

The purpose of the Location Reporting Failure Indication procedure is to allow the NG-RAN node to inform the AMF that the location reporting request contained in the Location Reporting Control procedure, the Handover Resource Allocation procedure or the Initial Context Setup procedure has failed. The procedure uses UE-associated signalling.

==== content in TS 38.413====

Observation 2: The Location Reporting Failure Indication procedure is only used by NG-RAN when the configrued location reporting has failed.

Considering the enhancement of the aerial UE flight information reporting is based on the Location Reporting Procedure, it is clear that there is no scenario for NG-RAN to actively cancle the Location Reporting for aerial UE in normal case. Hence, no matter NG-RAN node refuses to report aerial UE altitude information either it receives this requirement from CN immediately or after it reports the aerial UE information for a while, this belongs to a kind of error or failure. From this point of view, we prefer to only enhance the Location Reporting Failure Indication for NG-RAN to indicate aerial UE altitude reporting is stopped.

Proposal 1: Enhance the NGAP Location Reporting Failure Indication messagefor NG-RAN to indicate aerial UE altitude reporting is stopped.

Furthermore, as we explained above, there is no difference when the reporting is stopped during the initiation of this procedure or during the ongoing reporting. Both of these cases belong to failure. To minimize the spec impact, we prefer to introduce a new cause value whcih is used to indicate NG-RAN node can not report the aerial UE information.

Proposal 2: Introduce a new cause value for Cause IE to indicate NG-RAN node can not report the aerial UE information.

At last, we also provide CR to capture our views shown above and can be considered as baseline.

- 3	Conclusion

Observation 1: Under normal conditions, NG-RAN shall not proactively cancel location reporting for a given UE.

Observation 2: The Location Reporting Failure Indication procedure is only used by NG-RAN when the configrued location reporting has failed.

Proposal 1: Enhance the NGAP Location Reporting Failure Indication messagefor NG-RAN to indicate aerial UE altitude reporting is stopped.

Proposal 2: Introduce a new cause value for Cause IE to indicate NG-RAN node can not report the aerial UE information.