3GPP TSG-RAN WG3 Meeting #129	R3-255700

Bengaluru, IN, August 25th – 29th, 2025

Title: 	(TP to BLCR for TS 38.470) F1-U function for AIML

Source: 	CMCC

Agenda item: 	11.4

Document Type:	Other

- Introduction

In RAN3#127-bis meeting, it has been agreed that:

Packet delay measured for UE Performance feedback is sent from DU to CU via ASSISTANCE INFORMATION DATA frame over F1-U in the case of CU-DU split architecture.

And in RAN3-128 meeting, BL CR to TS 38.425 has been agreed to add the detailed protocol description.

However, in stage 2 TS 38.470 for F1, the assistance information reported to gNB-CU is never mentioned as a F1-U function. It is useful to add the function description.

- TP to TS 38.470

<< Start of the change>>

5.3	F1-U functions

5.3.1	Transfer of user data

This function allows to transfer of user data between gNB-CU and gNB-DU.

This function also allows to transfer PDU Set Information of a QoS flow, and indication of End of Data Burst to the gNB-DU. The detailed protocol is specified in TS 38.415 [12].

5.3.2	Flow control function

This function allows to control the downlink user data flow to the gNB-DU. The detailed protocol is specified in TS 38.425 [7].

5.3.3      Assistance Information function

This function allows gNB-DU to provide assistance information to gNB-CU, for QoS monitoring, or AI/ML data collection. The detailed protocol is specified in TS 38.425 [7].

<< end of change>>

- Reference

- TS 38.470, F1 General Description

- RAN3_128_agenda_20250523_1530_EOM, chairnote