3GPP TSG-RAN WG3 #129	              R3-255706

Bengaluru, India, 25th - 29th Aug, 2025

Agenda Item:	18.2

Source:	CMCC

Title:	Discussion on Paging Capability Loss Issue

Document for:	Discussion

- Introduction

In RAN2#129bis, the following LS is sent to SA2, RAN3 and CT1 [1]:

RAN2 further discussed the issue of UE radio paging capability loss mentioned in the LS S2-2412876 and acknowledged that based on the legacy signalling, a gNB implementing Release X may not be able to forward the radio paging capabilities for a feature introduced after Release X, which results in UE missing the paging message. For UE Radio Paging Capabilities introduced up to Rel-18, RAN2 agreed not to introduce any standard solution, i.e., rely on proper network implementation, as follows. We will not change legacy UE capabilities and rely on network implementation To address the issue for UE Radio Paging Capabilities introduced in Rel-19 onwards, RAN2 made the following agreement: Use a “container” using OCTET STRING with content generated by UE when new UE Radio Paging Capabilities are introduced. This is applicable only to new Rel-19 and future capabilities added to paging capabilities. This will only work for gNB that support this Rel-19 “container” In particular for Rel-19 LP-WUS feature, if it is deployed, all the gNBs belonging to a certain area need to support the Rel-19 “container”.

The following response was given at the recent SA2-169 meeting [2]:

1. Overall Description: SA2 thanks RAN2 for this LS and their earlier LS in R2-2501388=S2-2504576. In the latest LS, RAN 2 state that: In particular for Rel-19 LP-WUS feature, if it is deployed, all the gNBs belonging to a certain area need to support the Rel-19 “container”. The 5GS design means that the UE Radio Capability for Paging is loaded into the AMF only in the cell where the UE attaches (does “initial registration”) or first moves from LTE to NR. After that the UE can move throughout the whole PLMN and normally there is no event that triggers any update of the UE Radio Capability for Paging information stored in the AMF. Hence the “certain area” mentioned by RAN 2 is the whole NR coverage of that PLMN. It is essential that operators are able to use RAN equipment from different vendors and that competition between those RAN vendors is not inhibited by RAN 2 requiring all RAN vendors to upgrade their equipment before new UE functionality can be used with the first RAN vendor to develop a solution. Hence this RAN2 solution is highly undesirable compared to what was proposed in RAN2’s earlier LS in R2-2501388=S2-2504576. SA2 are investigating whether a solution that builds on the current process in TS 23.501 clause 5.4.4.3 for handling UEs that support both “LTE connected to 5GC” and NR can be used to solve the problem. For example, when (as already specified) the UE Radio Capability for Paging is sent in the NGAP Initial UE Context Setup Request to the gNB, the gNB could use the UE Radio Access Capabilities (received in the same message) to check that the UE Radio Capability for Paging contains the full information on paging related features. If the gNB detects an omission, then the gNB should ensure that an update of the UE Radio Capability for Paging Information is sent to the AMF. 2. Actions: To RAN3, RAN2 SA2 asks RAN3 and RAN2 to provide views on the potential solution described above.

This contribution provides our views on the paging loss issue and reply to SA2.

- Discussion

First, the issue of missing NG-RAN RAT UE radio capability for paging information has been fully discussed in the current RAN3 NGAP CR and has been resolved in conjunction with SA2 replied in [3].

| R3-252188 | Correction on UE radio capability for paging information (CMCC, Ericsson, Huawei, CATT, Qualcomm Inc., Vodafone, ZTE, Nokia) | CR1275r, TS 38.413 v16.16.0, Rel-16, Cat. F ZTE: The current spec describes that AMF shall replace the received information from RAN HW, Nok: Technically endorsed the CRs and check with SA2 Missing NG-RAN RAT(s) information or total NG-RAN RAT(s) information sent to AMF from NG-RAN? |
| --- | --- | --- |
| R3-252189 | Correction on UE radio capability for paging information (CMCC, Ericsson, Huawei, CATT, Qualcomm Inc., Vodafone, ZTE, Nokia) | CR1276r, TS 38.413 v17.11.0, Rel-17, Cat. A |
| R3-252190 | Correction on UE radio capability for paging information (CMCC, Ericsson, Huawei, CATT, Qualcomm Inc., Vodafone, ZTE, Nokia) | CR1277r, TS 38.413 v18.5.0, Rel-18, Cat. A |

Note that the latest CR text regarding UE radio capability for paging information handling is as follows:

Interactions with UE Radio Capability Info Indication procedure: If the UE Radio Capability for Paging IE is included in the INITIAL CONTEXT SETUP REQUEST message, which does not include UE radio capability for paging of all the NG-RAN RAT(s) that the UE supports, the NG-RAN node shall, if supported, send the UE RADIO CAPABILITY INFO INDICATION message to the AMF containing the UE radio capability for paging of all the NG-RAN RAT(s) the UE supports in the serving PLMN.

Furthermore, in the latest response to the paging capability loss issue, SA2 proposed that the full information on paging related features should be further checked and updated. However, we believe that this does not solve the problem of LP-WUS capability loss between different release of NG-RAN nodes.

Currently, in TS 38.331, UERadioPagingInformation is generated or derived by the gNB, uploaded to the CN, and then sent to the gNB when paging the UE. That is, the SA2 solution still cannot solve the LP-WUS capability loss problem in the following two cases:

- Problem case 1: The UE initially accesses a pre-R19 gNB and then switches to an R19 gNB

In this scenario, a pre-R19 gNB is unable to recognize the LP-WUS feature, so the UE's LP-WUS capability information is lost. Consequently, after a handover to a R19 gNB, the new gNB will also not know that the UE has LP-WUS capability, unless the network triggers a UE radio capability enquiry again—an action the NW rarely performs.

- Problem case 2: The UE initially accesses an R19 gNB, then switches to a pre-R19 gNB, and finally switches to another R19 gNB.

In this scenario, while the initial R19 gNB and the CN know that the UE has LP-WUS capability, the pre-R19 gNB is unable to recognize this capability. Consequently, after a second handover to another R19 gNB, problems may still arise if RAN paging is executed before CN paging, even though CN paging can update paging related features using the SA2 method.

Therefore, our view is that the paging capability loss issue can only be solved through network deployment.

Observation 1: SA2's method cannot solve the paging capability loss issue.

Proposal 1: RAN3 confirmed RAN2's conclusion that LP-WUS deployment relies on proper network implementation.

- Conclusions

In this contribution, we discussed paging capability loss issue in RRC_IDLE/INACTIVE mode, and following are our observation and proposal.

Observation:

Observation 1: SA2's method cannot solve the paging capability loss issue.

Proposal:

Proposal 1: RAN3 confirmed RAN2's conclusion that LP-WUS deployment relies on proper network implementation.

- References

- R2-2503172 Reply LS on paging capability loss issue

- S2-2506087 Response to Reply LS on paging capability loss issue

- S2-2506082 Reply LS on Handling of UE Radio Capability for Paging