3GPP TSG-RAN WG3 Meeting #129	             R3-255710

Bengaluru, India, Aug 25th- Aug 29th, 2025

Agenda Item:	16.2

Source:	CMCC

Title:	Discussion on Session Release Aspects

Document for:	Other

# 1	Introduction

In RAN3#128 meeting, the discussion on Session Release Aspects for Ambient IoT has made following agreements and remaining FFSs.

Session Release Aspects: Introduce new CN triggered Class 1 Session Release procedure, which includes A-IoT SESSION RELEASE REQUEST message and A-IoT SESSION RELEASE RESPONSE message. Introduce a class2 procedure to allow NG-RAN node to trigger Session Release procedure towards AIoT CN. In the case of Inventory only scenario, introduce Inventory Complete indication to inform the AIoT CN about the complete of the triggered Inventory session. FFS on the detail about Inventory Complete indication via new procedure or introducing indication IE in current Inventory Report message. In the case of Command after Inventory scenario, whether need to introduce the complete indication needs to be further checked.

In this contribution, we will provide our views on Session Release Aspects for Ambient IoT.

# 2	Discussion

In last meeting, companies reached the consensus to introduce two procedures: the CN-triggered A-IoT Session Release procedure and the gNB-triggered Session Release procedure. The CN-triggered A-IoT Session Release procedure is agreed to introduce. While, the gNB-triggered Session Release procedure, currently named the A-IoT Session Release Request procedure, allows the gNB to request the A-IoT CN node to release the A-IoT session.

The intention behind the A-IoT Session Release Request procedure is to notify the A-IoT CN to release the dedicated A-IoT session. Upon reception of the AIOT SESSION RELEASE REQEUST message, the A-IoT CN node should initiate the A-IoT Session Release procedure for the session denoted by the AIOTF Identifier IE and the Correlation ID contained in the AIOT SESSION RELEASE REQUEST message.

Introducing the A-IoT Session Release Request procedure provides the flexibility of terminating the dedicated A-IoT session. Once the gNB determines that the current A-IoT session is completed or be unable to complete the A-IoT session due to some reasons, it can trigger the A-IoT Session Release Request procedure and await the A-IoT Session Release procedure to release the device context over NG interface and the radio resource over A-IoT Uu interface. Once the A-IoT Session Release procedure is completed, the gNB can start a new A-IoT session after the previous A-IoT session is released.

Observation 1: Introducing the A-IoT Session Release Request procedure provides the flexibility of terminating the dedicated A-IoT session.

Proposal 1: The intention behind the A-IoT Session Release Request procedure is to notify the A-IoT CN to release the dedicated A-IoT session.

In the case of Inventory only scenario, RAN3 agreed to introduce Inventory Complete indication to inform the A-IoT CN about the triggered Inventory session complete with no further Inventory Report messages. If the reader does not receive any A-IoT device ID(s) over the A-IoT Uu interface, gNB can send the Inventory Complete indication to the A-IoT CN.

Observation 2: In the case of Inventory only scenario, RAN3 agreed to introduce Inventory Complete indication to inform the A-IoT CN about the triggered Inventory session complete with no further Inventory Report messages.

In the case of Command after Inventory scenario, it is still undecided in RAN3 that whether to introduce the Inventory Complete indication and Command Complete indication.

Regard to the Command Complete indication, the Command Request procedure is a per single device procedure, the A-IOT CN can distinguish whether the command operation for single A-IoT device is successful by receiving COMMAND RESPONSE message and COMMAND FAILURE message. If the command operation is successful, the A-IOT CN retrieves the command response carried in A-IoT NAS PDU within Command Response Transfer IE. As awareness of the command result, the A-IoT CN triggers Session Release procedure with whole things cleaned up. Consequently, the per device Command Complete indication is not required.

Proposal 2: In the case of Command after Inventory scenario, the per device Command Complete indication is not required.

With regard to the Inventory Complete indication in the Command after Inventory scenario, its effect shows some similarities to that in the Inventory only scenario. Upon reception of the Inventory Complete indication from the gNB, the A-IoT CN shall consider the triggered Inventory session complete and assume that no remaining uninventorized A-IoT devices are within its coverage. Since the command procedure does not wait for collection of all A-IoT devices through the A-IoT Inventory Report procedure, this indication shortens the overall Command after Inventory procedure. Shortly after receiving this indication, if the A-IoT CN determines that command results for all devices have been collected, it can trigger the A-IoT Session Release procedure to release the context in both the A-IoT CN and the gNB, avoiding the need to wait for the timer expiration to trigger the A-IoT Session Release procedure.

Observation 3: The effect of the Inventory Complete indication in the Command after Inventory scenario shows some similarities that in the Inventory only scenario.

Proposal 3: In the case of Command after Inventory scenario, propose to introduce the Inventory Complete indication.

As the Inventory Complete indication is already agreed to introduce above two cases, the way of providing Inventory Complete indication from gNB to A-IOT CN is still undecided. Currently, three options are on the table:

Option 1: Providing the Inventory Complete indication via new procedure;

Option 2: Providing the Inventory Complete indication via Inventory Report procedure;

Option 3: Providing the Inventory Complete indication via the A-IoT Session Release Request procedure with procedure name changed;

These three options are theoretically feasible to provide the Inventory Complete indication from A-IOT CN, but the Option 1 is not recommended since a new procedure that only transmits one indication seems to be unnecessary. Both Option 2 and Option 3 require the updates on the existing A-IoT procedure to include the Inventory Complete indication, which changes the purpose of INVENTORY REPORT message or AIOT SESSION RELEASE REQUEST message.

Observation 4: Providing the Inventory Complete indication via new procedure is not recommended since a new procedure that only transmits one indication seems to be unnecessary.

Observation 5: Necessary updates on existing A-IoT procedure to include the Inventory Complete indication as it changes the purpose of INVENTORY REPORT message or AIOT SESSION RELEASE REQUEST message.

If the Option 2 is adopted, the Inventory Complete indication needs to be included in Inventory Report Transfer IE where the presence of Reader Report List IE should be changed from mandatory to optional. If the Option 3 is adopted, the A-IoT Session Release Request procedure should be renamed to take into account the Command after Inventory scenario. Difference from the Inventory only scenario, A-IoT CN does not immediately release the A-IoT session upon receiving the Inventory Complete indication. Thus, there will be two IEs respectively used to indicate different meaning corresponding to two scenarios. One IE is used to request the A-IoT CN to release the A-IoT session, the other IE is used to inform the A-IoT CN the triggered Inventory session complete. Given that both options are considered to be feasible, in terms of the extent of protocol changes, we slightly prefer Option 2.

Proposal 4: Slightly support to provide the Inventory Complete indication via Inventory Report procedure.

# 3	Conclusion

Based on the discussions mentioned above, in this contribution we provide following observations and proposals on Session Release Aspects for Ambient IoT:

Observation 1: Introducing the A-IoT Session Release Request procedure provides the flexibility of terminating the dedicated A-IoT session.

Observation 2: In the case of Inventory only scenario, RAN3 agreed to introduce Inventory Complete indication to inform the A-IoT CN about the triggered Inventory session complete with no further Inventory Report messages.

Observation 3: The effect of the Inventory Complete indication in the Command after Inventory scenario shows some similarities that in the Inventory only scenario.

Observation 4: Providing the Inventory Complete indication via new procedure is not recommended since a new procedure that only transmits one indication seems to be unnecessary.

Observation 5: Necessary updates on existing A-IoT procedure to include the Inventory Complete indication as it changes the purpose of INVENTORY REPORT message or AIOT SESSION RELEASE REQUEST message.

Proposal 1: The intention behind the A-IoT Session Release Request procedure is to notify the A-IoT CN to release the dedicated A-IoT session.

Proposal 2: In the case of Command after Inventory scenario, the per device Command Complete indication is not required.

Proposal 3: In the case of Command after Inventory scenario, propose to introduce the Inventory Complete indication.

Proposal 4: Slightly support to provide the Inventory Complete indication via Inventory Report procedure.

# 4	Reference

[1] RP-250796, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA;

[2] 3GPP TS 23.369 v0.3.0, Architecture support for Ambient power-enabled Internet of Things; Stage 2 (Release 19);