3GPP TSG-RAN WG3 Meeting #129														       R3-255718

Bengaluru, India, 25th – 29th August 2025

Agenda Item:	9.2

Source:	Ericsson, Lenovo, Google, Jio Platforms, CATT, Ofinno, Verizon Wireless

Title:	Essential corrections to TCI states for LTM

Document for:	Discussion, Agreement

# Introduction

In this contribution, we discuss a correction for LTM concerning TCI state configuration.

# Discussion

In LTM, the network needs to send in the LTM cell switch command a TCI state for the target cell which the UE can use. If no TCI state list is configured for an LTM candidate, an LTM cell switch MAC CE cannot be executed by the UE since the UE would not have any TCI state to refer to and the LTM execution fails.

In Rel-18 solution for LTM, the TCI states configurations are only included when early synchronization is requested. Bothe the request for Early Synchronization and the corresponding response are optional in the current signaling:

| Early Sync Information Request |  | 0..1 |  |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >Request for RACH Configuration | M |  | ENUMERATED (true, …) |  | - |  |

| Early Sync Information |  | 0..1 |  |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >TCI States Configurations List | M |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| >Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |

Still, TCI states are required during a cell switch (since the LTM cell switch MAC CE includes a TCI state which the UE should use). To ensure TCI state information is always available for cell switch, it should be included regardless of early synchronization information.

Currently the RRC LTM-Candidate IE already contains this information. To ensure future-proofing in case RAN2 decides that more details for an LTM candidate are needed, we propose adding a new LTM Candidate Configuration IE in the F1AP messages, referencing it. However, that there could be a concern about whether any additional information within the LTM-Candidate needs to be reviewed. To keep changes to a minimum in Rel-18, we propose adding another TCI States Configurations IE to the LTM Information.

With that, a candidate DU can provide the needed information in case of inter-DU LTM. Furthermore, to pass this information to the source DU, and align all the candidate DUs for subsequent LTM, we propose to add similar content in the UE CONTEXT MODIFICATION REQUEST and UE CONTEXT MODIFICATION RESPONSE messages.

- Include a new TCI States Configurations IE in the LTM Configuration IE in the UE CONTEXT SETUP RESPONSE message.

- Include a new LTM TCI States Configurations List IE in the UE CONTEXT MODIFICATION REQUEST message.

- Include a new TCI States Configurations IE in the LTM Configuration IE in the UE CONTEXT MODIFICATION RESPONSE message.

- Agree to the CR in R3-255719 that captures the above proposals.

# Conclusions and Proposals

In this contribution we made the following proposals.

Proposal 1	Include a new TCI States Configurations IE in the LTM Configuration IE in the UE CONTEXT SETUP RESPONSE message.

Proposal 2	Include a new LTM TCI States Configurations List IE in the UE CONTEXT MODIFICATION REQUEST message.

Proposal 3	Include a new TCI States Configurations IE in the LTM Configuration IE in the UE CONTEXT MODIFICATION RESPONSE message.

Proposal 4	Agree to the CR in R3-255719 that captures the above proposals.