3GPP TSG-RAN WG3 Meeting #129	R3-255724

Bengaluru, India, 25th – 29th August 2025

Agenda Item:	13.2

Source:	Ericsson, Jio Platforms, Verizon Wireless, ZTE, Ofinno

Title:	Support for Semi-persistent CSI-RS transmission

Document for:	Discussion, Agreement

# Introduction

At the last RAN3 meeting, in relation to the network coordination for semi-persistent CSI-RS transmission in a candidate cell, the following progress was made:

WA: RAN3 agrees that, for both F1AP and XnAP, the activation and deactivation of CSI-RS transmission in LTM candidate cells are performed at the level of individual CSI-RS Resource IDs.

Reuse the CSI-RS coordination procedure over F1AP and XnAP for source gNB/gNB-DU to activate or deactivate the SP CSI-RS resource for CSI acquisition in candidate cell.

The SP CSI-RS activation/(de)activation for the inter-CU LTM in DC is deprioritized in rel-19.

FFS on whether to add a new IE for SP CSI-RS resource for CSI acquisition in the corresponding procedure of SP CSI-RS resource for L1 RSRP measurement in F1AP and XnAP.

FFS on whether gNB-DU/gNB provides the report type (periodic or semi-persistent) of the CSI-RS resources in both F1AP and XnAP.

In this paper, we provide further discussion on this topic and the signaling details of the solution.

# Discussion

## Granularity for CSI-RS coordination

The network coordination for semi-persistent CSI-RS transmission is supported with the newly introduced procedures:

- F1AP DU-CU CSI-RS Coordination

- F1AP CU-DU CSI-RS Coordination

- XnAP CSI-RS Coordination

As mentioned, at the last RAN3 meeting the following WA was captured:

WA: RAN3 agrees that, for both F1AP and XnAP, the activation and deactivation of CSI-RS transmission in LTM candidate cells are performed at the level of individual CSI-RS Resource IDs.

At this meeting, RAN3 received an LS from RAN2 in R3-255011, concerning SP CSI-RS activation/deactivation for LTM. In the LS, RAN2 informs RAN3 about the following agreements made at RAN2#130:

- UE deactivates SP CSI-RS resource of candidate cells (other than the target cell) after cell switch. FFS on the target cell.

- Instead of candidate cell id and SP CSI-RS resource set id, LTM-CSI-ResourceConfigId is included into SP CSI-RS activation/deactivation MAC CE.

In RRC, the LTM-CSI-ResourceConfig is defined as follows:

LTM-CSI-ResourceConfig-r18 ::=      SEQUENCE {

ltm-CSI-ResourceConfigId-r18    LTM-CSI-ResourceConfigId-r18,

ltm-SSB-ResourceSet-r18         LTM-SSB-ResourceSet-r18,

...,

[[

ltm-NZP-CSI-RS-ResourceSet-r19   LTM-NZP-CSI-RS-ResourceSet-r19                     OPTIONAL, -- Need R

ltm-CSI-IM-ResourceSet-r19       LTM-CSI-IM-ResourceSet-r19                         OPTIONAL, -- Need R

resourceType-r19                 ENUMERATED {periodic, semiPersistent}              OPTIONAL  -- Cond CSI-RS

]]

}

LTM-SSB-ResourceSet-r18 ::=     SEQUENCE {

ltm-SSB-ResourceList-r18      SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF SSB-Index,

ltm-CandidateIdList-r18       SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF LTM-CandidateId-r18,

...

}

LTM-NZP-CSI-RS-ResourceSet-r19 ::=     SEQUENCE {

ltm-CSI-RS-ResourceList-r19    SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF NZP-CSI-RS-ResourceId,

ltm-CandidateIdList-r19        SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF LTM-CandidateId-r18,

...

}

The F1AP and XnAP CSI-RS coordination procedures being specified should take this into account. In particular, to reflect the RAN2 agreements, the NR CGI indicating the candidate cell can be removed, and the FFS on the granularity (currently indicated as CSI-RS Resource ID) can be resolved by changing it to CSI Resource Configuration ID. Finally, the encoding of CSI Resource Configuration ID should consider the RRC encoding for this, i.e., an INTEGER in the range from 0 to 111:

CSI-ResourceConfigId ::=            INTEGER (0..maxNrofCSI-ResourceConfigurations-1)

maxNrofCSI-ResourceConfigurations-1     INTEGER ::= 111     -- Maximum number of resource configurations minus 1

- For both F1AP and XnAP, the activation and deactivation of CSI-RS transmission in LTM candidate cells are performed at the level of CSI Resource Configuration ID.

- For both F1AP and XnAP, remove the NR CGI IE in the CSI-RS coordination procedures.

## CSI resources for CSI acquisition

In this section we analyse the network signaling needed to support CSI resources for CSI acquisition (or Early CSI acquisition) looking separately to the two phases involved: the Preparation and the Activation/Deactivation.

### Preparation

During the preparation, the source CU needs to receive information from its underlying DUs and from other CUs about the configuration of SP CSI resources. For this phase, there were a few open issues left from the last RAN3 meeting:

FFS on whether to add a new IE for SP CSI-RS resource for CSI acquisition in the corresponding procedure of SP CSI-RS resource for L1 RSRP measurement in F1AP and XnAP.

FFS on whether gNB-DU/gNB provides the report type (periodic or semi-persistent) of the CSI-RS resources in both F1AP and XnAP.

We note that in the running RRC CR, there is only one CSI-RS resource configuration, defined by the ltm-NZP-CSI-RS-ResourceToAddModList IE within the LTM-Candidate IE. Also, according to the current F1 and Xn baselines, the CU can request the CSI-RS resource configuration with the Request for CSI-RS Resource Configuration IE, and the DU (or the candidate DU) can provide it in the CSI-RS Resource Configuration IE. In RRC there is no distinction for the CSI-RS resource configuration used for L1 measurements or for CSI acquisition. The CSI resource configuration includes the CSI-RS resource configuration, and the network signaling can support transferring this information if we change the Request for CSI-RS Resource Configuration to Request for CSI Resource Configuration IE and the corresponding response from CSI-RS Resource Configuration IE to CSI Resource Configuration IE (ref R3-255269). With this changes, we don’t think any new IE is needed in F1 and Xn, and we propose to close the FFS.

- There is no need to add a new IE for SP CSI-RS resource for CSI acquisition in the corresponding procedure of SP CSI-RS resource for L1 measurement in F1AP and XnAP.

Let’s now discuss the network signaling needed to support the “CSI-RS report configuration”. To our knowledge, the RRC encoding for the CSI-RS report configuration for CSI acquisition and for the CSI-RS report configuration for L1 measurements will be different. The source CU receives the CSI-RS report configuration for L1 measurements in the LTM-CSI-ReportConfig IE, included in the CellGroup (CellGroupConfig-> SpCellConfig-> ServingCellConfig->CSI-MeasConfig-> LTM-CSI-ReportConfig), but does not receive the CSI-RS report configuration for CSI acquisition. Since the RRC encoding of the two pieces of information is different, we propose to add a new IE to transfer this inform from candidate DU to source/candidate CU (e.g., a CSI-RS report configuration for CSI acquisition IE in the UE CONTEXT SETUP RESPONSE message and a similar IE in Xn (in the HANDOVER REQUEST ACKNOWLEDGE message).

- Add a new IE (e.g., CSI-RS report configuration for CSI Acquisition) in the UE CONTEXT SETUP RESPONSE F1AP messages.

- Add new IE (e.g., CSI-RS report configuration for CSI Acquisition) in the HANDOVER REQUEST ACKNOWLEDGE message.

An addition aspect, also concerning the Preparation, which we think is worth to clarify, is whether there is a need for an explicit request (from source CU to candidate CU, and from source/candidate CU to a candidate DU), asking to provide the CSI-RS report configuration for CSI acquisition. According to the LS received at the last meeting from RAN1 (R3-253008 / R1-2503079) the support of semi-persistent CSI-RS resource for CSI acquisition is subject to separate UE capability. This UE capability will be known at the candidate DUs, since the source CU sends it to candidate CU in the HANDOVER REQUEST message, and the source/candidate CU can send it to the candidate DUs in the UE CONTEXT SETUP REQUEST (as part of UE-CapabilityRAT-ContainerList included in CU to DU RRC Information IE). Therefore, a candidate DU can decide whether it wants the UE to be configured for Early CSI acquisition of a candidate cell. If so, without being explicitly requested, the DU can prepare the CSI-RS report configuration for CSI acquisition, and send it to its CU.

- A candidate DU knows if the UE support SP CSI-RS resources for CSI acquisition.

- There is no need for an explicit request to provide the CSI-RS report configuration for CSI acquisition.

### Activation/Deactivation of SP CSI Resources

The activation/deactivation is practically implemented with the new CSI-RS Coordination procedures. The source DU requests for activation/deactivation of certain CSI-RS resources and the candidate DU/CU can accept the request or not. In relation to this phase, the following progress was made at the last RAN3 meeting:

Reuse the CSI-RS coordination procedure over F1AP and XnAP for source gNB/gNB-DU to activate or deactivate the SP CSI-RS resource for CSI acquisition in candidate cell.

When RAN3 started to discuss the CSI-RS Coordination procedures, only one possible use of the CSI-RS resources was on the table. Now – according to the agreement above - there are two possible uses, and we think this creates an problem that needs to be solved. With the CSI-RS coordination procedures, a candidate CU (or a candidate DU) receives from the source CU (or from a candidate CU) the request to activate/deactivate certain CSI-RS resources. These resources are “owned” by the candidate DU, so we think that, before the candidate DU takes the decision to activate them, it should know for which purpose they will be used: for L1 measurements, or for CSI acquisition? In the current signaling the request for activation/deactivation is generic (it does not tell the purpose).

We think RAN3 needs to resolve this issue, i.e. the candidate gNB (DU) should know for which purpose the CSI-RS resources will be used, so that it can take proper decisions, and we propose to discuss the following two options:

- Option 1: activation/deactivation is per CSI-RS resource and the CSI-RS resources are “grouped” in two lists: one list refers to the CSI-RS resources that will be used for L1 measurements, and another list refers to the CSI-RS resources that will be used for CSI acquisition.

- Option 2: activation/deactivation is per CSI-RS resource, and a new IE is added in the CSI-RS Coordination Request (both in F1 and Xn), to clarify for which purpose the resources will be used.

Both options allow to maintain an activation/deactivation of CSI-RS transmission per CSI-RS Resource ID. With Option 1 it is possible to use only one coordination for two different uses of CSI-RS resources, at the expense of a small restructuring in the signaling. With Option 2 the current baseline remains the same, but the source DU wants to use different CSI-RS resources for two different purposes, two coordination will be needed, which increases the amount of signalling.

- RAN3 to discuss the following options to resolve the ambiguity of using the same CSI-RS Coordination procedures for L1 measurements and for CSI acquisition: 
Option 1) Activation/deactivation is per CSI Resource Configuration and the Semi persistent CSI resources are split in two list, one for L1 measurements, one for CSI acquisition.
Option 2) Activation/deactivation is per CSI Resource Configuration and a new IE indicates for which purpose all the resources are used.

An example of Xn signaling for the two options is shown in the tables below.

Option 1

9.1.x.x7 	CSI-RS COORDINATION REQUEST

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 UE XnAP ID | M |  | NG-RAN node1 UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| NG-RAN node2 UE XnAP ID | M |  | NG-RAN node2 UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | reject |
| CSI Resource Request | M |  |  |  | YES | reject |
| >Layer 1 Measurements List |  | 0..1 |  |  | – |  |
| >>Layer 1 Measurements Item |  | 1 .. <maxnoofCSIResourceConfigurationsPlus1> |  |  | – |  |
| >>>Request For Semi Persistent CSI Resources | M |  | ENUMERATED(activate, deactivate, …) |  | – |  |
| >>>CSI Resource Configuration ID | M |  | INTEGER (0..111, …) |  | – |  |
| >CSI Acquisition List |  | 0..1 |  |  | – |  |
| >>CSI Acquisition Item |  | 1 .. <maxnoofCSIResourceConfigurationsPlus1> |  |  | – |  |
| >>>RequestFor Semi Persistent CSI Resources | M |  | ENUMERATED(activate, deactivate, …) |  | – |  |
| >>>CSI Resource Configuration ID | M |  | INTEGER (0..111,…) |  | – |  |

Option 2

9.1.x.x7 	CSI-RS COORDINATION REQUEST

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 UE XnAP ID | M |  | NG-RAN node1 UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| NG-RAN node2 UE XnAP ID | M |  | NG-RAN node2 UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | reject |
| CSI Acquisition | O |  | ENUMERATED (true, …) | Indicates that transmission of SP CSI resources is for CSI Acquisition | YES | ignore |
| CSI Resource Request | M |  |  |  | YES | reject |
| >CSI Resource Request Item |  | 0 .. <maxnoofCSIResourceConfigurations> |  |  | – |  |
| >>Request for Semi Persistent CSI Resources | M |  | ENUMERATED(activate, deactivate, …) |  | – |  |
| >>CSI-RS Resource Configuration ID | M |  | INTEGER (0..111, …) |  | – |  |

- RAN3 to agree either Option 1 (TPs in R3-255725/R3-255726) or Option 2 (TPs in R3-255727/R3-255728) for TS 38.423 and TS 38.473, to support CSI-RS coordination.

# Conclusions and Proposals

In this contribution we made observations and proposals for different LTM related topics, as listed below.

Observation 1	A candidate DU knows if the UE support SP CSI-RS resources for CSI acquisition.

Proposal 1	For both F1AP and XnAP, the activation and deactivation of CSI-RS transmission in LTM candidate cells are performed at the level of CSI Resource Configuration ID.

Proposal 2	For both F1AP and XnAP, remove the NR CGI IE in the CSI-RS coordination procedures.

Proposal 3	There is no need to add a new IE for SP CSI-RS resource for CSI acquisition in the corresponding procedure of SP CSI-RS resource for L1 measurement in F1AP and XnAP.

Proposal 4	Add a new IE (e.g., CSI-RS report configuration for CSI Acquisition) in the UE CONTEXT SETUP RESPONSE F1AP messages.

Proposal 5	Add new IE (e.g., CSI-RS report configuration for CSI Acquisition) in the HANDOVER REQUEST ACKNOWLEDGE message.

Proposal 6	There is no need for an explicit request to provide the CSI-RS report configuration for CSI acquisition.

Proposal 7	RAN3 to discuss the following options to resolve the ambiguity of using the same CSI-RS Coordination procedures for L1 measurements and for CSI acquisition:  Option 1) Activation/deactivation is per CSI Resource Configuration and the Semi persistent CSI resources are split in two list, one for L1 measurements, one for CSI acquisition. Option 2) Activation/deactivation is per CSI Resource Configuration and a new IE indicates for which purpose all the resources are used.

Proposal 8	RAN3 to agree either Option 1 (TPs in R3-255725/R3-255726) or Option 2 (TPs in R3-255727/R3-255728) for TS 38.423 and TS 38.473, to support CSI-RS coordination.