3GPP TSG-RAN WG3 Meeting #129	R3-255729

Bengaluru, IN, 25 – 29 August, 2025

Agenda Item:	11.3

Source:	ZTE Corporation

Title:	Discussion on AI/ML assisted Coverage and Capacity Optimization

Document for:	Discussions & Approval

- Introduction

The following progress have been made in RAN3#127bis:

Adopt Opt2.

(Note: Opt2: gNB-CU directly forwards the received further CCO state of neighbor cells to gNB-DU.)

Specify mechanisms (F1, Xn) to update/cancel a prediction, no new IE is needed to identify the previous prediction (affected cells and beams will identify the previously signalled issue).

Addition of new code-point in predicted coverage modification cause (Xn) / predicted CCO issue (F1), with details FFS.

Timing information for predicted CCO issue over Xn is not needed?

Based on that and the objectives [1] of Rel-19 WI on 'Enhancements for AI/ML in NG-RAN', we present our analysis of the normative work for the Coverage and Capacity Optimization. Additionally, the corresponding TP is included.

- Discussion

2.1 Timing and update CCO info

The following issue has been left after previous meeting:

Timing information for predicted CCO issue over Xn is not needed?

For transmitting the time info for coverage modification cause over Xn, we do not find any explicit benefit to support this transmission. Based on our understanding, the AI assisted CCO is used to prevent the cause issue occur. Hence, before the cause issue(e.g. either coverage or cell edge capacity) happens, the modification has been made at the sending node. From receiving node point of view, only transmitting the predicted coverage modification cause information is good enough for either nodes. It’s good for receiving node to know the reason for this coverage modification. However, the timing of a CCO issue that has already been resolved holds little significance for the receiving node.

Observation 1: The future CCO state execution time must precede the CCO issue, making it impractical to send the CCO issue time to the neighboring base station.

In addition, some companies provided views that the timing information can be used to distinguish multiple predicted CCO issues and/or future CCO state which has been transmitted from CU1 to CU2. Thus, CU2 knows whether the received predicted CCO issue and/or future CCO state is a new one or the update of the previously received one by using the timing info. As we discussed in previous meeting on how to define the max duration for predicted CCO issue and future CCO state, the prediction results would be uncontrollable since it is imppossible to guarantee what will happen within a long duration. Considering the deadline of Rel-19 is approaching and the AI assisted CCO has never been deployed, we prefer to support the simplest scenario in Rel-119, which is:

- For each CU, it only exposure 1 valid predicted CCO issue in a certain duration.

- For each DU, it only exposure 1 valid future CCO state in a certain duration.

In addition, some companies have noted that the timing information can be used to distinguish between multiple predicted CCO issues and/or future CCO states transmitted from CU1 to CU2. Thus, CU2 can determine whether the received predicted CCO issue and/or future CCO state is new or an update of a previously received one by using the timing information.

As discussed in the previous meeting on defining the maximum duration for predicted CCO issues and future CCO states, the prediction results could become uncontrollable since it is impossible to guarantee what will happen within a long duration. Considering the approaching deadline of Rel-19 and the fact that AI-assisted CCO has never been deployed, we prefer to support the simplest scenario in Rel-19, which is:

- For each CU, only one valid predicted CCO issue is exposed within a certain duration.

- For each DU, only one valid future CCO state is exposed within a certain duration.

Although only one valid predicted CCO issue is transmitted from the CU to the DU, the CU isn’t restricted to generating just one predicted CCO issue for different timing in terms of implementation. The CU can generate multiple predicted CCO issues for subsequent timings and transmit only the most imminent one to the DU. Once this predicted CCO issue expires, the CU sends the next predicted CCO issue in the queue. Base on that, RAN3 also do not need to discuss the update prediction result issue. If companies find limitations regarding the number of valid prediction results during deployment, RAN3 could further explore enhancing support for multiple valid prediction results in future releases.

Proposal 1: It is proposed that RAN3 should firstly discuss the benefits of time information related to coverage modification cause for the receiving node before discussing whether to send this time information.

Proposal 2: RAN3 confirm that:

  - Each DU is affected by only one predicted CCO issue generated by its CU within a period.

  - Each CU has exactly one valid future CCO state for each DU it serves within a period.

2.2 New codepoint for wrong prediction

The related left issue is shown below:

Addition of new code-point in predicted coverage modification cause (Xn) / predicted CCO issue (F1), with details FFS.

To solve this issue, 3 alternatives are on the table:

Alt1: If the Predicted CCO issue IE or Predicted Coverage Modification Cause IE does not exist, the receiving side shall believe that the previous prediction is invalid.

Alt2: Introduce new cause value(e.g. cancel previous prediction) for the Predicted CCO issue IE and Predicted Coverage Modification Cause IE.

Alt3: new IE for invalide prediction

From our point of view, introduce new cause value for the existing IE is the most simple and straightforward one. Hence, we encourage companies to consider and agree the following proposal:

Proposal 3: Introduce new cause value(e.g. cancel previous prediction) for the Predicted CCO issue IE and Predicted Coverage Modification Cause IE which is used to notice the receiving side that the previous prediction is invalid.

The corresponding TP to 38.423, 38.473 38.300 and 38.401 are provided in our other contributions this meeting.

- Conclusion

Following is our proposals and observations:

Observation 1: The future CCO state execution time must precede the CCO issue, making it impractical to send the CCO issue time to the neighboring base station.

Proposal 1: It is proposed that RAN3 should firstly discuss the benefits of  time information related to coverage modification cause for the receiving node before discussing whether to send this time information.

Proposal 2: RAN3 confirm that:

  - Each DU is affected by only one predicted CCO issue generated by its CU within a period.

  - Each CU has exactly one valid future CCO state for each DU it serves within a period.

Proposal 3: Introduce new cause value(e.g. cancel previous prediction) for the Predicted CCO issue IE and Predicted Coverage Modification Cause IE which is used to notice the receiving side that the previous prediction is invalid.

- Reference

[1] RP-242385, New WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, ZTE Corporation, NEC

[2] TR 38.743

[3] TS 38.401