3GPP TSG-RAN WG3 Meeting #129	R3-255735

Bengaluru, India, 25 – 29 August 2025

Agenda Item:       10.2

Source:     ZTE Corporation

Title: 	(TP for SON BLCR for 38.473 and 38.401) LTM Beam and TA Left Issues

Document for:	Discussion and Approval

1	Introduction

During RAN3#128, the TP in R3-253943 was agreed for LTM related case 1 and case 2, namely, the case of BFR shortly after a successful LTM cell, and the case of LTM  failure due to wrong beam.In the approved TP, for both cases, only a single recovery beam or reconnecting beam information is included in the corresponding F1AP messages, along with the F1AP UE DU ID. This implies that immediate reporting is applied in both cases, otherwise, the message would be expected to contain a list of relevant beam information. Therefore, the corresponding stage 2 should also be updated to clarify that prompt notification is required for both case 1 and case 2.

Proposal 1:Upon detecting a BFR shortly after a successful LTM cell switch or a RACH-based LTM using the beam different from the one indicated in the cell switch command, the target DU shall promptly notify the source DU of the relevant beam information via the CU. Update the corresponding stage 2 to clarify that prompt notification is required for above cases.

During RAN3#128, the solution for outdated TA optimization in the LTM failure case was agreed. Specifically, when the DU detects an outdated TA issue after the UE has successfully performed a RACH-based LTM, the DU sends the TA value used during the access to the source DU via the CU.However, the issue of outdated TA optimization in the near failure case is left. In this scenario, if the RAN signaled TA is imprecise and causes the RACH-less LTM access to near failure, only the RAN is capable of detecting such a condition. Therefore, we propose supporting a RAN based solution for outdated TA optimization in the LTM near failure case and updating the corresponding stage 2/3 accordingly.

Furthermore, following the same principle outlined in Proposal 1, since only a single TA value rather than a list of TA is included in the F1AP messages, this implies that immediate reporting is applied for outdated TA optimization in both failure and near failure cases. Accordingly, the specification should be updated to clarify that prompt reporting is required when outdated TA is detected under such conditions.

Proposal 2: Support a RAN based solution for outdated TA optimization in the LTM near failure case. The prompt notification is required when outdated TA is detected in both failure and near-failure cases. Update the corresponding stage 2/3 accordingly.

- 2 Conclusion:

Proposal 1:Upon detecting a BFR shortly after a successful LTM cell switch or a RACH-based LTM using the beam different from the one indicated in the cell switch command, the target DU shall promptly notify the source DU of the relevant beam information via the CU. Update the corresponding stage 2 to clarify that prompt notification is required for above cases.

Proposal 2: Support a RAN based solution for outdated TA optimization in the LTM near failure case. The prompt notification is required when outdated TA is detected in both failure and near-failure cases. Update the corresponding stage 2/3 accordingly.

3	TP for BL CR for TS38.401

<< Start of Change >>

7.X	MRO support for LTM

The gNB-CU receives a RLF report associated to an LTM mobility event from the UE or via the Failure indication message over Xn. The gNB-CU performs initial analysis and in case of failure due to inappropriate cell switch triggering may forward the RLF report to the last serving gNB-DU in case of too late LTM, or to the source gNB-DU in case of too early LTM or LTM to wrong cell.

The target gNB-DU identifies that a Beam Failure Recovery (BFR) has happened in the UE shortly after a successful LTM cell switch by detecting a time gap between the successful LTM cell switch and the BFR in the same cell is smaller than the configured threshold (e.g. Tstore_UE_cntxt).The target gNB-DU performs initial analysis and may promptly send the recovery beam information to the source gNB-DU via the gNB-CU.

The target gNB-DU identifies that the UE successfully performs a RACH-based access while reconnecting/re-establishing/recovering to the same target cell but to a beam different from the beam used during the failed LTM cell switch execution. The target gNB-DU may promptly send the reconnected/re-established/recovery beam information to the source gNB-DU via the gNB-CU. The source gNB-DU performs root cause analysis to detect RACH-less LTM failure caused by wrong beam selection.

The target gNB-DU identifies that the UE successfully performed a RACH-based access while reconnecting/re-establishing/recovering to the same target cell and same target beam but with TA value different from the one which was included in CU-DU CELL SWITCH NOTIFICATION message received from gNB-CU at LTM Cell Switch execution. The target gNB-DU promptly sends the TA value used at successful RACH-based access to the source gNB-DU via the gNB-CU.

The target gNB-DU identifies that the TA value included in the CU-DU CELL SWITCH NOTIFICATION message received from the gNB-CU at LTM Cell Switch execution is improper, resulting in degraded performance and near failure of the RACH less LTM access. The target gNB-DU performs initial analysis and may promptly send a near failure indication related to TA to the source gNB-DU via the gNB-CU.

<< End of Change >>

4	 TP to SON BL CR for 38.473

<< Start of Change >>

8.11.1.2	Successful Operation

Figure 8.11.1.2-1: Access and Mobility Indication procedure. Successful operation

The Access and Mobility Indication procedure is initiated by ACCESS AND MOBILITY INDICATION message sent from gNB-CU to gNB-DU.

If the ACCESS AND MOBILITY INDICATION message contains the RA Report List IE the gNB-DU shall take it into account for optimisation of RACH access procedures.

If the ACCESS AND MOBILITY INDICATION message contains the RLF Report Information List IE the gNB-DU shall take it into account for optimisation of mobility parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Successful HO Report Information List IE the gNB-DU may take it into account for optimisation of mobility parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Successful PSCell Change Report Information List IE, the gNB-DU may take it into account for optimisation of PSCell change/addition related parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Beam Failure Recovery Information IE, the gNB-DU shall, if supported, consider that a Beam Failure Recovery after successful LTM Cell Switch has occurred in the target DU for the UE identified by the gNB-DU UE F1AP ID IE in the Beam Failure Recovery Information IE , and may take it into account for optimisation of target beam selection.

If the ACCESS AND MOBILITY INDICATION message contains the LTM Cell Switch Failure due to Wrong Beam Information IE, the gNB-DU shall, if supported, consider that an LTM Cell Switch Failure due to Wrong Beam has occurred in the target DU for the UE identified by the gNB-DU UE F1AP ID IE in the LTM Cell Switch Failure due to Wrong Beam Information IE, and may take it into account for optimisation of target beam selection.

If the ACCESS AND MOBILITY INDICATION message contains the TA Information Related LTM Failure IE, or TA Information Related LTM Near Failure , the gNB-DU may take it into account for optimisation of Timing Advance related parameters for the UE identified by the gNB-DU UE F1AP ID IE in the corresponding IEs.

<< Next of Change >>

8.11.2	DU-CU Access and Mobility Indication

8.11.2.1	General

This procedure is initiated by the gNB-DU to send the Access and Mobility related Information to the gNB-CU.

The procedure uses non-UE-associated signalling.

8.11.2.2	Successful Operation

Figure 8.11.2.2-1: DU-CU Access and Mobility Indication procedure. Successful operation

The DU-CU Access and Mobility Indication procedure is initiated by DU-CU ACCESS AND MOBILITY INDICATION message sent from the gNB-DU to the gNB-CU.

If the DU-CU ACCESS AND MOBILITY INDICATION message contains the DL LBT Failure Information List IE, the gNB-CU shall take it into account for optimisation of mobility parameters.

Interaction with the Access and Mobility Indication procedure:

If the Beam Failure Recovery Information IE, the LTM Cell Switch Failure due to Wrong Beam Information IE, the TA Information Related LTM Failure, or the TA Information Related LTM Near Failure IE for the UE identified by the gNB-DU UE F1AP ID IE assigned by the target gNB-DU in corresponding IEs  is included in the DU-CU ACCESS AND MOBILITY INDICATION message, the gNB-CU shall, if supported, forward relevant information to the source gNB-DU via the ACCESS AND MOBILITY INDICATION message including the Beam Failure Recovery Information IE, the LTM Cell Switch Failure due to Wrong Beam Information IE, the TA Information Related LTM Failure or the TA Information Related LTM Near Failure IE respectively,along with the gNB-DU UE F1AP ID IE assigned by the source gNB-DU in corresponding IEs.

<< Next of Change >>

9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >>C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU. This IE is included in case the gNB-DU responsible for the LTM failure is not the gNB-DU serving the UE at the time of LTM failure. | YES | ignore |
| >>RLF Report Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| Successful PSCell Change Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report information Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in TS 38.331 [8]. | - |  |
| Beam Failure Recovery Information | O |  |  |  | YES | ignore |
| >Recovery TCI State | M |  | OCTET STRING | Includes the TCI-StateId IE used at Beam Failure Recovery, as defined in TS 38.331 [8]. | - |  |
| >gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | - |  |
| LTM Cell Switch Failure due to Wrong Beam Information | O |  |  |  | YES | ignore |
| >TCI State | M |  | OCTET STRING | Includes the TCI-StateId IE as defined in TS 38.331 [8]. | - |  |
| >gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | - |  |
| TA Information Related LTM Failure | O |  |  |  | YES | ignore |
| >TA Value | M |  | INTEGER (0..4095) | Indicates the TA value used at successful Random Access, as defined in TS 38.213 [31]. | - |  |
| >gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | - |  |
| TA Information Related LTM Near Failure | O |  |  |  | YES | ignore |
| >Almost Outdate Indication | M |  | ENUMERATED (true,...) | Indicates the TA value used in LTM cell switch resulting in near failure of the RACH less LTM access | - |  |
| >gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports. Value is 64. |

<< Next of Change >>

9.2.10.2	DU-CU ACCESS AND MOBILITY INDICATION

This message is sent by the gNB-DU to provide access and mobility information to the gNB-CU.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| DL LBT Failure Information List |  | 0..1 |  |  | YES | ignore |
| > DL LBT Failure Information Item |  | 1 .. <maxnoofLBTFailureInformation> |  |  | - |  |
| >>DL LBT Failure Information | M |  | 9.3.1.327 |  | - |  |
| Beam Failure Recovery Information | O |  |  |  | YES | ignore |
| >Recovery TCI State | M |  | OCTET STRING | Includes the TCI-StateId IE used at Beam Failure Recovery, as defined in TS 38.331 [8]. | - |  |
| >gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | - |  |
| LTM Cell Switch Failure due to Wrong Beam Information | O |  |  |  | YES | ignore |
| >TCI State | M |  | OCTET STRING | Includes the TCI-StateId IE as defined in TS 38.331 [8]. | - |  |
| >gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | - |  |
| TA Information Related LTM Failure | O |  |  |  | YES | ignore |
| >TA Value | M |  | INTEGER (0..4095) | Indicates the TA value used at successful Random Access, as defined in TS 38.213 [31]. | - |  |
| >gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | - |  |
| TA Information Related LTM Near Failure | O |  |  |  |  |  |
| >Almost Outdate Indication | M |  | ENUMERATED (true,...) | Indicates the TA value used in LTM cell switch resulting in near failure of the RACH less LTM access |  |  |
| >gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLBTFailureInformation | Maximum no. of UEs for which LBT Failure Information is provided, the maximum value is 64. |

<< Next of Change  >>

[snip]

9.4	Message and Information Element Abstract Syntax (with ASN.1)

[snip]

9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::= ¨

[snip]

F1U-PathFailure,

LTMResetInformation,

CuBeamFailureRecoveryInformation,

DuBeamFailureRecoveryInformation,

TAInformationRelatedLTMFailure,

TAInformationRelatedLTMNearFailure,

[snip]

id-LTMResetInformation,

id-PreconfiguredSRSInformation,

id-CuBeamFailureRecoveryInformation,

id-DuBeamFailureRecoveryInformation,

id-TAInformationRelatedLTMFailure,

id-TAInformationRelatedLTMNearFailure,

[snip]

-- **************************************************************

--

--  Access And Mobility Indication ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- Access And Mobility Indication

--

-- **************************************************************

AccessAndMobilityIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { AccessAndMobilityIndicationIEs} },

...

}

[snip]

AccessAndMobilityIndicationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory }|

{ ID id-RAReportList				CRITICALITY ignore	TYPE RAReportList			PRESENCE optional }|

{ ID id-RLFReportInformationList				CRITICALITY ignore	TYPE RLFReportInformationList				PRESENCE optional }|

{ ID id-SuccessfulHOReportInformationList		CRITICALITY ignore	TYPE SuccessfulHOReportInformationList	PRESENCE optional }|

{ ID id-SuccessfulPSCellChangeReportInformationList	CRITICALITY ignore	TYPE SuccessfulPSCellChangeReportInformationList		PRESENCE optional }|

{ ID id-TAInformationRelatedLTMFailure	CRITICALITY ignore	TYPE TAInformationRelatedLTMFailure		PRESENCE optional }|

{ ID id-TAInformationRelatedLTMNearFailure	CRITICALITY ignore	TYPE TAInformationRelatedLTMNearFailure		PRESENCE optional }|

{ ID id-CuBeamFailureRecoveryInformation			CRITICALITY ignore 	TYPE CuBeamFailureRecoveryInformation  		PRESENCE optional },

...

}

[snip]

-- **************************************************************

--

--  DU-CU Access And Mobility Indication ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- DU-CU Access And Mobility Indication

--

-- **************************************************************

DUCUAccessAndMobilityIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { DUCUAccessAndMobilityIndicationIEs} },

...

}

DUCUAccessAndMobilityIndicationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID							PRESENCE mandatory }|

{ ID id-DLLBTFailureInformationList				CRITICALITY ignore	TYPE DLLBTFailureInformationList		PRESENCE optional}|

{ ID id-TAInformationRelatedLTMFailure	CRITICALITY ignore	TYPE TAInformationRelatedLTMFailure		PRESENCE optional }|

{ ID id-TAInformationRelatedLTMNearFailure	CRITICALITY ignore	TYPE TAInformationRelatedLTMNearFailure		PRESENCE optional }|

{ ID id-DuBeamFailureRecoveryInformation		CRITICALITY ignore	TYPE DuBeamFailureRecoveryInformation	PRESENCE optional },

...

}

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

[snip]

id-TagIDPointer,

id-LocalOrigin,

id-SRSPosPeriodicConfigHyperSFNIndex,

id-rLFReportFailureType

[snip]

CuBeamFailureRecoveryInformation	::= SEQUENCE {

recoveryTCIState									OCTET STRING,

lTMgNB-DU-ID										GNB-DU-ID,

iE-Extensions		ProtocolExtensionContainer { { CuBeamFailureRecoveryInformation-ExtIEs} } OPTIONAL,

...

}

CuBeamFailureRecoveryInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

[snip]

DuBeamFailureRecoveryInformation	::= SEQUENCE {

recoveryTCIState									OCTET STRING,

lTMgNB-CU-ID										GNB-CU-ID,

iE-Extensions		ProtocolExtensionContainer { { DuBeamFailureRecoveryInformation-ExtIEs} } OPTIONAL,

...

}

DuBeamFailureRecoveryInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

[snip]

TAInformationRelatedLTMFailure	::= SEQUENCE {

TA-Value	INTEGER (0..40950),

lTMgNB-DU-ID										GNB-DU-ID,

iE-Extensions		ProtocolExtensionContainer { { TAInformationRelatedLTMFailure-ExtIEs} } OPTIONAL,

...

}

TAInformationRelatedLTMFailure-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TAInformationRelatedLTMNearFailure	::= SEQUENCE {

outdateIndication	OutdateIndication,

lTMgNB-DU-ID										GNB-DU-ID,

iE-Extensions		ProtocolExtensionContainer { { TAInformationRelatedLTMNearFailure-ExtIEs} } OPTIONAL,

...

}

TAInformationRelatedLTMNearFailure-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

OutdateIndication	::=	ENUMERATED {true, ...}

[snip]

RLFReportInformationList	::= SEQUENCE (SIZE(1.. maxnoofRLFReports)) OF RLFReportInformationItem

RLFReportInformationItem	::= SEQUENCE {

nRUERLFReportContainer		NRUERLFReportContainer,

uEAssitantIdentifier		GNB-DU-UE-F1AP-ID		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { RLFReportInformationItem-ExtIEs} }	OPTIONAL,

...

}

RLFReportInformationItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ID id-C-RNTI				CRITICALITY ignore EXTENSION C-RNTI					PRESENCE optional },

{ID id-rLFReportFailureType	CRITICALITY ignore EXTENSION RLFReportFailureType		PRESENCE optional },

...

}

RLFReportFailureType ::= ENUMERATED{ toolateLTM, tooearlyLTM, lTMtowrongcell,...}

[snip]

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules

[snip]

id-LocalOrigin										ProtocolIE-ID ::= 854

id-LTMResetInformation								ProtocolIE-ID ::= 855

id-SRSPosPeriodicConfigHyperSFNIndex				ProtocolIE-ID ::= 856

id-PreconfiguredSRSInformation						ProtocolIE-ID ::= 857

id-rLFReportFailureType								ProtocolIE-ID ::= 999 --to be assigned by MCC

id-CuBeamFailureRecoveryInformation					ProtocolIE-ID ::= 998 --to be assigned by MCC

id-DuBeamFailureRecoveryInformation					ProtocolIE-ID ::= 997 --to be assigned by MCC

id-TAInformationRelatedLTMFailure					ProtocolIE-ID ::= xxx

id-TAInformationRelatedLTMNearFailu					ProtocolIE-ID ::= yyy

<< End of Change >>