3GPP TSG-RAN WG3 Meeting #129	R3-255741

Bengaluru, India, 25th ~29th Aug, 2025

Title:	Consideration on Muting of always-on signals in 5G broadcast [5GB_CASMuting]

Agenda Item:	8.1

Source:	Huawei, Qualcomm Incorporated, EBU

Document for:	Discussion

# Introduction

In [1], RAN1 sends the agreed Rel-19 higher layers parameters list to RAN3, including the following one:

- TEI19 (TEI sheet)

  - Identifier: [5GB_CASMuting]

    - Description: Muting of always-on signals in 5G broadcast

In this contribution, we discuss the RAN3 impacts related to this this [5GB_CASMuting] topic.

# 2	Discussion

In RAN1#120bis meeting Session notes for 9.14 (TEI) [3], the following agreements can be found:

Agreement - The non-MBSFN subframes containing the cell acquisition signals (PSS/SSS/PBCH/SIB1) in an MBMS-dedicated cell shall only be transmitted within the first (k×40) ms out of every (n×160) ms. - Introduce a new parameter n, to be configured for support MBMS, from the set {2,4,8,16}. - Introduce a new parameter k, to be configured for support MBMS, from the set {4,5,6,7,…,63} - In the absence of the above parameters {n,k}, all non-MBSFN subframes containing cell acquisition signals in an MBMS-dedicated cell shall be transmitted NOTE: For a MBMS-dedicated cell, there is no RAN4 impact from the above TEI proposal.

In [2], the following can be found for [5GB_CASMuting]:

It can be understood that in an MBMS dedicated cell, the non-MBSFN subframes containing the cell acquisition signals (PSS/SSS/PBCH/SIB1) shall only be transmitted within the first (k×40) ms out of every (n×160) ms. And the values of N_CAS and K_CAS are to be configured in SIB1-MBMS.

Considering that the MCE is responsible for deciding on whether to use MBSFN and the allocation of the radio resources used by all eNBs in the MBSFN area for multi-cell MBMS transmissions using MBSFN operation, it should be the MCE to decide the values of the K_CAS and the N_CAS, and then informs the eNBs via M2AP.

Proposal 1: MCE makes decision on the values of the K_CAS and the N_CAS, and informs to the eNBs via M2AP.

As these parameters are cell level from radio point of view, to ensure synchronized transmission in an MBSFN area, these pareameters should be indicated from MCE to the eNB in per MBSFN area granularity, i.e., include them in the MBSFN Area Configuration Item Ies IE in the M2AP: MBMS SCHEDULING INFORMATION message.

Proposal 2: Include the CAS Muting Parameters IE (includes sub-IEs K_CAS and N_CAS) in the MBSFN Area Configuration Item Ies IE in the M2AP: MBMS SCHEDULING INFORMATION message.

The value of K_CAS includes the set of values {4,5,6,7,…,63}which are continuous positive integers, therefore it is better to define it as INTEGER (4..63, ...).

The value of N_CAS includes the set of values {2,4,8,16}, therefore it is better to define it as ENUMERATED (n2, n4, n8, n16, ...).

Proposal 3: Define the K_CAS IE as INTEGER (4..63, ...), and define the N_CAS IE as ENUMERATED (n2, n4, n8, n16, ...).

# 3	Conclusion

In this contribution, we discussed the RAN3 impact of the RAN1 TEI19 topic [5GB_CASMuting], and got the following proposals:

Proposal 1: MCE makes decision on the values of the K_CAS and the N_CAS, and informs to the eNBs via M2AP.

Proposal 2: Include the CAS Muting Parameters IE (includes sub-IEs K_CAS and N_CAS) in the MBSFN Area Configuration Item Ies IE in the M2AP: MBMS SCHEDULING INFORMATION message.

Proposal 3: Define the new K_CAS IE as INTEGER (4..63, ...), and define the new N_CAS IE as ENUMERATED (n2, n4, n8, n16, ...).

The corresponding M2AP CR is provided in [4].

# 4	Reference

- R1-2503242 LS on Rel-19 higher layers parameters list Post RAN1#121, Source: RAN1, To: RAN WG2, RAN WG3, Cc: RAN WG4

- R1-2503243 Consolidated Rel-19 higher layers parameters list Post RAN1#121

- R1-2503118 Session notes for 9.14 (TEI), Source:	Ad-hoc Chair (CMCC)

- R3-255742 Support of Muting of always-on signals in 5G broadcast [5GB_CASMuting], Huawei, Qualcomm Incorporated, EBU